| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0240_r1 |
| --- | --- |

| Title: | Consistent coding of motion information for B slices with identical reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

B slices with two identical reference picture lists, or GPB pictures, are widely used in HEVC. Those slices are specially handled in the current design. When uni-directional prediction is applied, only prediction from list 0 can be used; and when bi-directional prediction is applied, the mvd for list 1 are both zero. Both methods are helpful to improve the coding efficiency. However, in the current draft, the former one is performed implicitly by an encoder and the latter one is signaled explicitly. This document discusses two different ways to unify the design.

# Introduction

B slices with two identical reference picture lists (we will call them B0 slices in this document) are widely used in HEVC. There are some specials for those slices. As far as we know, when uni-directional prediction is applied, only prediction from list 0 can be used; and when bi-directional prediction is applied, the mvd for list 1 are both zero. These aspects are both helpful to improve the coding efficiency, but they are achieved by different ways. Only applying list 0 prediction to B0 slices when uni-directional prediction is used, as the adoption of JCTVC-I0125 [1], is achieved by encoder only method. Even for B0 slices, the encoder can still select to use list 1 prediction when uni-directional prediction is applied, although it may not be helpful to improve the coding efficiency. Restricting the mvd of list 1 to zero [2], is achieved by explicitly signalling a new syntax element called mvd_l1_zero_flag at slice header. When this flag is 1, the mvd of list 1 is not signaled when bi-directional prediction is applied. And when this flag is 1, the encoder does not have the flexibility to set the mvd of list 1 to be other values. This document discusses those two aspects in HEVC.

# Solution

These two aspects are quite similar and they could be achieved by similar method, i.e. explicitly signalling a syntax element at slice header or not.

## Get rid of explicitly signalling at slice header

If the restriction of mvd of list 1 is achieved by encoder only decision, a new context (actually we split the original context used for abs_mvd_greater0_flag to two, one is used for abs_mvd_greater0_flag in list 0 and the other is used for one is used for abs_mvd_greater0_flag in list 1) is need to be added to be used as the context of whether mvd is 0 or not in list 1. JCTVC-I0125 [1] also introduced a new context. The experimental results show that this modification has 0.0% impact on the BD-Rate on average and no impact on the complexity. Detailed results are provided in Section 3 and the text modifications are provided in Section 5.

## Explicitly signalling at slice header

We can also explicitly signalling a syntax element at slice header to indicate whether list 1 prediction is allowed for uni-directional prediction. When the current slice does not use list 1 prediction for uni-directional prediction, the current slice does not need to distinguish whether it is list 0 prediction or list 1 prediction if it is uni-directional prediction. The experimental results show that this modification also has 0.0% impact on the BD-Rate on average and no impact on the complexity. Detailed results are provided in Section 3 and the text modifications are provided in Section 5.

# Experimental results

The proposed two methods are implemented in HM-7.0. The common test conditions specified in [3] are followed.

## Get rid of explicitly signalling the mvd in list 1 at slice header

The detailed results can be found in JCTVC-J0240_B0l1Mvd.xlsm.

Table 1 Y BD-Rate of not explicitly signalling the mvd in list 1 at slice header

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 100% | 100% | 100% |

## Explicitly signalling list 1 prediction at slice header

The detailed results can be found in JCTVC-J0240_B0l0Pred.xlsm.

Table 2 Y BD-Rate of explicitly signalling list 1 prediction at slice header

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  | -0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 100% | 100% | 100% |

# Conclusion

We suggest keeping consistent on those two aspects, explicitly signalling a syntax element at slice header the two cases or not explicitly signalling. So we suggest

Either (1) not to explicitly indicate whether the mvds of list 1 are both 0 when bi-directional prediction is used by adding a new context;

or (2) to add a syntax element at slice header to indicate whether list 1 prediction is applied for uni-directional prediction.

# Text Modification

The text modifications are based on JCTVC-I1003_d4 [4].

## Get rid of explicitly signalling the mvd in list 1 at slice header

7.3.3 Slice header syntax

| …… |  |
| --- | --- |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| …… |  |

7.4.3 Slice header semantics

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

7.3.7 Prediction unit syntax

| …… |  |
| --- | --- |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ], 0) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ], 1) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |

7.3.7.1 Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y , list) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| …… |  |

9.2.1 Table 9-4

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Table 9-1 |  | 0…1 | 12…3 |
|  | abs_mvd_greater1_flag | Table 9-1 |  | 24 | 35 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-21

Table 9-1 – Values of variable initValue for abs _mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialization variable | abs_mvd _greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 24 | 35 |
| initValue | 140 | 198154 | 169 | 31 | 169198 | 198 |

Note: the initial values in this table and the tables around do not match the HM-7.0. The initial values are changed to match those in HM-7.0 first and these modifications are not marked.

And we only add one context (position 3). The context at position 1 is never used (which is CNU in HM software).

9.2.2 Table 9-34

| Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-1 | 0 |
|  | 2 |  | 0 | Table 9-1 | 12 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-1 | 24 |
|  | 2 |  | 0 | Table 9-1 | 35 |

9.2.3 Table 9-39

| Table 9-3 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| abs_mvd_greater0_flag[ ] | Table 9-1 | 0 | 0list | na | na | na | na |
|  |  | 12 | 0list | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Table 9-1 | 24 | 0 | na | na | na | na |
|  |  | 35 | 0 | na | na | na | na |

## Explicitly signalling list 1 prediction at slice header

7.3.3 Slice header syntax

| …… |  |
| --- | --- |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| use_l0_only_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| …… |  |

7.4.3 Slice header semantics

use_l0_only flag equals to 1 indicates that list 1 prediction is not applied for uni-directional prediction and only list 0 prediction is applied for uni-directional prediction.

7.4.7 Prediction unit semantics

There is something incorrect in Table 7-11 (the original text without this modification is still incorrect). The fix is provided.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1 or bi-prediction is used for the current prediction unit according to Table 7-4. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

There is something incorrect in Table 7-11. Actually in the WD7, inter_pred_idc is decoded as TU, where 1 is prefix bin and 0 is stop bin. But in the HM-7.0, for inter_pred_idc, 0 is used as prefix bin and 1 is used as stop bin. We assume that inter_pred_idc is decoded as TU and ignore which bin is used as prefix bin and which bin is used as stop bin.

Table 7-4 – Name association to inter prediction mode

| slice_type | inter_pred_idc | Name of inter_pred_idc |
| --- | --- | --- |
| P | inferred | Pred_L0 |
| B | 0 | Pred_L0BI |
|  | 1 | Pred_L1L0 |
|  | 2 | Pred_BIL1 |

9.2.2 Table 9-34

| Table 9-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| inter_pred_idc |  | TU, cMax = 2−use_l0_only_flag |  |  |  |
|  | 2 |  | 0 | Error: Reference source not found | 0 |

# References

- JCTVC-I0125, “On Reference List Combination,” Geneva, CH, April – May 2011.

- JCTVC-H0111, “CE9: Modification of bi-prediction”, San José, CA, USA, February, 2012.

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, April – May 2011.

- JCTVC-I1003, “High efficiency video coding (HEVC) text specification draft 7,” Geneva, CH, April – May 2011.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).