| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0241_r1 |
| --- | --- |

| Title: | Simplification on CBF inference and coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

At the previous meeting, the inference technique in cbf coding is removed due to the complicated conditions. This document presents a simplified cbf inference condition, which results in 0.0%, 0.0%, −0.1%, −0.1, −0.2%, and −0.2% Y BD Rate changing for AI-Main, AI-HE10, RA-Main, RA-HE10, LB-Main and LB-HE10 respectively. This document also discusses to remove all conditions for coding cbf_luma in case more simplification is more preferable than achieving coding gains.

# Introduction

This document studies the inference techniques in cbf coding in HM-7.0. First we simplify the cbf inference conditions for the last TU, which brings about 0.0%, 0.0%, −0.1%, −0.1, −0.2%, and −0.2% Y BD Rate changing for AI-Main, AI-HE10, RA-Main, RA-HE10, LB-Main and LB-HE10 respectively. Second, we find that except the inference technique for the last TU, there is another inference technique in HM for cbf coding, which is inferring the Y cbf at root level to 1 when the cbf for U and V are both 0 for inter coded CUs. We find that when removing this inference technique, the Y BD Rate changing is about 0.0%, 0.1, 0.1%, and 0.1% Y BD Rate changing for RA-Main, RA-HE10, LB-Main and LB-HE10 respectively. The impact of the inference technique on Y root cbf is smaller than the inference technique on the last TU.

# Solution

## Simplified cbf inference technique for the last TU

This document introduces a simplified cbf inference technique for the last TU. For U and V, the condition to infer the last cbf to 1 is only all the first three cbfs are 0. For Y, the conditions to infer the last cbf to 1 are all the first three cbfs are 0 and the current TU size is smaller than the maximum TU size and the current block is inter coded block. Besides these normative modifications, we also modify the TU RDO process with more accurate bit estimation for cbf coding, including when the conditions for inference are satisfied, the bit to represent the last cbf is excluded, and considering the bit cost when the cbf is 0. Detailed results are provided in Section 3 and the text modifications are provided in Section 5.

## Removing the Y cbf inference technique at root level

This document also tests the performance when removing the inference technique for the Y cbf at the root level. Detailed results are provided in Section 3 and the text modifications are provided in Section 5.

# Experimental results

The proposed two methods are implemented in HM-7.0. The common test conditions specified in [1] are followed.

## Simplified cbf inference technique for the last TU

The detailed results can be found in JCTVC-J0241_SimpCbfInfer.xlsm.

Table 1 Y BD-Rate of simplified cbf inference technique for last TU

|  | AI-Main | AI-HE10 | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% | -0.1% | -0.1% |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class C | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% |
| Class E | -0.1% | -0.1% |  |  | -0.2% | -0.1% |
| Overall | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% |
| Enc. Time | 99% | 100% | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 100% | 100% | 100% | 100% | 101% |

## Removing the Y cbf inference technique at root level

The detailed results can be found in JCTVC-J0241_cbf_NoCbfRI.xlsm.

Table 2 Y BD-Rate of removing Y cbf inference at root level

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.1% |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.1% |
| Class E |  |  | 0.2% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 99% | 100% | 97% |

Table 3 shows the performance of removing the Y cbf inference technique at root level together with fix in TU RDO process (roughly take the bit to represent 0 cbf into consideration). The detailed results can be found in JCTVC-J0241_NoCbfRI_TURDO.xlsm.

Table 3 Y BD-Rate of removing Y cbf inference at root level with TU RDO fix

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.1% |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  | 0.0% | 0.2% |
| Overall | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 100% | 100% | 100% |

And there is also some minor improvements for U and V.

## TU RDO fix only

Table 4 shows the performance of TU RDO fix only. Detailed results are provided in JCTVC-J0241_TURDO.xlsm.

Table 4 Y BD-Rate of TU RDO fix

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E |  |  | 0.0% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 100% | 100% | 101% | 100% |

The Y BD-Rate is almost unchanged, and there is some minor gain (-0.1% for RA_Main and RA_HE10, -0.4~-0.5 for LB_Main and LB_HE10) for U and V.

# Conclusion

The simplified cbf inference technique for the last TU can still bring about 0.1%~0.2% bit saving for RA and LD. The impact of the cbf inference for the last TU is larger than the cbf inference technique for Y cbf at root level, which is 0.0%~0.1%.

# Text Modification

The text modifications are based on JCTVC-I1003_d4 [2].

## Simplified cbf inference technique for the last TU

7.3.8 Transform tree syntax

| transform_tree( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) ) |  |
| split_transform_flag[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| if( blkIdx < 3 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ) |  |
| cbf_cb[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| if( blkIdx < 3 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ) |  |
| cbf_cr[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0L ][ y0L ][ trafoDepth ] ) { |  |
| if( interTbSplitDirectionL = = 2 ) { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1L = y0L |  |
| x2L = x0L |  |
| y2L = y0L + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3L = x1L |  |
| y3L = y2L |  |
| } else { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y1L = y0L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x2L = x1L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y2L = y1L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| x3L = x2L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y3L = y2L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL ) |  |
| } |  |
| if( interTbSplitDirectionC = = 2 && log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| xC3 = xC1 |  |
| yC3 = yC2 |  |
| } else if ( log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y1C = y0C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x2C = x1C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y2C = y1C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| x3C = x2C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y3C = y2C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC ) |  |
| } else { |  |
| x1C = x0C |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C |  |
| x3C = x0C |  |
| y3C = y0C |  |
| } |  |
| if( interTbSplitDirectionL ! = 2 ) { |  |
| log2TrafoWidth = log2TrafoWidth − 2 * interTbSplitDirectionL + 1 |  |
| log2TrafoHeight = log2TrafoHeight + 2 * interTbSplitDirectionL − 1 |  |
| } |  |
| transform_tree( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( ( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) && ( blkIdx < 3 ¦ ¦ PredMode = = MODE_INTRA ¦ ¦ log2TrafoSize = = Log2MaxTrafoSize ¦¦ cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ) ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

7.4.8 Transform tree semantics

split_transform_flag[ x0L ][ y0L ][ trafoDepth ] specifies whether a block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and PredMode is equal t oMODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

When split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is inferred to be equal to 1.

- log2TrafoSize is greater than Log2MaxTrafoSize

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0

- interSplitFlag is equal to 1

Otherwise, the value of split_transform_flag[ x0L ][ y0L ][ trafoDepth ] is inferred to be equal to 0.

For inter coded CU, when log2TrafoSize is not greater than Log2MaxTrafoSize, the split_transform_flag equals to 1 also imply that the Y cbf of the four sub blocks cannot be all 0.

cbf_cb[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cb[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

–	If all of the following conditions are true, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

–	blkIdx is equal to 3

–	cbf_cb[ xBase ][ yBase ][ trafoDepth ] is equal to 0

–	cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] is equal to 0

–	cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] is equal to 0

- Otherwwise iIf trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0C ][ y0C ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0C, y0C specify the location ( x0C, y0C ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cr[ x0C ][ y0C ][ trafoDepth ] is not present, the value of cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred as follows.

–	If all of the following conditions are true, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

–	blkIdx is equal to 3

–	cbf_cr[ xBase ][ yBase ][ trafoDepth ] is equal to 0

–	cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] is equal to 0

–	cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] is equal to 0

- Otherwise iIf trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

Otherwise, cbf_cr[ x0C ][ y0C ][ trafoDepth ] is inferred to be equal to 0.

## Removing the Y cbf inference technique at root level

7.3.8 Transform tree syntax

| …… |  |
| --- | --- |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

7.4.8 Transform tree semantics

cbf_luma[ x0L ][ y0L ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to 1.

# References

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, April – May 2011.

- JCTVC-I1003, “High efficiency video coding (HEVC) text specification draft 7,” Geneva, CH, April – May 2011.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).