| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0242 |
| --- | --- |

| Title: | Encoding with fixed Lagrange multipliers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Li Li Jinlei Zhang Jizheng Xu Houqiang Li | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution presents a fixed Lagrange multiplier encoding method instead of fix QP encoding as used in HM. When using the proposed fixed Lagrange multiplier encoding algorithm, −1.7%, −1.8%, −1.4%, and −1.4% Y BD-Rate changing for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively. The bit saving for U and V are reported somewhat larger. The encoding/decoding time of the proposed fixed Lagrange multiplier encoding algorithm is about 100%. This contribution does not include any normative changes.

# Introduction

In the current HM-7.0, when encoding one sequence, the QP values are first specified and the Lagrange multiplier (also called lambda, λ) values are calculated according to the QP value and the coding structure. We call it fixed QP encoding. But according to our investigation, if we use fixed Lagrange multiplier encoding instead of fix QP encoding, −1.7%, −1.8%, −1.4%, and −1.4% Y BD-Rate changing for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively can be obtained without significant impact on the encoding/decoding time. This contribution does not include any normative changes.

# Discussion

Usually the optimization target of video coding is
minJ=D+λR	(1)
where D is the distortion, R is the bitrate, λ is the Lagrange multiplier, and J is usually called RD cost.

In (1), QP is one of the parameters to be determined by the optimization process. λ is the only known parameter before the optimization process. And λ is the only parameter can be used to adjust the optimization target. Larger λ results in smaller bit cost and larger distortion; and vice versa. According to our understanding, we should not fix any parameters to be optimized before the optimization process. So we propose to determine the QP value in the RDO process.

As discussed in [1], λ is a more important factor on determining the bitrate. This document proposes to use the fix λ encoding. When the λ value is given, the QP is one of the parameters to be determined by the RDO process. Of course multiple-QP optimization (we consider multiple-QP optimization a kind of fix λ encoding) can be applied to obtain the optimal QP values, although this optimization will increase the encoding time significantly. In this proposal, we suggest to use (2) to determine the QP value, the same as proposed in [1]

QP=4.2005×ln(λ)+13.7122	(2)

We do not change the original HM-7.0 λ setting, but use the fix λ encoding algorithm. When applying fix λ encoding method, QP is to be determined and (2) provides a simple method to determine the QP value. The original QP values and the QP values determined by (2) are shown in Table 1 and 2. The QPi in those two tables are the input QP values for the original HM-7.0 encoding.

Table 1 QP used for each QPi and hierarchical level (original HM-7.0 QP)

|  | QP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| QPi | I | L0 | L1 | L2 | L3 | I | L0 | L1 | L2 |
| 22 | 22 | 23 | 24 | 25 | 26 | 22 | 23 | 24 | 25 |
| 27 | 27 | 28 | 29 | 30 | 31 | 27 | 28 | 29 | 30 |
| 32 | 32 | 33 | 34 | 35 | 36 | 32 | 33 | 34 | 35 |
| 37 | 37 | 38 | 39 | 40 | 41 | 37 | 38 | 39 | 40 |

Table 2 QP used for each QPi and hierarchical level (QP determined by )

|  | QP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| QPi | I | L0 | L1 | L2 | L3 | I | L0 | L1 | L2 |
| 22 | 19 | 21 | 24 | 25 | 29 | 20 | 22 | 25 | 26 |
| 27 | 24 | 26 | 30 | 31 | 35 | 25 | 27 | 31 | 33 |
| 32 | 29 | 31 | 36 | 37 | 41 | 30 | 32 | 37 | 38 |
| 37 | 34 | 36 | 41 | 42 | 46 | 33 | 37 | 42 | 43 |

Although the QP difference among different levels is enlarged, the quality difference among different levels does not change much. The detailed results are shown in Section 3.

# Experimental results

The proposed fixed Lagrange multiplier encoding method is implemented on HM-7.0 and the HM-7.0 is used as the anchor in the tables below. The bitrate ratio is the total bit cost ratio per level, which is calculated as (3). There are four levels for RA and three levels for LD. PSNR difference is the difference of the average PSNR of the lowest level and the average PSNR of the highest level, which is calculated as (4) and (5). Note that the Class F is not included in the overall average and the results with Class F and those without Class F do not change much. The detailed results in Table 3 are provided in JCTVC-J0242_NewQP_PerLevel.xlsm.

BitrateRatio=totalBitrateInEachLevelOfTesttotalBitrateInEachLevelOfAnchor	(3)

DeltaPNSR=PSNRavgLowestLevel−PSNRavgHighestLevel	(4)

PSNRDifference=DeltaPSNRtest−DeltaPSNRanchor	(5)

Table 3 Y BD-Rate of fixed Lagrange multiplier encoding

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | -1.3% | -1.2% |  |  |
| Class B | -1.4% | -1.5% | -1.0% | -1.0% |
| Class C | -2.2% | -2.3% | -1.3% | -1.3% |
| Class D | -1.9% | -2.2% | -1.3% | -1.2% |
| Class E |  |  | -2.3% | -2.3% |
| Overall | -1.7% | -1.8% | -1.4% | -1.4% |
| Enc. Time | 100% | 100% | 100% | 99% |
| Dec. Time | 100% | 100% | 100% | 100% |
| Bitrate ratio | 112%, 97% 98%, 98% | 112%, 97% 98%, 98% | 105% 97%, 97% | 105% 97%, 97% |
| PSNR difference | 0.43 dB | 0.45 dB | 0.10 dB | 0.10 dB |

To compare the performance of the proposed fixed Lagrange multiplier encoding, we also perform some multiple-QP optimization tests. In the multiple-QP optimization, LCU level multiple-QP RDO is performed. We performed 3 QPs RDO, 5QPs RDO. We also provide the results of multiple-QP based on the QP determined by (2), which means several QPs around the QP determined by (2) is in the search range of multiple-QP optimization.

The summary results are provided in Table 4-7. The detailed results can be found in JCTVC-J0242_LCU_3QP_PerLevel.xlsm, JCTVC-J0242_LCU_5QP_PerLevel.xlsm, JCTVC-J0242_LCU_newQPStart_3QP_PerLevel, and JCTVC-J0242_LCU_newQPStart_5QP_PerLevel.

Table 4 Y BD-Rate of LCU 3QP optimization

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | -1.2% | -0.9% |  |  |
| Class B | -1.1% | -1.0% | -0.5% | -0.4% |
| Class C | -1.3% | -1.3% | -0.8% | -0.7% |
| Class D | -1.1% | -1.2% | -0.8% | -0.8% |
| Class E |  |  | -1.3% | -0.9% |
| Overall | -1.2% | -1.1% | -0.8% | -0.7% |
| Enc. Time | 299% | 297% | 299% | 297% |
| Dec. Time | 101% | 101% | 101% | 101% |
| Bitrate ratio | 104%, 99% 99%, 100% | 105%, 99% 99%, 100% | 103% 99%, 99% | 103% 99%, 99% |
| PSNR difference | 0.18 dB | 0.19 dB | 0.07 dB | 0.08 dB |

Table 5 Y BD-Rate of LCU 5QP optimization

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | -1.5% | -1.1% |  |  |
| Class B | -1.6% | -1.5% | -1.0% | -0.9% |
| Class C | -2.1% | -2.2% | -1.4% | -1.2% |
| Class D | -1.8% | -2.0% | -1.3% | -1.3% |
| Class E |  |  | -2.2% | -1.7% |
| Overall | -1.7% | -1.7% | -1.4% | -1.2% |
| Enc. Time | 497% | 494% | 497% | 494% |
| Dec. Time | 101% | 101% | 101% | 101% |
| Bitrate ratio | 109%, 98% 98%, 100% | 109%, 98% 98%, 99% | 105% 98%, 98% | 105% 98%, 98% |
| PSNR difference | 0.35 dB | 0.36 dB | 0.11 dB | 0.12 dB |

Table 6 Y BD-Rate of LCU 3QP optimization around new QP

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | -1.6% | -1.3% |  |  |
| Class B | -2.0% | -1.9% | -1.5% | -1.4% |
| Class C | -2.8% | -2.9% | -2.0% | -1.8% |
| Class D | -2.3% | -2.6% | -1.7% | -1.8% |
| Class E |  |  | -2.7% | -2.6% |
| Overall | -2.2% | -2.2% | -1.9% | -1.8% |
| Enc. Time | 298% | 296% | 298% | 297% |
| Dec. Time | 101% | 101% | 101% | 101% |
| Bitrate ratio | 113%, 97% 97%, 98% | 113%, 97% 97%, 98% | 105% 96%, 97% | 106% 96%, 97% |
| PSNR difference | 0.48 dB | 0.50 dB | 0.11 dB | 0.13 dB |

Table 7 Y BD-Rate of LCU 5QP optimization around new QP

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | -1.7% | -1.4% |  |  |
| Class B | -2.2% | -2.0% | -1.7% | -1.6% |
| Class C | -3.0% | -3.1% | -2.2% | -2.0% |
| Class D | -2.4% | -2.7% | -1.9% | -1.8% |
| Class E |  |  | -2.9% | -2.6% |
| Overall | -2.3% | -2.3% | -2.1% | -1.9% |
| Enc. Time | 496% | 493% | 496% | 494% |
| Dec. Time | 101% | 101% | 101% | 101% |
| Bitrate ratio | 113%, 96% 97%, 98% | 114%, 96% 97%, 97% | 105% 96%, 96% | 106% 96%, 96% |
| PSNR difference | 0.49 dB | 0.51 dB | 0.12 dB | 0.13 dB |

From the tables above we can conclude that

- When using fixed Lagrange multiplier encoding and determine the QP value by (2), −1.7%, −1.8%, −1.4%, and −1.4% Y BD-Rate changing for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively can be obtained without increasing the encoding time. The bit saving for U and V are reported somewhat larger than that on Y. Per level bitrate changing and the PSNR difference are similar to the case when multiple-QP optimization is applied.

- The QP determined by (2) can be used as a good start point when multiple-QP optimization is applied. When using the QP determined by (2) as start point, 3 QPs optimization outperforms 5 QPs optimization using the original QP as start point.

# Lagrange multiplier based rate control

As discussed above and in [1], Lagrange multiplier is a more important factor on determining the bitrate and Lagrange multiplier is the only parameter determined before the RDO process in (1). We build the relationship between the Lagrange multiplier and the bitrate as shown in (6), and then using that relationship for rate control.

R=α⋅λβ	(6)

With (6), we can determine the λ value for a given R. And then we can encode the current picture (or LCU, the basic unit in our rate control is LCU) with the determined λ value. The parameters α and β are updated with the encoding process. We find that more accurate rate control performance can be obtained via the Rate-λ model.

The rate control algorithm is still under development, and we only have the results for LB-Main. For the sequences in Class B to Class E, when targeting the HM-7.0 anchor bitrate, the average bitrate error of the proposed algorithm is only about 0.04% (the number is 0.23% for the rate control algorithm existing in HM-7.0). And the largest bitrate error is 1.02% (the number is 3.63% for the rate control algorithm existing in HM-7.0). And the average Y PSNR is improved by about 0.37 dB over the original HM-7.0 rate control algorithm. The situation is similar when targeting the bitrate specified in the HEVC CfP [2]. The summary results are shown in Table 8 and the detailed results can be seen from JCTVC-J0242_RC AnchorRate_H0.xlsm and JCTVC-J0242_RC CfPRate_H0.xlsm. The results from here use the original HM-7.0 rate control algorithm as the anchor. We find that [5] also provides the results of using the HM-7.0 rate control results, targeting the HM-7.0 anchor bitrate. But The HM-7.0 rate control results in this document are different from those in [5]. The reason may be, although we both target HM-7.0 anchor bitrate, in this document, we target round(HM-7.0 anchor bitrate); while [5] perhaps targets floor(HM-7.0 anchor bitrate). Because if the decimal part of the HM-7.0 anchor bitrate is < 0.5, this document and [5] get identical results. If it is > 0.5, we obtain different results.

Table 8 Rate Control Results using Rate-λ model

|  | Targeting HM-7.0 Anchor bitrate |  | Targeting CfP Anchor bitrate |  |
| --- | --- | --- | --- | --- |
|  | Average bitrate error | Maximum bitrate error | Average bitrate error | Maximum bitrate error |
| HM-7.0 RC | 0.23% | 3.36% | 0.18% | 2.38% |
| Rate-λ RC | 0.04% | 1.02% | 0.02% | 0.10% |
| Y PSNR improvement | 0.37 dB |  | 0.33 dB |  |

And using the proposed Rate-λ model can obtained more consistent bit cost among pictures. Fig.1 below show some example per-picture bit cost of the proposed Rate-λ rate control algorithm and the original rate control in HM-7.0.

Fig. 1 Per-picture bit cost

And as the proposed rate control by Rate-λ model can achieve high accurate rate control results, we can also intentionally hierarchically allocate bits among pictures in GOP, because this kind of hierarchical bit allocation brings non-negligible performance improvements for both subjective quality [3] and objective quality [4]. We know that the bitrate variation among pictures may not be welcomed by some applications, but for the applications which allow a relative larger (larger than one picture, maybe a GOP, such as four pictures) buffer, the performance advantage of the hierarchical bit allocation will apparent. As the HEVC common test condition uses the hierarchical bit allocation among pictures, our proposed rate control algorithm supports both the hierarchical bit allocation and normally even bit allocation for different applications. If the hierarchical bit allocation is enabled, better coding efficiency can be obtained. The summary results are shown in Table 9 and the detailed results can be seen from JCTVC-J0242_RC AnchorRate_H1.xlsm and JCTVC-J0242_RC CfPRate_H1.xlsm. The results from here use the original HM-7.0 rate control algorithm as the anchor.

Table 9 Rate Control Results using Rate-λ model (hierarchical bit allocation)

|  | Targeting HM-7.0 Anchor bitrate |  | Targeting CfP Anchor bitrate |  |
| --- | --- | --- | --- | --- |
|  | Average bitrate error | Maximum bitrate error | Average bitrate error | Maximum bitrate error |
| HM-7.0 RC | 0.23% | 3.36% | 0.18% | 2.38% |
| Rate-λ RC | 0.12% | 1.18% | 0.10% | 0.69% |
| Y PSNR improvement | 0.61 dB |  | 0.65 dB |  |

Fig. 2 shows some per picture bit cost of the proposed rate control compared with the original HM-7.0 anchor without rate control algorithm. We can know that when we intentionally enable the hierarchical bit allocation, the per-picture bit allocation is quite similar with the hierarchical bit allocation in original HM without rate control. The proposed rate control algorithm can achieve the target bitrate perfectly while keeping this kind of hierarchical bit allocation.

Fig.2 Per-picture bit cost for hierarchical bit allocation

# References

- JCTVC-I0426, “QP determination by lambda value,” Geneva, CH, April – May 2012.

- VCEG-AM91, “Joint Call for Proposals on Video Compression Technology,” Kyoto, JP, Jan. 2010.

- JCTVC-A204, “Report of subjective testing of responses to Joint Call for Proposals (CfP) on video coding technology for High Efficiency Video Coding (HEVC),” Dresden, DE, Apr. 2010.

- JCTVC-D093, “Reference Lists For B Pictures Under Low Delay Constraints”, Daegu, KR, Jan. 2011.

- JCTVC-J0057, “Adaptive rate control for HEVC,” Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).