| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0244 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0053 and JCTVC-J0093 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a cross-check report of JCTVC-J0053 and JCTVC-J0093. Both documents propose to reverse the order for transform skipping TUs. JCTVC-J0093 further applies it to TUs in lossless CUs. The results obtained match those reported by the proponents.

# Introduction

Cross-check requests were made by MediaTek (JCTVC-J0053) and RIM (JCTVC-J0093) separately. When looking into the softwares, I realized that both proposals present the same method to reverse the scanning order for transform skipping TUs. JCTVC-J0093 further proposes to reverse the scanning order for lossless TUs.

The software of JCTVC-J0053 was provided by MediaTek and the software of JCTVC-J0093 was provided by RIM. The executable files were compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Code study and test results

Both JCTVC-J0053 and JCTVC-J0093 propose to reverse the scanning order for transform skipping TUs. But they have different modifications to the HM7.0 software.

In the software of JCTVC-J0053, the method is implemented as

Forward process: psCoeff[j * height + k] = piBlkResi[(height-1-j) * uiStride + (width-1-k)] << iTransformShift;

Inverse process: pResidual[j * uiStride + k] = (plCoef[(height-1-j) * width + (width-1-k)] + iAdd) >> iTransformShift;

While in the software of JCTVC-J0093, the method is implemented as

Forward process: psCoeff[(height-j-1)*height + width-k-1] = piBlkResi[ j * uiStride + k ] << iTransformShift;

Inverse process: pResidual[j * uiStride + k] =  (plCoef[(height-j-1)*width+ width-k-1] + iAdd) >> iTransformShift;

It is not difficult to see that both implementations are essentially the same. And JCTVC-J0053 and JCTVC-J0093 show exactly the same results under HE10 common test conditions. Thus, it can be considered as the situation of independent implementations. Conceptually they have verified each other’s results under HE10 common test conditions. Thus, I did not run the test under HE10 common test conditions to get yet another set of duplicate results.

JCTVC-J0093 provides more tests when RDOQ is off. It also extends the method to lossless coding and lossless coding results. Thus, I performed the tests for the case of RDOQ=0 and the case of lossless coding. The RD results I obtained, as shown in Table 1 and 2, are identical to those shown in JCTVC-J093.

Table 1. Results when RDOQ is off. Details can be found in JCTVC-J0244_1.xlsm.

Table 2. Results of lossless coding. Details can be found in JCTVC-J0244_2.xlsm.