| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0246 |
| --- | --- |

| Title: | On Temporal Layer Access Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Youngo Park Ilkoo Kim Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com youngo.park@samsung.com ilkoo.kim@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Similarly to the BLA picture, a new picture type “broken link TLA (BLT)” is proposed for identifying the TLA pictures with a broken link or a temporal layer switching. The leading pictures associated with the TLA or BLT is also marked as TFD pictures for easy discarding in systems.

# Introduction

In the last Geneva meeting, the broken link access (BLA) pictures, the tagged for discard (TFD) pictures and their corresponding NAL unit types were adopted. The BLA picture identifies the picture to be a CRA picture with a "broken link". The TFD pictures are the “leading pictures” associated with the previous CRA or BLA picture in decoding order and precedes the associated picture in output order. When the associated picture is a BLA picture or a CRA picture that is the first coded picture in the bitstream, the TFD picture may not be correctly decoded. In decoders, the TFD pictures of a BLA picture may simply be ignored (removed from the bitstream and discarded), as they are not specified for output and have no effect on the decoding process of any other pictures that are specified for output.

For temporal layer switching, the temporal layer access (TLA) picture, of which concept is similar to the CRA picture, was adopted. A TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order. The main difference between the CRA and TLA is that the temporal_id of the CRA picture is equal to 0, while the temporal_id of the TLA picture is not equal to 0.

The leading pictures associated with TLA pictures are also present. In figure 1, one leading picture is associated with the TLA picture. This leading picture can not be correctly decoded, when a temporal layer switching to upper layer is performed at the temporal switching point associated with the TLA picture. The leading pictures of TLA pictures should be also treated as TFD pictures, similarly to the case of CRA and BLA pictures. For easy discarding those TFD pictures associated with the TLA pictures in system/network level, the TLA pictures with “layer switching” or “broken link” should be identified by a new picture type, similarly to the BLA pictures.

Figure 1	. Categorization of pictures compared to the current picture X.

Figure 2	. Categorization of pictures compared to the current picture X.

# Proposal

We suggest to add a new picture type, broken link TLA (BLT) picture, which indentifies the TLA picture with any broken link of enhancement temporal layer (temporal_id > 0) or any temporal layer switching to upper layer. When BLT picture is present in the middle of stream, the bitstream would be allowed to contain "leading pictures" of the BLT picture that are not decodable due to missing preceding reference pictures. Those leading pictures will be marked as TFD pictures and will be ignored and discarded in actual decoders. In figure 2, when the temporal layer switching is performed at a TLA picture, the TLA picture will become the BLT picture and the associated TFD pictures is not decodable.

In summary, similarly to the BLA picture, a new picture type BLT is proposed for identifying the TLA pictures with broken links. The leading pictures associated with the TLA or BLT is also marked as TFD pictures.

# Syntax change

NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9, 10 | Coded slice of a BLT picture slice_layer_rbsp( ) | VCL |
| 11..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

- NOTE 4 - A CRA picture having nal_unit_type equal to 4 may have associated TFD pictures present in the bitstream. A CRA picture having nal_unit_type equal to 5 does not have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 6 may have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 7 does not have associated TFD pictures present in the bitstream. A BLT picture having nal_unit_type equal to 9 may have associated TFD pictures present in the bitstream. A BLT picture having nal_unit_type equal to 10 does not have associated TFD pictures present in the bitstream.

tagged for discard (TFD) picture: A coded picture for which each slice has nal_unit_type equal to 2; a TFD picture is associated with the previous CRA picture, BLA picture or BLT picture in decoding order and precedes the associated picture in output order; when the associated picture is a BLA picture, a BLT picture, or a CRA picture that is the first coded picture in the bitstream, the TFD picture may not be correctly decodable and is not output.

It is a requirement of bitstream conformance that no TFD pictures shall be present in the bitstream that are associated with a CRA picture having nal_unit_type equal to 5, or a BLA picture having nal_unit_type equal to 7, or a BLT picture having nal_unit_type equal to 9.

# Decoding process

8.1    General

–	PicOutputFlag is set as follows:

–	If the first coded picture in the bitstream is a CRA picture, and the current picture is a TFD picture associated with the CRA picture, or if the previous RAP picture preceding the current picture in decoding order is a BLA picture and the current picture is a TFD picture associated with the BLA picture, or if the previous temporal switching point picture preceding the current picture in decoding order is a BLT picture, and the current picture is a TFD picture associated with the BLT picture, PicOutputFlag is set equal to 0 and the decoding process for generating unavailable reference pictures specified in subclause Error: Reference source not found is invoked (only needed to be invoked for one slice of a picture).

### 8.3.2   Decoding process for reference picture set

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

- Unless either of the following conditions is true, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture": a) the first coded picture in the bitstream is a CRA picture and the current coded picture is a TFD picture associated with the first coded picture in the bitstream; b) the previous RAP picture preceding the current coded picture in decoding order is a BLA picture and the current coded picture is a TFD picture associated with the BLA picture; c) the previous temporal switching point picture preceding the current picture in decoding order is a BLT picture, and the current picture is a TFD picture associated with the BLT picture.

### 8.3.3   Decoding process for reference picture set

### 8.3.3.1  General

This process is invoked once per coded picture, after the invocation of the decoding process for reference picture set as specified in subclause , when the first coded picture in the bitstream is a CRA picture and the current coded picture is a TFD picture associated with the CRA picture, or when the previous RAP picture preceding the current coded picture in decoding order is a BLA picture and the current coded picture is a TFD picture associated with the BLA picture, or when the previous temporal switching point picture preceding the current picture in decoding order is a BLT picture, and the current picture is a TFD picture associated with the BLT picture.

When the first coded picture in the bitstream is a CRA picture and the current coded picture is a TFD picture associated with the CRA picture, or when the previous RAP picture preceding the current coded picture in decoding order is a BLA picture and the current coded picture is a TFD picture associated with the BLA picture, the following applies, or when the previous temporal switching point picture preceding the current picture in decoding order is a BLT picture, and the current picture is a TFD picture associated with the BLT picture.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).