| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0248 m25576 |
| --- | --- |

| Title: | On prevRefPic definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document claims that the current definition of prevRefPic breaks the TLA design and requires the decoder to store more values of pic_order_cnt_lsb and PicOrderCntMsb. Therefore, this document proposes to revert the change of the definition of prevRefPic that was made at the previous JCT-VC meeting and use the Comitte Draft definition. Instead of using the previous reference picture that has a temporal_id equal to or less than the temporal_id of the current picture for prevRefPic, this document proposes to use the previous reference picture that has a temporal_id equal to 0.

# Problem Statement

In response to JCTVC-I0345 it was adopted to change the definition of prevRefPic from

“Let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic”

to

“Let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to or less than the temporal_id of the current picture. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic”

## Problems with the new definition

The proposal was motivated by a case where only the IDR pictures and CRA pictures are coded in the lowest temporal layer and that “In some cases, the DPB may contain no picture with temporal_id equal to 0”. However, the only values that are needed from the prevRefPic is its pic_order_cnt_lsb and its PicOrderCntMsb. There is no requirement that the picture prevRefPic is available in the DPB.

With the new definition of prevRefPic, a decoder needs to store up to eight different pic_order_cnt_lsb values and PicOrderCntMsb values, one for each temporal layer. With the old definition, only one pic_order_cnt_lsb value and one PicOrderCntMsb value needs to be stored.

## More problems with the new definition

With the new definition of prevRefPic the TLA picture functionality is broken. Consider the example in figure 1.

Figure 1.

If the two “normal” P pictures are removed in temporal scaling and temporal switching is performed at the TLA picture the prevRefPic will be different (Picture with POC = 0 instead of picture with POC = 8) and the PicOrderCntVal will be wrong for the TLA picture as shown in figure 2.

Figure 2.

Figure 2 shows that the TLA picture in the example will get a PicOrderCntVal that is lower than IDR picture and the output order of the pictures will be incorrect.

# Proposal

We propose that the definition of prevRefPic is reverted back to the definition from the Committee Draft:

“Let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic”

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).