| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0250 m25578 |
| --- | --- |

| Title: | Indication of non-reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to change the semantics of nal_ref_flag so that in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer. This contribution claims that this makes it possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed and thus indicate whether that picture can safely be removed from the sub-stream without affecting the decoding of the remaining pictures in the substream.

# Background

The nal_ref_flag is the second bit in the HEVC NAL unit header. It primary purpose is to indicate whether the current picture is a reference picture (nal_ref_flag=1) or a non-reference picture (nal_ref_flag=0).

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| ... |  |

In section 8.3.5 of the HEVC draft specification it is stated that if nal_ref_flag of the current picture is equal to 0, the current picture is marked as "unused for reference", otherwise, the current picture is marked as "used for short-term reference".

Since the reference picture set (RPS) design marks pictures after the slice header has been decoded, the marking of the current picture after it has been decoded has no real mandatory effect. A decoder does not need to mark decoded pictures according to nal_ref_flag; it can instead mark all of them "used for short-term reference". If the picture was a non-reference picture, the decoder would anyway mark the picture unused for reference after the first slice header of the next picture is received. The nal_ref_flag could therefore be regarded as informative.

# Problem Statement

A problem with the current meaning of nal_ref_flag is that it is not possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed. If the highest layer of the picture structure below is removed, it is not possible to indicate that pictures of the middle layer are not reference pictures and individually discardable.

One possible solution would be to rewrite nal_ref_flag when a layer is removed. But it is very difficult for a decoder or network node to deduce whether a picture in a sub-stream with nal_ref_flag=1 can be marked as non-reference by setting nal_ref_flag to 0 when a higher layer has been removed.

Removing an entire temporal layer is always possible since lower temporal layers cannot use pictures in higher temporal layers for reference. The highest layer can thus safely be removed. But the second highest layer is generally used for predicting the highest layer and the pictures in that layer must therefore have nal_ref_idc equal to 1. But when nal_ref_idc is equal to 1 for a picture in the middle layer, a network node will not know whether that picture will be referenced by a future picture of the same temporal layer or not. If it is a reference picture for a future picture in the same temporal layer, the picture can not be removed without decoding errors. For a network node to determine whether a picture A is used for reference or not, it must decode information from pictures following A in decoding order in order to verify that picture A is not used for reference by a picture of the same temporal layer. This includes keeping track of picture marking of future pictures. It is complicated and will induce latency in the node.

# Proposal

We propose that the semantics of nal_ref_flag is changed so that it in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer:

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a picture that may be included in the reference picture set of a picture of the same temporal layer. nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a picture that is not included in the reference picture set of any other picture of the same temporal layer.

# Proposed text

3.72 non-reference picture: A picture coded with nal_ref_flag equal to 0. A non-reference picture is not used for inter prediction of any other pictures.

3.98 reference picture: A picture with nal_ref_flag equal to 1. A reference picture that contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture that may be included in the reference picture set of a picture of the same temporal layer.

nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture. nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a picture that is not included in the reference picture set of any other picture of the same temporal layer.

nal_ref_flag shall be equal to 1 for video parameter set, sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one VCL NAL unit of a particular picture, it shall be equal to 0 for all VCL NAL units of the particular picture. All VCL NAL units of a particular picture shall have nal_ref_flag equal to the same value.

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4, 5, 6, 7, or 8.

nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 29, 30, or 31.

##### 7.4.1.2.2 Order of access units and association to coded video sequences

The values of picture order count for the coded pictures in consecutive access units in decoding order containing non reference pictures shall be increasing.

## 8.1 General

After all slices of the current picture have been decoded, if it is a reference picture the decoded reference the picture is marked as "used for short-term reference", otherwise (it is a non-reference picture) the decoded picture is marked as "unused for reference", as specified in subclause 8.3.5.

##### 7.4.1.2.4 Detection of the first VCL NAL unit of a coded picture

–	nal_ref_flag differs in value with one of the nal_ref_flag values being equal to 0.

### 8.3.5 Marking of the current picture after decoding

This process is invoked after all slices of the current picture have been decoded.

–	If nal_ref_flag of the current picture is equal to 0, the current picture is marked as "unused for reference".

–	Otherwise, tThe current picture is marked as "used for short-term reference".

### C.5.3 Picture decoding, marking and storage

If tThe current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).