| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0251 m25579 |
| --- | --- |

| Title: | Restrictions on leading pictures of CRA and BLA |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document proposes two restrictions on pictures that are leading pictures of clean random access (CRA) pictures and broken link access (BLA) pictures. A leading picture is defined as a coded picture that follows some other particular picture in decoding order and precedes it in output order. The two proposed restrictions are:

- All leading pictures of CRA and BLA pictures must directly follow the CRA/BLA picture in decoding order.

- All leading pictures of CRA and BLA pictures must be tagged for discard (TFD) pictures.

It is argued that these restrictions will improve the possibilities for units operating at the bitstream, such as media-aware network elements (MANEs) or splicing units.

# Introduction

At the ninth JCTVC meeting in Geneva the broken link access (BLA) picture type and the tagged for discard (TFD) picture type were adopted. These new NAL unit types are intended to provide network elements and splicing elements with more information in the NAL unit header (via the NAL unit type) so that there is no need to parse the slice header in order to deduce how to perform splicing and random access.

# Proposal 1

In order to improve the usability of the random access NAL unit types for an element that wants to perform random access or splicing and discard the leading pictures we propose the following:

All leading pictures of CRA and BLA pictures must directly follow the CRA/BLA picture in decoding order.

Figure 1. Proposal 1.

With proposal 1 the leading pictures of the CRA/BLA picture will all be together in the bitstream directly after the CRA/BLA picture. An element that wants to perform random access or splicing and discard the leading pictures will then only need to discard from one part of the bitstream. Once it receives a picture that is not a leading picture of the CRA/BLA picture it will know that no more leading pictures of that CRA/BLA picture will occur in the future.

# Proposal 2

To further improve the usability of the random access NAL unit types we propose to add the following restriction:

All leading pictures of CRA and BLA pictures must be tagged for discard (TFD) pictures.

Figure 2. Proposal 2.

With the current design (even after proposal 1) it is possible to use a mixture of TFD pictures and “normal” pictures when encoding leading pictures of a CRA picture.

We do not see a use case for mixing leading pictures that are TFD pictures and leading pictures that are not TFD. In fact, we consider it an advantage to make a clear distinction: IDR pictures can only have leading pictures that are not TFD pictures (as in the current design). CRA and BLA pictures can only have leading pictures that are TFD pictures (proposal 2).

It can be noted that the proposed restriction does not imply that all leading pictures of CRA and BLA pictures must use inter prediction from pictures preceding the CRA/BLA in decoding order. The leading pictures can still be encoded for example using only the CRA/BLA picture for reference. The restriction only requires that also those pictures use NAL unit type equal to TFD (which is currently not the case).

For an element that wants to perform random access or splicing and discard the leading pictures of CRA and BLA pictures it is much more convenient if all leading pictures are encoded with TFD NAL unit type, especially in combination with proposal 1 since that will make it possible to remove one consecutive part of the bitstream only based on the NAL unit type.

# Proposed text

leading picture: A coded picture that follows some other particular picture in decoding order and precedes it in output order.

tagged for discard (TFD) picture: A coded picture for which each slice has nal_unit_type equal to 2; a TFD picture is associated with the previous CRA picture or BLA picture in decoding order and precedes the associated picture in output order; when the associated picture is a BLA picture, or when the associated picture is a CRA picture that is the first coded picture in the bitstream, the TFD picture may not be correctly decodable and is not output.

It is a requirement of bitstream conformance that any coded picture that follows a RAP picture in decoding order and uses inter prediction either from any picture that precedes the RAP picture in decoding order or from another TFD picture associated with the RAP picture shall be a TFD picture associated with the RAP picture.

It is a requirement of bitstream conformance that all leading pictures of CRA and BLA pictures shall be TFD pictures.

It is a requirement of bitstream conformance that the picture directly preceding the TFD picture in decoding order is either its associated CRA or BLA picture or another TFD.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).