| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0252-v2 |
| --- | --- |

| Title: | AHG9: Cross-check of JCTVC-J0221: Clean-up of semantics and decoding process on weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Pierre Andrivon 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33-2-99-27-32-42 philippe.bordes@technicolor.com pierre.andrivon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results on the range limit of the signalling values of the weighting factor and the additive offset.

In the JCTVC-J0221 proposal (fix point 2), the signalling values (delta weights) of the weighting factor should be in the range of -128 to 127, inclusive and the signalling value (delta offsets) of the additive offset is in the range of -512 to 511, inclusive. In addition, the reconstructed additive offset is clipped to the range of -128 to 127.

The verification task has been done successfully and the results match those provided by proponent. The encoding time and the decoding time of this cross-check are also similar to those provided by proponents.

The experimental results in HM software version 7.0 with weighted prediction under common test conditions are reported. The results show that BD-bitrate loss for both black/white-fade sequences are 0.0%, 0.0% and 0.0% for RA-MP, LDB-MP and LDP-MP compared with conventional WP method on average, respectively. It is reported that these schemes does not affect the encoding and decoding time.

# Cross-checking

The software based on HM version 7.0 and the experimental results were provided on June 29, 2012 by Toshiba. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [1] for Main profile (MP) conditions and high efficiency (HE10) conditions in the case of random access (RA), low delay B (LB) and low delay P (LP) coding structures using black/white-fade sequences provided by [2].

The anchors were generated using the HM7.0. In the software provided by the proponents one added macro were set as follows:

#define WP_PARAM_RANGE_LIMIT             1

# Results

Table 1 and Table 2 show BD-bitrate and relative average encoding/decoding time for the proposed signalling values limitations compared with HM7.0 weighted prediction anchor. The negative value of BD-bitrate indicates gain. It is observed that the loss of the proposal is 0.0%, 0.0% and 0.0% on average for RA-MP, LDB-MP and LDP-MP and 0.1%, 0.0% and 0.0% on average for RA-HE10, LDB-HE10 and LDP-HE10 in black-fade case, respectively. And the loss of the proposed fix is 0.0%, 0.0% and 0.0% on average for RA-MP, LDB-MP and LDP-MP and 0.1%, 0.0% and 0.0% on average for RA-HE10, LDB-HE10 and LDP-HE10 in white-fade case, respectively.

Table 1: BD-bitrate[%] and relative encoding/decoding time[%] of the fix of point 2 of JCTVC-J0221 compared with HM7.0 WP anchor for black-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.1% | 0.1% | 0.3% | 0.3% | -1.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | -0.4% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 97% |  |  |

Table 2: BD-bitrate[%] and relative encoding/decoding time[%] of the fix of point 2 of JCTVC-J0221 compared with HM7.0 WP anchor for white-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.2% | 0.2% | 0.4% | 0.2% | -1.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.3% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% | -0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | 0.0% | -0.4% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

# Conclusion

We cross-checked the clean-up text of the weighted prediction semantics and decoding process proposed by Toshiba. We confirm the results presented in JCTVC-J0221.

Regarding the signaling of weighted prediction in PPS, we propose to use a single non_default_weight_pred_flag (in place of 2 separate flags for P (weight_pred_flag) and B slices (weighted_bipred_flag)) to indicate whether the default weighted sample prediction process (0) or (non-default, explicit) weighted sample prediction process (1) shall be applied for P and B slices.

non_default_weighted_pred_flag equal to 0 specifies that weighted prediction shall not be applied to P slices and the default weighted prediction is applied to B slices. non_default_weighted_pred_ flag equal to 1 specifies that weighted prediction is applied to P and B slices.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-I1100. May 2012.

- AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).