| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0254 |
| --- | --- |

| Title: | Position-based Context Derivation for coeff_abs_level_greater1_flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianying Ji Xiang Yu Dake He Gaëlle Martin-Cocher | Tel: Email: | (519)888-7465 tiji@rim.com xiayu@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a position-based context derivation scheme for the syntax element coeff_abs_level_greater1_flag coding. In the current HM7.0 design, the context indices for coeff_abs_level_greater1_flag can only be derived in a sequential manner, since the context for one coeff_abs_level_greater1_flag is dependent on its immediately previous coeff_abs_level_greater1_flag . It is proposed here to derive contexts for coeff_abs_level_greater1_flag in a CG based on the position of the coeff_abs_level_greater1_flag in the scanning order. Thus, it completely removes the context dependency for coeff_abs_level_greater1_flag within a CG, and allows all context indices for coeff_abs_level_greater1_flag in one CG to be derived in parallel. The BD rate penalty is on average 0.1%.

# Introduction

While the context modeling design in HM7.0 greatly improves the coding rate efficiency, it sees an inter-bins dependency issue with its context derivation for coeff_abs_level_greater1_flag coding, which holds back the maximal throughput to 1bin/cycle for coeff_abs_level_greater1_flag. The context derivation for coeff_abs_level_greater1_flag may be illustrated as a finite state machine shown in Figure 1. The tight dependency has become one of the most important bottlenecks in high throughput entropy coding design for HEVC [1].

Figure 1. The finite state machine for context selection for coding coeff_abs_level_greater1_flag in HM7.0

A similar throughput issue existed in HM6.0 for the significance flag coding and had been resolved in HM7.0 [2]. Specifically, position-based context derivation was used to replace the template-based method, and improved the maximum throughput for significance flags from 2bin/cycle to 16bin/cycle.

This contribution proposes a unified design based on positions for coeff_abs_level_greater1_flag context derivation, as [2] did for sig-flag, and addresses the last throughput bottleneck in residual coding.  A position-based solution is used to replace the finite-state machine for coeff_abs_level_greater1_flag context derivation.

# Proposed solution

The essence in the state machine of Figure 1 is the switching from trailing ones to levels greater than one, in other words, the switch from a state where the probability of seeing a level being 1 (gr1=0) is high to a state where the probability of seeing a level being greater than 1 (gr1=1) is high. Instead of using the value of the current level to determine the switch as in Figure 1, it is proposed to use the position of the last nonzero coefficient in a CG and the position of the coeff_abs_level_greater1_flag in its coding order. Also, since the coeff_abs_level_greater1_flag context set selection reveals statistics difference of coeff_abs_level_greater1_flag, the ctxSet is also used to further differentiate the context selection.

For context set 0 and 2, the context selection may be illustrated by the following table, based on the position of the last nonzero and the index of the coeff_abs_level_greater1_flag in its coding order.

| Position of the last NZ | Context selection for coeff_abs_level_greater1_flag ( bin0 to bin7) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin0 | Bin1 | Bin2 | Bin3 | Bin4 | Bin5 | Bin6 | Bin7 |
| 0,1 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2,3 | 1 | 2 | 3 | 3 | 0 | 0 | 0 | 0 |
| 4,5 | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |
| 6,7,8,9 | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |
| 10,11 | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |
| 12,13 | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |
| 14,15 | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |

The above tables corresponds to a case where Gr1=1 is rarely seen in previous CGs. Thus the number of trailing ones increases as the number of nonzero increases.

For context set 1 and 3, the context selection may be illustrated by the following table, based on the position of the last nonzero and the index of the coeff_abs_level_greater1_flag in its coding order.

| Position of the last NZ | Context selection for coeff_abs_level_greater1_flag,( bin0 to bin7) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin0 | Bin1 | Bin2 | Bin3 | Bin4 | Bin5 | Bin6 | Bin7 |
| 0,1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2,3 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4,5 | 1 | 2 | 3 | 0 | 0 | 0 | 0 | 0 |
| 6,7,8,9 | 1 | 2 | 3 | 3 | 0 | 0 | 0 | 0 |
| 10,11 | 1 | 2 | 3 | 0 | 0 | 0 | 0 | 0 |
| 12,13 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 14,15 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The above table corresponds to a case where Gr1=1 is observed in previously coded CG. Thus, larger levels are more likely to be seen and the number of trailing ones is no more than 2 when the whole CGs are nonzeros.

# Results

3.1 Common Test Condition Result with RDOQ On

The performance of the proposed change compared against the HM7.0 anchor with RDOQ on.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.18% | 0.22% | 0.19% | 0.17% | 0.23% | 0.20% |
| Class B | 0.11% | 0.18% | 0.18% | 0.11% | 0.16% | 0.13% |
| Class C | 0.14% | 0.17% | 0.19% | 0.12% | 0.16% | 0.12% |
| Class D | 0.13% | 0.21% | 0.16% | 0.12% | 0.15% | 0.11% |
| Class E | 0.17% | 0.23% | 0.18% | 0.16% | 0.21% | 0.19% |
| Overall | 0.14% | 0.20% | 0.18% | 0.13% | 0.18% | 0.15% |
|  | 0.14% | 0.20% | 0.18% | 0.13% | 0.19% | 0.15% |
| Class F | 0.35% | 0.35% | 0.45% | 0.36% | 0.05% | 0.16% |
| Enc Time[%] | 104% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.15% | 0.28% | 0.03% | 0.33% | 0.71% |
| Class B | 0.05% | 0.19% | 0.08% | 0.06% | 0.01% | 0.08% |
| Class C | 0.09% | 0.22% | 0.14% | 0.09% | 0.22% | 0.03% |
| Class D | 0.12% | 0.20% | 0.03% | 0.10% | 0.09% | -0.30% |
| Class E |  |  |  |  |  |  |
| Overall | 0.06% | 0.12% | 0.13% | 0.07% | 0.16% | 0.13% |
|  | 0.12% | 0.13% | 0.14% | 0.08% | 0.12% | 0.10% |
| Class F | 0.26% | 0.31% | 0.23% | 0.24% | 0.25% | -0.19% |
| Enc Time[%] | 106% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | 0.21% | -0.05% | 0.03% | 0.23% | -0.04% |
| Class C | 0.04% | 0.11% | 0.02% | 0.04% | 0.21% | 0.23% |
| Class D | 0.08% | 0.83% | -0.06% | 0.09% | 0.38% | -0.53% |
| Class E | -0.01% | 0.07% | 0.43% | 0.24% | -0.23% | -0.11% |
| Overall | 0.04% | 0.31% | 0.05% | 0.09% | 0.18% | -0.11% |
|  | 0.04% | 0.29% | 0.05% | 0.09% | 0.21% | -0.11% |
| Class F | 0.15% | 0.63% | 0.34% | 0.43% | 0.84% | 0.60% |
| Enc Time[%] | 107% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

Note: Enc/Dec time is not reliable due to the different computers in cluster.

3.2 Common Test Condition Result with RDOQ Off

The performance of the proposed change compared against the HM7.0 anchor with RDOQ off.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.15% | 0.16% | 0.13% | 0.14% | 0.20% | 0.20% |
| Class B | 0.09% | 0.12% | 0.13% | 0.09% | 0.12% | 0.13% |
| Class C | 0.11% | 0.11% | 0.11% | 0.10% | 0.10% | 0.10% |
| Class D | 0.10% | 0.10% | 0.15% | 0.10% | 0.05% | 0.09% |
| Class E | 0.14% | 0.22% | 0.22% | 0.13% | 0.17% | 0.18% |
| Overall | 0.12% | 0.14% | 0.14% | 0.11% | 0.12% | 0.14% |
|  | 0.12% | 0.13% | 0.14% | 0.11% | 0.13% | 0.14% |
| Class F | 0.29% | 0.26% | 0.29% | 0.33% | 0.20% | 0.31% |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 112% |  |  | 110% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.12% | 0.23% | 0.05% | 0.47% | 0.10% |
| Class B | 0.08% | 0.01% | 0.07% | 0.06% | 0.15% | 0.21% |
| Class C | 0.02% | 0.09% | 0.09% | 0.07% | 0.18% | 0.17% |
| Class D | 0.05% | 0.30% | -0.12% | 0.00% | -0.01% | -0.10% |
| Class E |  |  |  |  |  |  |
| Overall | 0.04% | 0.06% | 0.07% | 0.04% | 0.20% | 0.10% |
|  | 0.04% | 0.06% | 0.07% | 0.05% | 0.16% | 0.09% |
| Class F | 0.20% | -0.04% | 0.05% | 0.28% | 0.13% | 0.34% |
| Enc Time[%] | 104% |  |  | 112% |  |  |
| Dec Time[%] | 104% |  |  | 113% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | 0.03% | -0.10% | 0.04% | 0.26% | -0.11% |
| Class C | 0.08% | 0.11% | -0.04% | 0.01% | 0.17% | 0.08% |
| Class D | 0.04% | 0.04% | 0.01% | 0.09% | 0.37% | 0.16% |
| Class E | 0.01% | -0.77% | 0.23% | 0.05% | 0.80% | 0.17% |
| Overall | 0.05% | -0.10% | 0.00% | 0.05% | 0.37% | 0.06% |
|  | 0.05% | -0.14% | -0.02% | 0.05% | 0.42% | 0.15% |
| Class F | 0.08% | 0.06% | -0.51% | 0.24% | 0.20% | -0.59% |
| Enc Time[%] | 106% |  |  | 102% |  |  |
| Dec Time[%] | 107% |  |  | 102% |  |  |

Note: Enc/Dec time is not reliable due to the different computers in cluster.

3.3 Low QP Test Result with RDOQ On

The performance of the proposed change compared against the HM7.0 anchor with RDOQ on, with low QP value 12, 17, 22, 27.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.20% | 0.19% | 0.19% | 0.20% | 0.17% | 0.18% |
| Class B | 0.12% | 0.17% | 0.15% | 0.12% | 0.15% | 0.13% |
| Class C | 0.20% | 0.22% | 0.20% | 0.17% | 0.19% | 0.17% |
| Class D | 0.24% | 0.22% | 0.20% | 0.21% | 0.18% | 0.15% |
| Class E | 0.24% | 0.31% | 0.23% | 0.25% | 0.31% | 0.25% |
| Overall | 0.19% | 0.22% | 0.19% | 0.18% | 0.19% | 0.17% |
|  | 0.20% | 0.22% | 0.19% | 0.19% | 0.20% | 0.17% |
| Class F | 0.48% | 0.40% | 0.38% | 0.39% | 0.19% | 0.29% |
| Enc Time[%] | 103% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.15% | 0.03% | 0.13% | 0.12% | 0.17% | 0.16% |
| Class B | 0.09% | 0.17% | 0.19% | 0.07% | 0.12% | 0.13% |
| Class C | 0.19% | 0.28% | 0.19% | 0.14% | 0.23% | 0.18% |
| Class D | 0.20% | 0.10% | 0.08% | 0.19% | 0.12% | 0.07% |
| Class E |  |  |  |  |  |  |
| Overall | 0.15% | 0.15% | 0.15% | 0.13% | 0.16% | 0.14% |
|  | 0.16% | 0.16% | 0.18% | 0.13% | 0.15% | 0.14% |
| Class F | 0.36% | 0.27% | 0.20% | 0.26% | 0.33% | 0.17% |
| Enc Time[%] | 107% |  |  | 103% |  |  |
| Dec Time[%] | 107% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.07% | 0.10% | 0.16% | 0.05% | 0.10% | 0.16% |
| Class C | 0.15% | 0.26% | 0.21% | 0.13% | 0.22% | 0.12% |
| Class D | 0.19% | 0.22% | 0.16% | 0.17% | 0.20% | -0.09% |
| Class E | -0.02% | -0.04% | -0.08% | 0.06% | 0.37% | 0.01% |
| Overall | 0.10% | 0.14% | 0.13% | 0.10% | 0.21% | 0.06% |
|  | 0.11% | 0.15% | 0.15% | 0.10% | 0.26% | 0.04% |
| Class F | 0.36% | 0.38% | 0.30% | 0.35% | 0.36% | 0.73% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |

Note: Enc/Dec time is not reliable due to the different computers in cluster.

# Conclusions

This contribution proposes a position based context derivation method for coeff_abs_level_greater1_flag. It completely removes the dependency within a CG for coeff_abs_level_greater1_flag context derivation and allows deriving all coeff_abs_level_greater1_flag contexts in parallel for high throughput designs.

# Patent rights declaration(s)

Research In Motion Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text changes for Solution

##### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the colour component index cIdx, the 16 coefficient subset index i, the current coefficient scan index n within the current subset, the coding order index of the current coeff_abs_level_greater1_flag idx, and the last nonzero position in CG lastNZPosInCG.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-1)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-2)

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause Error: Reference source not found for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-3)

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- If ctxSet is 0 or 2, greater1Ctx is derived using gr1Ctx_0_2[] specified in Table 9-44 as follows:

- greater1Ctx = gr1Ctx_0_2[idx]    (9-33)

- Otherwise, greater1Ctx is derived using gr1Ctx_1_3[] specified in Table 9-45 as follows:

- greater1Ctx = gr1Ctx_1_3[lastNZPosInCG][idx]

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )	(9-4)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  16

Table 9-xx – Specifcation of gr1Ctx_0_2[ idx ]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gr1Ctx_0_2[ idx ] | 1 | 2 | 3 | 3 | 3 | 0 | 0 | 0 |

Table 9-45 – Specifcation of gr1Ctx_1_3[lastNZPosInCG][ idx ]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastNZPosInCG |  |  |  |  |  |  |  |  |
| 0, 1, 14, 15 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2, 3, 12, 13 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4, 5, 10, 11 | 1 | 2 | 3 | 0 | 0 | 0 | 0 | 0 |
| 6, 7, 8, 9 | 1 | 2 | 3 | 3 | 0 | 0 | 0 | 0 |

# Reference

[1] W.-J. Chien, J. Chen, J. Sole, and M. Karczewicz, “CE3 subtest 2.1: conrtext assignment for parallel coefficient level coding”, JCTVC-I0303, May 2012.

[2] T. Kumakura, S. Fukushima, “Non-CE3: Simplified context derivation for significance map”, JCTVC-I0296, May 2012.