| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 July – 20 July 2012 | Document: JCTVC- J0255 |
| --- | --- |

| Title: | Slice Prefix for sub-picture and slice level HLS signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Schierl V. George R. Skupin D. Marpe Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-227 thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document we propose to apply SEI messages as well as additional high level syntax items, beyond the ones included in the NAL unit header, on a per slice level basis. Therefore we described the Slice Prefix NAL Unit. We describe the syntax and semantics of the slice prefix and slice-level/sub-picture SEI messages, use cases for low delay/sub-picture CPB operations, tile signalling and region of interest (ROI) signalling.

# Introduction

HEVC [1] allows for different means of High Level Syntax signalling to the application layer. Such means are the NAL unit header, Parameter Sets and Supplemental Enhancement Information (SEI) Messages. The latter are not used in the decoding process.

In this proposal we describe an additional high level syntax signalling strategy, which allows to apply SEI messages as well as additional high level syntax items, beyond the ones included in the NAL unit header, on a per slice level. Therefore we described the Slice Prefix NAL unit. We describe the syntax and semantics of the slice prefix and slice-level/sub-picture SEI messages, use cases for low delay/sub-picture CPB operations, tile signalling and region of interest (ROI) signalling.

## Problems of the current design

The timing of the decoding units needs to be known before coding and sending the data in a low delay scenario, where NAL units will already be sent out by the encoder, while the encoder is still coding parts of the picture, i.e. other sub-picture decoding units. This is because the NAL unit order in an access unit only allows SEI messages to precede the VCL (Video Coding NAL units) in an access unit, but in such a low delay scenario, the non-VCL NAL units need to be already on the wire, i.e. sent out, if the encoder starts encoding the decoding units. The following figure illustrates the structure of an access unit as defined in the current draft specification. The current draft specification does not yet specify end of sequence or stream, so their presence in the access unit is tentatively.

Figure 1: Structure of an access unit according to the current draft specification.

Furthermore the, the number of NAL units associated with a sub-picture also needs to be known beforehand in a low delay scenario, as the picture timing SEI message contains this information and has to be compiled and send out before the encoder starts to encode the actual picture. An application designer reluctant to insert filler data NAL units, with potentially no filler data, to comply with the NAL unit number, as signaled per decoding unit in the picture timing SEI, needs means to signal this information on a sub-picture level. The same holds for sub-picture timing, which is currently fixed at the being of an access unit by the parameters given in the timing SEI message.

The signalling of further high-level information at sub-picture/slice level may be advantageous for various applications but current means of signalling such information would be restricted to the ue-coded slice header or additional NAL unit header bits. For devices not capable of entropy decoding, e.g. due to lack of computational resources (simple MANE) or time constraints (high-throughput MANE), prevent applications from making use of sub-picture/slice level information. This information includes, but is not limited to:

- region of interest signalling

- priority signalling

- tile information, such as tile identifiers

- tile dimensions in picture coordinates

- additional signalling for sub-picture presentation, such as display constraints or suggestions (e.g. in a large multi-monitor environment signalling)

For the information above, it may be advantageous to provide such information in the bitstream for stream processing devices such as MANEs,

# Proposal

## Proposal overview

A solution for the problems stated above is to allow the assignment of SEI messages, in the sub-picture CPB/HRD case, to slices of decoding units. I.e. the assignment of buffering period and timing SEI messages to the NAL units containing the slices of a decoding unit.

This can be achieved by a new NAL unit type, which is a non-VCL NAL unit, which is allowed to directly precede one or more slice/VCL NAL units of a decoding unit.

This new NAL unit is proposed later in this document as Slice Prefix NAL unit. The following figure illustrates the structure of an access unit in this proposal, omitting the tentative NAL units for end of sequence and stream.

Figure 2: Structure of an access unit according to proposal. The dashed line reflects the case of a non-mandatory slice prefix NAL unit.

All information signaled in the slice prefix and associated sub-picture SEI messages is either valid for all VCL-NAL units in the access unit or until the occurrence of a second prefix NAL unit or for the single following VCL-NAL unit in decoding order, depending on a flag given in the slice prefix NAL unit.

The slice VCL NAL units which the information signaled in the slice prefix is valid for are referred to as prefixed slices in the following. Prefixed slices associated with a single slice prefix do not necessarily constitute a complete decoding unit but can be a part of it. However, a single slice prefix cannot be valid for multiple decoding units ( sub-pictures) and the start of a decoding unit is signaled via a flag in the slice prefix. signalling

Only certain SEI message with (identified via payloadType in the syntax proposal section) are to be sent exclusively on sub-picture level within the slice prefix NAL unit, while some SEI messages can be sent either in the slice prefix NAL unit on sub-picture level or as a regular SEI message on access unit level.

Apart from the sub-picture/slice level SEI message concept in this proposal, the slice prefix NAL unit can be used to signal further sub-picture/slice level related information directly in the slice prefix syntax, directly readable by application layers. Such information can for example include the tile ID directly. In earlier designs of HEVC, the slice header/the slice data contained an identifier for the tile contained in the slice.

Old Section 7.4.4 Slice data semantics

tile_idx_minus_1 specifies the TileID in raster scan order.  The first tile in the picture shall have a TileID of 0.  The value of tile_idx_minus_1 shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1.

This parameter was not consider useful, since this ID can be easily derived from slice address and the slice dimensions as signaled in the Picture Parameter Set, if tiles_or_entropy_coding_sync_idc is equal to 1.

Although the tile id can be derived implicitly in the decoding process, the knowledge of this parameter on the application layer is also important for different use cases, e.g. in a video conferencing scenario, different tiles may have different priority for the playback (the tiles region of the ROI, which typically contains the speaker in a conversational use case) may have higher priority than other tiles. In case of losing network packets in the transmission of multiple tiles, those network packets containing tiles representing the ROI may be re-transmitted with higher priority in order to keep the quality of experience at the receiver terminal higher than in the case re-transmitting tiles without any priority order. Another use case may be to assign tiles, if the dimensions and their position is known, to different screens, e.g., in a video conferencing scenario.

In order to allow the application layer to handle tiles with a certain priority in transmission scenarios, the tile_id may be provided

- as an sub-picture or slice-specific SEI message or

in a special NAL unit in front of one or more NAL units of the tile. Furthermore, an SEI Message should be included to allow signalling of the Region of Interest (ROI) a tile-ID / Tile belongs to. The message should allow signalling ROI identifier and priority of the region of interest. The following figure illustrates the use of tiles in ROI signalling.

Figure 3: Exemplary illustration of tiles in region of interest signalling.

## Proposed syntax

We propose two versions of the slice prefix syntax, one with the functionality of SEI messaging only and one with the extended functionality of signalling sub-picture/slice level information outside the SEI message concept directly in the slice header for the following slices. The first simple version 1 syntax is given in the following table.

| slice_prefix_rbsp( ) { | Descriptor |
| --- | --- |
| reserved_one_6bits | u(6) |
| decoding_unit_start_flag | u(1) |
| single_slice_flag | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The extended version 2 syntax, including tile_id signalling, decoding unit start identifier, slice prefix id and slice header data apart from the SEI message concept is given in the following table.

| slice_prefix_rbsp( ) { | Descriptor |
| --- | --- |
| reserved_one_8bits | u(2) |
| slice_prefix_id | u(2) |
| decoding_unit_start_flag | u(1) |
| single_slice_flag | u(1) |
| tile_idc | u(2) |
| if( tile_idc > 0 ) { |  |
| if( tile_idc == 2 ) /* multiple tiles in prefixed slices */ |  |
| num_tiles_in_prefixed_slices_minus1 | u(16) |
| first_tile_id_in_prefixed_slices | u(16) |
| } |  |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

slice_prefix_id indicates the slice prefix identifier.

Note: This parameter may be used to identify the repetition of a slice prefix in the bitstream.

decoding_unit_start_flag indicates the start of a decoding unit within the access unit, thus that the following slices up to the end of the access unit or the start of another decoding unit belong to the same decoding unit.

single_slice_flag with a value of 0 indicates that the information provided with in the prefix slice NAL unit and the associated sub-picture SEI messages is valid for all following VCL-NAL units until the start of the next access unit, the occurrence of another slice prefix or another complete slice header. single_slice_flag with a value 1 indicates that all information provided in the slice prefix NAL unit and associated sub-picture SEI messages is valid only for the next VCL-NAL unit in decoding order.

tile_idc indicates the amount of tiles to be present in the following slice. tile_idc equal to 0 indicates that no tiles are used in the following slice. tile_idc equal to 1 indicates that a single tile is used in the following slice and its tile identifier is signaled accordingly. tile_idc with a value of 2 indicates that multiple tiles are used within the following slice and number of tiles and the first tile identifier are signaled accordingly.

num_tiles_in_prefixed_slices_minus1 indicates the number of tiles used in the following decoding unit minus 1.

first_tile_id_in_prefixed_slices indicates the tile identifier of the first tile in the following decoding unit.

The slice prefix NAL unit is proposed to have a NAL unit type of 24 and the NAL unit type overview table to be extended according to the following table.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9..234 | Reserved | n/a |
| 24 | Slice Prefix NAL unit | non-VCL |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

Of course, the proposed concept to shift part of the slice header data into the slice header prefix requires changes to the slice header as specified in the current draft specification. The following table proposes a syntax for the slice header, where certain syntax elements were shifts to a lower hierarchy syntax element, referred to as slice_header_data(). This following syntax of the slice header and slice header data only applies to the extended slice header prefix NAL unit concept.

## Slice/sub-picture – specific SEI Message Syntax

In order to allow for SEI messages on decoding unit level, the SEI message syntax remains unchanged with this proposal and is given for convenience in the following.

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadType += 255 |  |
| } |  |
| last_payload_type_byte | u(8) |
| payloadType += last_payload_type_byte |  |
| payloadSize = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadSize += 255 |  |
| } |  |
| last_payload_size_byte | u(8) |
| payloadSize += last_payload_size_byte |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

In order to introduce new SEI messages types, the SEI payload syntax has to be extended as given in the following table.

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 140 ) |  |
| roi_info( payloadSize ) |  |
| else if( payloadType = = 180 ) |  |
| subpic_buffering ( payloadSize ) |  |
| else if( payloadType = = 181 ) |  |
| subpic_timing( payloadSize ) |  |
| else if( payloadType = = 182 ) |  |
| subpic_tile_info ( payloadSize ) |  |
| else if( payloadType = = 183) |  |
| subpic_tile_dimensions_info( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ |  |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ |  |
| } |  |
| } |  |

The SEI messages with payloadType in the range of 180 to 183 have to be sent exclusively on sub-picture or slice level within the slice prefix NAL unit. Additionally the region of interest SEI messages with payloadType equal to 140 can to be sent either in the slice prefix NAL unit on sub-picture level or as a regular SEI message on access unit level.

The following subsections define the new SEI messages to be used on slice or sub-picture level. Additionally, we propose a region of interest SEI message that can be used on sub-picture and access unit level.

### Sub-picture buffering SEI message

The sub-picture buffering SEI message provides information on the removal times of coded data from the CPB on a decoding unit (sub-picture) level. The proposal includes removing sub-picture level signalling of CPB removal times from the buffering period SEI message (payload type 0).

| subpic_buffering( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |

seq_parameter_set_id specifies the sequence parameter set that contains the sequence HRD attributes. The value of seq_parameter_set_id shall be equal to the value of seq_parameter_set_id in the picture parameter set referenced by the primary coded picture associated with the buffering period SEI message. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

initial_cpb_removal_delay[ SchedSelIdx ] specifiesthe initial CPB removal delays for the SchedSelIdx-th CPB of the decoding unit (the sub-picture). The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock. The values of the syntax elements shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ]  BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cpb_removal_delay_offset[ SchedSelIdx ] and are used for the SchedSelIdx-th CPB to specify the initial delivery time of coded data units to the CPB of the decoding unit (the sub-picture). The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1 and are in units of a 90 kHz clock. These syntax elements are not used by decoders and may be needed only for the delivery scheduler (HSS) specified in Annex Error: Reference source not found.

Over the entire coded video sequence, the sum of initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] per decoding unit (sub-picture) shall be constant for each value of SchedSelIdx, and the sum of initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.

### Sub-picture timing SEI message

The sub-picture timing SEI message provides information on CPB removal delay of coded data within the decoding unit (sub-picture) it is associated with. The proposal includes removing decoding unit specific signalling from the picture timing SEI message (payload type 1).

| subpic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_8bits | u(8) |
| du_cpb_removal_delay | u(v) |
| } |  |

du_cpb_removal_delay specifies how many clock ticks to wait after removal from the CPB of the decoding unit (sub-picture) associated with the most recent sub-picture buffering period SEI message in a preceding access unit in the same decoding unit (sub-picture), if present, otherwise associated with the most recent buffering period SEI message in a preceding access unit, before removing from the buffer the decoding unit (sub-picture) data associated with the sub-picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit (sub-picture) data into the CPB for the HSS (Hypothetical Stream Scheduler [1]). The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1. The cpb_removal_delay is the remainder of a modulo 2(cpb_removal_delay_length_minus1 + 1) counter.

### Sub-picture tile info SEI message

The sub-picture tile info SEI provides information on the number of tiles in the decoding unit (sub-picture) associated with the prefix NAL unit and the identifier of the first tile within. Furthermore, the basic priority of all tiles within the decoding unit is signalled.

| subpic_tile_info( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_4bits | u(4) |
| tile_priority | u(3) |
| multiple_tiles_in_prefixed_slices_flag | u(1) |
| if( multiple_tiles_in_prefixed_slices_flag ) |  |
| num_tiles_in_prefixed_slices_minus1 | u(16) |
| first_tile_id_in_prefixed_slices | u(16) |
| } |  |

tile_priority indicates the priority of all tiles in the prefixed slices following in decoding order. The value of tile_priority shall be in the range of 0 to 7 inclusively, where 7 indicates the highest priority.

multiple_tiles_in_prefixed_slices_flag with a value of 1 indicates that there are more than 1 tiles in the prefixed slices to follow in decoding order. multiple_tiles_in_prefixed_slices_flag with a value of 0 indicates that the following prefixed slices contain only one tile.

num_tiles_in_prefixed_slices_minus1 indicates the number of tiles in the prefixed slices following in decoding order.

first_tile_id_in_prefixed_slices indicates the tile_id of the first tile in the prefixed slices following in decoding order.

### Sub-picture tile dimensions info SEI message

The sub-picture tile dimensions info SEI message provides information on the image dimension of all tiles in the decoding unit (sub-picture) associated with the slice prefix NAL unit. Having this information accessible on a high level is beneficial for various application, e.g. when assigning displays to tiles in a video conference application or in a multi monitor environment.

| subpic_tile_dimensions _info( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_7bits | u(7) |
| multiple_tiles_in_prefixed_slices_flag | u(1) |
| if( multiple_tiles_in_prefixed_slices_flag ) |  |
| num_tiles_in_prefixed_slices_minus1 | u(16) |
| for( i = 0; i <= num_tiles_in_roi_minus1 ; i++ ) { |  |
| tile_horz_start[ i ] | u(16) |
| tile_width[ i ] | u(16) |
| tile_vert_start[ i ] | u(16) |
| tile_height[ i ] | u(16) |
| } |  |
| } |  |

multiple_tiles_in_prefixed_slices_flag with a value of 1 indicates that there are more than 1 tiles in the prefixed slices to follow in decoding order. multiple_tiles_in_prefixed_slices_flag with a value of 0 indicates that the following prefixed slices contain only one tile.

num_tiles_in_prefixed_slices_minus1 indicates the number of tiles in the prefixed slices following in decoding order.

tile_horz_start[ i ] indicates the start in horizontal direction of the i-th tile in pixels within the picture.

tile_width[ i ] indicates the width of the i-th tile in pixels within the picture.

tile_vert_start[ i ] indicates the start in horizontal direction of the i-th tile in pixels within the picture.

tile_height[ i ] indicates the height of the i-th tile in pixels within the picture.

Note: the tile dimension SEI message may be used to in display operations, e.g., for assigning a tile to a screen in multiple screen display scenario.

### Region of interest SEI message

The region of interest SEI message can be either used on access unit level or on sub-picture level to signal one or more ROIs on a tile basis along with their priority. signalling Such high level signalling of ROIs is of interest for vaious application, e.g. in a video conference application with multiple speakers. It allows for a MANE to assign ROIs and their priority with NAL units. In combination with sub-picture SEI messages (e.g. the tile info SEI message) the granularity of this mapping can be increased.

| roi_info( payloadSize ) { | Descriptor |
| --- | --- |
| reserved_one_8bits | u(8) |
| roi_id | u(5) |
| roi_priority | u(3) |
| num_tiles_in_roi_minus1 | u(16) |
| for( i = 0; i <= num_tiles_in_roi_minus1 ; i++ ) { |  |
| roi_tile_id[ i ] | u(16) |
| } |  |
| } |  |

roi_id indicates the identifier of the region of interest.

roi_priority indicates the priority of all tiles that belongs to the region of interest in the prefixed slices or all slices following in decoding order depending on whether the SEI message is sent on sub-picture level or access unit level. The value of roi_priority shall be in the range of 0 to 7 inclusively, where 7 indicates the highest priority. If both, roi_priority in the roi info SEI message and tile_priority in the sub-picture tile info SEI messages are given, the highest value of both is valid for the priority of the individual tiles.

num_tiles_in_roi_minus1 indicates the number of tiles in the prefixed slices following in decoding order that belong to the region of interest.

roi_tile_id[ i ] indicates the tile_id of the i-th tile that belongs to the region of interest in the prefixed slices following in decoding order.

# Conclusion and further considerations

The slice prefix NAL unit along with the sub-picture SEI messages provides an extended signalling method for sub-picture/slice specific parameters. Various general applications can benefit from the proposed generic concept, e.g.:

- Low delay application

- Application layers, e.g., of conversational services

- Multi monitor environments

Furthermore, the slice prefix NAL unit syntax can be extended with syntax elements that address the slice level, as exemplified with tile_id in the extended slice prefix NAL unit syntax in the given proposal. Such signalling may also contain the slice header for the following dependent slices/prefixed slices in a similar fashion as introduced in [2].

# References

- JCT-VC, "High-Efficiency Video Coding (HEVC) text specification Working Draft 7", JCTVC-I1003, May 2012.

- M. Hannuksela, A. Hallapuro, ”Nested hierarchy of tiles and slices through slice header prediction“, JCTVC-I0070, May 2012

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).