| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0256 |
| --- | --- |

| Title: | Removal of the 8×2 and 2×8 coefficient groups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Coefficient coding in HEVC is processed in 4×4 sub-blocks (coefficient groups). The exceptions are the 8×2 and 2×8 sub-blocks for the 8×8 TU when horizontal and vertical scans are used. It is proposed to remove these non-square coefficient groups by using 4×4 based horizontal and vertical scans. The significance map coding is unified for all the scans and for TU sizes larger than 4×4. The performance of the method is 0.06% / −0.03% / −0.02% for AI / RA / LD Main, and 0.09% / 0.00% / 0.02% for AI  / RA / LD HE. For the RDOQ off case, results are 0.00% / −0.07% / 0.00% for AI / RA / LD Main and 0.01% / −0.05% / 0.06% for AI / RA / LD HE. The method is combined with the significance map patterns in JCTVC-J0354 giving a performance of 0.02% / −0.06% / −0.02% for AI / RA / LD Main, and 0.06% / −0.04% / 0.07% for AI  / RA / LD HE.

# Introduction

In HEVC, quantized coefficients are coded based on the unit of coefficient group (CG), which consists of 16 consecutive coefficients in the scan order. Five syntax elements related to coefficient significance, level and sign are sequentially coded in each CG in five scan passes.

A CG consists of a 4×4 sub-block, except for the case of horizontal/vertical scan in an 8×8 TU when the CG size is 8×2 / 2×8. This special case introduces irregularities in the coefficient coding design. The data access is processed in two different rectangular shapes. Also, context derivation for the syntax significant_coeff_group_flag is different for these CG shapes.

Therefore, the removal of the 8×2 and  2×8 coefficient groups simplifies the HEVC design.

# Proposed Method

The block-based horizontal/vertical scans [1] in figure 1 are used in this proposal.

Figure 1. 4x4 horizontal (left) and vertical (right) scans. CG are denoted by different colors.

The remaining technical dedx dtails of the proposal are the following:

- All coefficient coding processing is done in 4×4 CG

- Large TUs and 8×8 TU use the same context derivation for the significance map

  - The patterns to assign the contexts are the ones shown in figure 2 (same as in [2]).

  - All the scans use the same context derivation

  - 8×8 TU doesn’t share contexts with other TU sizes

    - Diagonal scan and horizontal/vertical scan don’t share contexts

    - Horizontal / vertical scans share contexts

    - Number of contexts is reduced by 3

    - Initial probabilities are not trained

- Context derivation for coefficient group flag is the same for all TU sizes and all scans

- 8×8 significance map table is removed

Figure 2: Context assignment depending on the CG flag below (CGB) and the CG flag on the right (CGR)

## Combination with JCTVC-J0354

The method is applied with the context assigment patterns in [4]:

- The patterns are the ones in figure 3:

  - The horizontal pattern is equal to the row number

  - The vertical pattern is equal to the column number

- Number of contexts is increased by 5

- Initial probabilities are not trained

Figure 3: Context assignment patterns in  [4]

# Simulation results

The proposed method is implemented in HM7.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM7.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate saving. Table 1 summarize the experimental results in common conditions. Table 2 show the results for RDOQ off case. Table 3 shows the results for low QPs (12, 17, 22, 27). Table 4 shows the results using the patterns in [4].

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06% | -0.03% | -0.09% | -0.06% | -0.13% | -0.10% |
| Class B | 0.05% | -0.07% | -0.13% | 0.09% | -0.07% | -0.13% |
| Class C | 0.10% | -0.01% | 0.01% | 0.13% | 0.05% | 0.01% |
| Class D | 0.05% | 0.03% | 0.01% | 0.07% | 0.06% | 0.01% |
| Class E | 0.22% | 0.01% | 0.00% | 0.27% | 0.02% | 0.00% |
| Overall | 0.06% | -0.02% | -0.05% | 0.09% | -0.02% | -0.05% |
|  | 0.06% | -0.02% | -0.04% | 0.09% | -0.02% | -0.04% |
| Class F | 0.02% | 0.01% | 0.10% | 0.00% | -0.22% | -0.09% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.49% | -0.38% | -0.04% | 0.04% | 0.02% |
| Class B | -0.07% | 0.00% | -0.21% | 0.01% | 0.11% | -0.06% |
| Class C | -0.04% | 0.15% | -0.09% | 0.01% | 0.09% | -0.03% |
| Class D | 0.00% | 0.24% | 0.06% | 0.00% | -0.19% | -0.21% |
| Class E |  |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.16% | 0.00% | 0.02% | -0.07% |
|  | -0.10% | -0.02% | -0.19% | -0.01% | -0.01% | -0.09% |
| Class F | 0.02% | 0.06% | -0.15% | 0.02% | 0.31% | -0.31% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.21% | -0.12% | 0.02% | 0.42% | 0.27% |
| Class C | -0.04% | 0.09% | 0.30% | 0.02% | 0.27% | 0.24% |
| Class D | 0.03% | 0.48% | -0.03% | -0.04% | 0.32% | -0.63% |
| Class E | -0.08% | -0.83% | 0.05% | 0.12% | 0.03% | 0.81% |
| Overall | -0.04% | 0.05% | 0.04% | 0.02% | 0.29% | 0.14% |
|  | -0.04% | 0.06% | -0.03% | 0.02% | 0.29% | 0.19% |
| Class F | -0.11% | -0.01% | 0.02% | 0.12% | 0.64% | 0.94% |
| Enc Time[%] | 96% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 103% |  |  |

Table 1. Results for the proposed method

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | -0.18% | -0.22% | -0.13% | -0.13% | -0.18% |
| Class B | 0.01% | -0.22% | -0.22% | 0.02% | -0.22% | -0.23% |
| Class C | 0.05% | -0.08% | -0.07% | 0.05% | -0.10% | -0.08% |
| Class D | 0.00% | -0.10% | -0.05% | 0.01% | -0.12% | -0.12% |
| Class E | 0.15% | -0.04% | -0.05% | 0.14% | -0.05% | -0.09% |
| Overall | 0.00% | -0.13% | -0.13% | 0.01% | -0.13% | -0.15% |
|  | 0.00% | -0.13% | -0.13% | 0.01% | -0.13% | -0.14% |
| Class F | 0.03% | -0.02% | -0.01% | 0.04% | -0.08% | 0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.17% | -0.08% | 0.11% | -0.17% | 0.03% | 0.24% |
| Class B | -0.02% | -0.04% | -0.14% | 0.01% | -0.04% | 0.06% |
| Class C | -0.05% | -0.08% | 0.07% | -0.01% | 0.05% | 0.04% |
| Class D | -0.03% | 0.20% | -0.01% | -0.03% | -0.01% | 0.17% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | 0.00% | 0.00% | -0.05% | 0.00% | 0.12% |
|  | -0.05% | 0.00% | -0.01% | -0.03% | -0.07% | 0.11% |
| Class F | 0.00% | -0.20% | -0.07% | 0.05% | 0.10% | 0.31% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.09% | 0.06% | 0.02% | 0.29% | -0.07% |
| Class C | 0.02% | 0.19% | 0.07% | -0.01% | 0.21% | 0.13% |
| Class D | 0.01% | 0.20% | 0.51% | 0.10% | 1.06% | 0.34% |
| Class E | 0.00% | -0.89% | 1.13% | 0.15% | 0.61% | -0.46% |
| Overall | 0.00% | -0.04% | 0.38% | 0.06% | 0.52% | 0.01% |
|  | 0.00% | -0.09% | 0.30% | 0.06% | 0.46% | -0.04% |
| Class F | -0.24% | 0.28% | -1.09% | 0.03% | 0.99% | -0.04% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. Results for the proposed method with RDOQ off

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.23% | 0.05% | 0.04% | -0.22% | 0.02% | 0.05% |
| Class B | 0.11% | 0.17% | 0.06% | 0.09% | 0.08% | -0.03% |
| Class C | 0.06% | 0.04% | 0.03% | 0.06% | 0.06% | 0.04% |
| Class D | -0.01% | -0.01% | -0.01% | -0.02% | 0.04% | 0.01% |
| Class E | 0.06% | 0.06% | 0.02% | 0.04% | 0.09% | 0.05% |
| Overall | 0.00% | 0.07% | 0.03% | -0.01% | 0.06% | 0.02% |
|  | 0.01% | 0.09% | 0.04% | 0.00% | 0.09% | 0.04% |
| Class F | 0.02% | -0.01% | 0.01% | 0.03% | -0.04% | 0.03% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.18% | 0.01% | 0.05% | -0.16% | 0.10% | 0.10% |
| Class B | 0.05% | 0.13% | 0.05% | 0.04% | 0.09% | -0.01% |
| Class C | -0.02% | 0.07% | -0.06% | -0.03% | 0.12% | 0.03% |
| Class D | -0.06% | 0.05% | -0.13% | -0.05% | -0.06% | -0.08% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | 0.07% | -0.02% | -0.04% | 0.06% | 0.01% |
|  | -0.04% | 0.04% | -0.05% | -0.05% | 0.06% | 0.00% |
| Class F | -0.03% | -0.04% | -0.17% | -0.03% | 0.14% | -0.08% |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | 0.21% | 0.10% | 0.03% | 0.28% | 0.16% |
| Class C | -0.05% | 0.18% | 0.16% | -0.03% | 0.19% | 0.14% |
| Class D | -0.06% | 0.06% | -0.14% | -0.04% | 0.18% | -0.06% |
| Class E | -0.10% | -0.52% | 0.17% | 0.08% | 0.32% | 0.77% |
| Overall | -0.03% | 0.03% | 0.07% | 0.01% | 0.24% | 0.22% |
|  | -0.03% | 0.03% | 0.11% | 0.00% | 0.24% | 0.23% |
| Class F | -0.04% | 0.04% | 0.01% | -0.09% | -0.10% | 0.41% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |

Table 3. Results for low QPs (12, 17, 22, 27)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.12% | -0.02% | -0.07% | -0.11% | -0.06% | -0.18% |
| Class B | 0.00% | 0.00% | -0.06% | 0.05% | 0.02% | -0.07% |
| Class C | 0.08% | 0.04% | 0.03% | 0.10% | 0.07% | 0.05% |
| Class D | 0.02% | 0.05% | 0.01% | 0.06% | 0.09% | 0.05% |
| Class E | 0.17% | 0.02% | -0.02% | 0.23% | 0.11% | 0.07% |
| Overall | 0.02% | 0.02% | -0.03% | 0.06% | 0.04% | -0.02% |
|  | 0.02% | 0.01% | -0.03% | 0.06% | 0.04% | -0.03% |
| Class F | 0.03% | 0.08% | 0.12% | 0.04% | -0.25% | -0.01% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | -0.53% | -0.23% | -0.06% | 0.40% | 0.41% |
| Class B | -0.10% | 0.15% | -0.05% | -0.04% | 0.17% | 0.04% |
| Class C | -0.05% | 0.25% | 0.15% | -0.01% | 0.16% | 0.05% |
| Class D | -0.02% | 0.27% | -0.01% | -0.06% | 0.04% | -0.38% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | 0.04% | -0.04% | -0.04% | 0.19% | 0.03% |
|  | -0.14% | 0.03% | -0.06% | -0.04% | 0.16% | 0.03% |
| Class F | 0.00% | 0.25% | 0.00% | 0.03% | 0.30% | 0.00% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.06% | 0.63% | 0.06% | 0.01% | 0.49% | 0.20% |
| Class C | 0.03% | 0.26% | 0.33% | 0.05% | 0.18% | 0.35% |
| Class D | 0.04% | 0.19% | 0.07% | 0.00% | 0.58% | 0.18% |
| Class E | -0.07% | 0.30% | -0.43% | 0.28% | -0.23% | 0.60% |
| Overall | -0.02% | 0.37% | 0.04% | 0.07% | 0.30% | 0.31% |
|  | -0.02% | 0.33% | 0.03% | 0.07% | 0.35% | 0.42% |
| Class F | -0.14% | -0.20% | 0.43% | 0.11% | 0.71% | 1.18% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |

Table 4. Results for the proposed method with patterns in [4]

# Conclusions

A regular coefficient coding design is obtained by removing the non-square coefficient groups. The proposed method achieves the goal without meaningful performance impact.

# References

[1] C. Rosewarne, M. Maeda, “Non-CE11: Harmonisation of 8x8 TU residual scan”, JCTVC-H0145, 8th JCT-VC Meeting, San Jose, USA, February, 2012.

[2] T. Tsukuba, T. Yamamoto, T. Ikai, “Refined significant map context derivation for large TU”, JCTVC-J0068, 10th JCT-VC Meeting, Stockholm, SE, July, 2012.

[3] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, 9th JCT-VC Meeting, Geneva, CH, April/May, 2012.

[4] R. Joshi, J. Sole, M. Karczewicz, “Modification of context assignment for significance flag coding for large TUs”, JCTVC-J0354, 10th JCT-VC Meeting, Stockholm, SE, July, 2012.

# Working Draft text

7.3.10 Residual Coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| … |  |

7.4.10 Residual coding semantics

significant_coeff_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero transform coefficient level. A coefficient group at location ( xCG, yCG ) is an array of 16 transform coefficient levels at locations ( xC, yC ).

The coefficient group location ( xCG', yCG' ) and the location ( xCGLastSig, yCGLastSig ) specifying the coefficient group location containing the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ) are derived as follows.

- If scanIdx is equal to 1 and log2TrafoWidth is equal to 3 and log2TrafoHeight is equal to 3,

- Coefficient groups are 8x2

- ( xCG', yCG' ) is equal to ( 0, yCG << 1 )

- ( xCGLastSig, yCGLastSig ) is equal to ( 0, LastSignificantCoeffY >> 1 )

- Otherwise, if scanIdx is equal to 2 and log2TrafoWidth is equal to 3 and log2TrafoHeight is equal to 3,

- Coefficient groups are 2x8

- ( xCG', yCG' ) is equal to ( xCG << 1, 0 )

- ( xCGLastSig, yCGLastSig ) is equal to ( LastSignificantCoeffX >> 1, 0 )

- Otherwise,

- Coefficient groups are 4x4

- ( xCG', yCG' ) is equal to ( xCG << 2, yCG << 2 )

- ( xCGLastSig, yCGLastSig ) is equal to ( LastSignificantCoeffX >> 2  LastSignificantCoeffY >> 2 )

significant_coeff_group_flag[ xCG ][ yCG ] implies the following.

- If significant_coeff_group_flag[ xCG ][ yCG ] is equal to 0, the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) are inferred to be equal to 0;

- Otherwise (significant_coeff_group_flag[ xCG ][ yCG ] is equal to 1), the following applies.

- If significant_coeff_group_flag[ xCG ][ yCG ] is present or ( xCG', yCG' ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ), at least one of the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) has a non zero value.

- Otherwise, at least one of the 16 significant_coeff_flag syntax elements is present for the coefficient group at location ( xCG, yCG )

When significant_coeff_group_flag[ xCG ][ yCG ] is not present, it is inferred as follows.

- If one or more of the following conditions are true, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 1.

- ( xCG, yCG ) is equal to ( xCGLastSig, yCGLastSig )

- ( xCG, yCG ) is equal to ( 0, 0 )

Otherwise, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 0.

9.3.3.1.4  Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the scanning order scanIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_group_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-1 as follows..

sigCtx  =  ctxIdxMap[ (yC << 2) + xC ]	(9-1)

- Otherwise, if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-1 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 0 : ( 9 + ctxIdxMap[ ((yC >> 1 ) << 2) + (xC >> 1) ] )	(9-2)

- Otherwise, if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  0		(9-3)

- Otherwise, sigCtx is derived using previously decoded bins of the syntax element significant_coeff_group_flag as follows.

- The horizontal and vertical coefficient group positions xCG and yCG are set equal to (xC >> 2) and (yC >> 2), respectively.

- The variable prevSig is set equal to 0.

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

prevSig  +=  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-4)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

prevSig  +=  (significant_coeff_group_flag[ xCG ][ yCG + 1 ] << 1)	(9-5)

- The variable sigCtx is derived as follows.

  - If prevSig is equal to 0, the the following applies.

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) == 0 ? 2 :  ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <=2 ? 1: 0			(9-6)

sigCtx  =  ( xC − (xCG << 2) ) + ( yC − (yCG << 2) ) <= 2 ? 1 : 0	(9 26)

  - Otherwise, if prevSig is equal to 1, the following applies.

sigCtx  =   ( yC - (yCG << 2) ) == 0 ? 2 : (( yC - (yCG << 2) ) == 1 ? 1 : 0	(9-7)

sigCtx  =  ( yC − (yCG << 2) ) <= 1 ? 1 : 0	(9 27)

  - Otherwise, if prevSig is equal to 2, the following applies.

sigCtx  =   ( xC - (xCG << 2) ) == 0 ? 2 : ( xC - (xCG << 2) ) == 1 ? 1 : 0	(9-8)

sigCtx  =  ( xC − (xCG << 2) ) <= 1 ? 1 : 0	(9 28)

  - Otherwise ( prevSig is equal to 3 ), the following applies.

sigCtx  =   2    		(9-29)

sigCtx  =  ( xC − (xCG << 2) ) + ( yC − (yCG << 2) ) <= 4 ? 2 : 1	(9 29)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx += 3 21		(9-9)

  - If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, the following applies.

sigCtx = sigCtx + ( scanIdx==0 ? 9 : 15 )		(9-10)

  - Otherwise, the following applies.

sigCtx += ( cIdx == 0 ? 21 : 12)  18	(9-11)

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-12)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27 24  +  sigCtx		(9-13)

Table 9-1 – Specifcation of ctxIdxMap[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |

6.5.5 Horizontal scanning array initialisation process

x_off  =  0
y_off  =  0
i_off  =  0
stopLoopSubblocks = FALSE
while( !stopLoopSubblocks ) { 
	i  =  0
	x  =  0
	y  =  0
	stopLoop  =  FALSE
	while( !stopLoop ) {
		while( x  <  4 ) {
			horScan [ i + i_off ][ 0 ]  =  x + x_off
			horScan [ i + i_off ][ 1 ]  =  y + y_off
		      i++
		      x++
		}
		y++
		x  =  0
		if( i  >=  16 ) 
			stopLoop  =  TRUE
	}
	i_off  =  i_off + 16
	if( i_off  >=  blkWidth * blkHeight )
		stopLoopSubblocks  =  TRUE
	else
		x_off  =  x_off + 4
		if( x_off  >  blkWidth  ) {
			y_off  =  y_off + 4
 			x_off  =  0
		}
}

6.5.6 Vertical scanning array initialisation process

x_off  =  0
y_off  =  0
i_off  =  0
stopLoopSubblocks = FALSE
while( !stopLoopSubblocks ) { 
	i  =  0
	x  =  0
	y  =  0
	stopLoop  =  FALSE
	while( !stopLoop ) {
		while( y  <  4 ) {
			horScan [ i + i_off ][ 0 ]  =  x + x_off
			horScan [ i + i_off ][ 1 ]  =  y + y_off
		      i++
		      x++
		}
		x++
		y  =  0
		if( i  >=  16 ) 
			stopLoop  =  TRUE
	}
	i_off  =  i_off + 16
	if( i_off  >=  blkWidth * blkHeight )
		stopLoopSubblocks  =  TRUE
	else
		y_off  =  y_off + 4
		if( y_off  >  blkHeight ) {
			x_off  =  x_off + 4
 			y_off  =  0
		}
}

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).