| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 July – 20 July 2012 | Document: JCTVC-J0257 |
| --- | --- |

| Title: | AHG9/AHG10: Design of the Video Parameter Set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes a design for the Video Parameter Set (VPS) that facilitates dealing with a video sequence on the application layer. We argue for the signalization of profile and level requirements in the VPS and for low level requirements, which are limited by level constraints, to be signaled as either u(n) or u(v). Furthermore, we explain the benefit of signaling coding dependencies and scalability information in the VPS and the advantages of using the parameter set extension mechanism for non-base layer signalization. Additionally, we propose to use an SEI message for indication of layer removal.

# Introduction

This proposal takes on the concept of a Video Parameter Set (VPS) as defined in the current draft specification [1]. We propose a VPS design that provides all information of interest to deal with a video bitstream on the application layer. The proposed VPS design combines the information that is distributed across the layer specific sequence parameters sets (SPSs) in H.264/AVC and its extensions. It is located above the SPS in the parameter set hierarchy and activated by the SPS.

Amongst other things, the initial document proposed removal of the syntax elements vps_max_latency_increase[ i ], vps_num_reorder_pics[ i ] and vps_max_dec_pic_buffering[ i ] from the VPS.

Revision 1 of this document excludes the removal of vps_max_latency_increase[ i ], vps_num_reorder_pics[ i ] and vps_max_dec_pic_buffering[ i ] from the VPS syntax but proposes to code them as u(n) or u(v) to keep the VPS parsable without entropy decoding.

Revision 2 of this document adds explanatory text, clarifications and figures on the structuring of layer_id_plus1 and the signaling of the maximum temporal layer per layer_id_plus1. Furthermore, it is proposed to remove redundant signaling of sps_max_latency_increase[ i ], sps_num_reorder_pics[ i ] and sps_max_dec_pic_buffering[ i ] in the SPS syntax and editorial corrections of VPS and VPS extension syntax are made.

Revision 3 of this document includes an additional proposal to reflect the latest adoptions of JCV-VC and the discussions of the BoG on  on high-level syntax for extension planning. The additions can be found in section 3.

Revision 4 of this document proposes a syntax that combines the two approaches (mapping and partitioning) discussed in the BoG on high-level syntax for extension planning. The additions can be found in section 4.

The key points of the proposal are detailed in the following subsections.

## Profile/level/low level requirements

Currently, the definition of the non-mandatory VPS in the HEVC draft base specification defines the VPS to hold redundant syntax related to low level requirements for temporal scalability, i.e. ue-coded low level requirements limited by the level constraints such as sps/vps_max_latency_increase[ i ], sps/vps_num_reorder_pics[ i ] and sps/vps_max_dec_pic_buffering[ i ].  In H.264/AVC and earlier HEVC WDs, these low level requirements were buried deep inside the SPS syntax reflecting the fact that they are of little interest for the application layer and that profile and level requirements are more useful and sufficiently fine-grained for application means.

The implicit hierarchy of requirements signaling (Profile/Level/low level) should be reflected in their occurrence within the bitstream, complying with the hierarchy of parameter sets (VPS/SPS/PPS). Therefore, we propose to mandate profile and level signaling in the VPS and code the low level requirements vps_max_latency_increase[ i ], vps_num_reorder_pics[ i ] and vps_max_dec_pic_buffering[ i ] as either u(n) or u(v). It is proposed to remove the signaling of sps_max_latency_increase[ i ], sps_num_reorder_pics[ i ] and sps_max_dec_pic_buffering[ i ] from the SPS syntax.

Furthermore, we propose to include a fine-grained level signalization per temporal layer to allow a decoder correct assessment of its capabilities of dealing with a given coded video sequence, e.g. in case the decoder is capable to decode only a temporal subset of the coded video sequence by excluding specific temporal layers. The level signaling in the current working draft does not allow for such a behavior.

Besides the benefit of having requirement information compiled in one place, the proposal keeps the VPS non-entropy-coded and parsable for simple Media Aware Network Elements (MANEs) as well and reduces redundancy that does not serve any purpose, e.g. error robustness.

## Non-base layer signaling

When introducing extensions to HEVC, the VPS concept has to be extended to provide signalization for the non-base layers. This signaling includes several parameters, e.g. maximum of temporal layers as can be expected through the experience with frame rate increase in enhancement layers of SVC and would facilitate the above mentioned fine-graining level signaling. Further parameters to be signaled per non-base layer are profile and level, coding dependencies between layers or others. This can be done in two ways: extending the VPS syntax with a loop over layer identifiers or using the dedicated extension mechanism already provided in the syntax of all parameter set. We propose to go the latter way for the following reasons:

- The extension mechanism allows providing information tailored to the needs of non-base layers, e.g. coding dependencies signaling is required only for non-base layers.

- Redundant profile and level signalization (for each layer: 56 bits per profile and (max_temporal_layers_minus1+1) * 8 bits per level) can be avoided by using referencing (6 bits for flag and reference layer_id_plus1).

- The needs of a specific HEVC extension could potentially be addressed by an individual VPS extension syntax.

## Coding dependencies between layers

Signaling the inherent dependencies between layers is of high interest in a scalable setup. For SVC, this is implicitly done through the order of dependency and quality identifiers whereas for MVC, this information was provided by means of the SPS MVC extension syntax. With the concept of layer_id_plus1 as a generic layer identifier for scalability in HEVC, we propose to signal coding dependencies within the VPS for each layer by listing the layer identifiers of layers that are used for reference. Again, this has the benefit of having the information compiled in one place and accessible for the application layer, yet being flexible enough for the demands of future HEVC extensions.

## Scalability information

The simple mapping of layer_id_plus1 to a certain profile and level set allows for identification of scalability type. Furthermore, if only a single scalable identifier (e.g. spatial, quality or view exclusively) is in use or of interest, it is also feasible to order layers by consecutive layer_id_plus1 values, i.e. a high value of layer_id_plus1 wrt. any other value can be associated with a specific step of resolution or quality depending on the extension in use without further look up. Such functionality is of benefit for the transport or application layer. The following figure illustrates the mapping of layer_id_plus1 to a scalable identifier.

Figure 1: Mapping of layer_id_plus1 to scalable identifier in one-dimensional setup.

However, as soon as combinations of the scalable identifiers (e.g. two such as spatial and quality, or three such as spatial, view and depth) are in use or of interest, this no longer holds true. In this case, e.g. a MANE is not able to position an operation point (i.e. a specific layer_id_plus1 value) within the scalability space (e.g. constituted by a scalable dimension) as there are multiple mappings of layer_id_plus1 to the scalable identifiers. This fact is illustrated in the following figure for the case of two-dimensional scalability (e.g. quality and spatial or view and depth).

Figure 2: Mapping of layer_id_plus1 to scalable identifiers in a two-dimensional setup.

To allow for such functionality on the transport or application layer, means for interpreting layer_id_plus1 are necessary. One solution is signalization of a direct mapping table providing values for all scalable identifiers for each value of layer_id_plus1 with the disadvantage of requiring a lookup for the interpretation of each value of layer_id_plus1 and uncomfortable means for addressing new scalability dimensions. Therefore, in this document we propose another solution that provides means for structuring the layer_id_plus1 syntax element for straight forwards interpretation without look ups and in an extensible manner. The proposal includes signalization of the scalability dimensions (cp. scalability_type) in use and the association of a bit range within layer_id_plus1 to a certain dimension, e.g. 1 bit as depth flag, 4 bits for view (cp. layer_id_plus1_dim_len). A change of scalability identifier (e.g. spatial or quality) is correspondingly indicated in the associated bit range of layer_id_plus1.

This allows for positioning of operation points in the scalability space and compiles all information of potential interest for the transport or application layer at the top of the parameter set hierarchy readily accessible. Again, this is designed in a generic way to be flexible and extensible for future HEVC extensions to come.

## Layer removal signalization

When a MANE performs adaptation of the video bitstream, it is of interest for subsequent devices (decoders, further MANEs, etc.) to be aware of the performed operations. As updating the VPS to reflect the current content of the coded video sequence is not an option due to the IDR requirement for its activation, we propose an SEI message to signal the removal of complete layers or temporal levels.

# Proposed Syntax

Table 1: Proposed NAL unit syntax.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| layer_id_plus1 | u(5) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

layer_id_plus1 minus 1 indicates the layer identification. A layer_id_plus1 equal to 1 indicates that the layer identification is equal to 0, and the NAL unit belongs to the base layer or base view. Higher values of layer_id_plus1 indicate that the corresponding NAL unit belongs to an enhancement layer or view associated with the specified layer_id_plus1. The association of scalable layers with their associated scalable identifiers to specific values of layer_id_plus1 follows the values of scalability_type and layer_id_plus1_dim_len as signaled in the active Video Parameter Set.

Table 2: Proposed SPS Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3: Proposed VPS syntax.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(8) |
| scalability_type | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_max_temporal_layers_minus1 | u(3) |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| for( i = 0; i < max_temporal_layers_minus1; i++ ) { |  |
| level_idc[ i ] | u(8) |
| vps_max_dec_pic_buffering[ i ] | ue(v)u(5) |
| vps_num_reorder_pics[ i ] | ue(v)u(5) |
| max_latency_increase_length[ i ] | u(5) |
| vps_max_latency_increase[ i ] | ue(v)u(v) |
| } |  |
| for( i = 0; i < MaxDim(scalability_type ); i++ ) { |  |
| layer_id_plus1_dim_len[ i ] | u(3) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data() | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

scalability_type indicates the scalability types in use in the coded video sequence and the dimensions signaled through layer_id_plus1 in the NAL unit header. When scalability_type is equal to 0, the coded video sequence conforms to the base HEVC specification, thus layer_id_plus1 of all NAL units is equal to 1 and there are no NAL units belonging to an enhancement layer or view. Higher values of scalability_type have to be interpreted according to Table 4.

profile_space, profile_idc, constraint_flags, profile_and compatability_flag signal information as specified in the current WD and the indicated values are valid for all NAL units with a layer_id_plus1 equal to 1.

level_idc[ i ] signals information as specified in the current WD and the indicated values are valid for all NAL units with a layer_id_plus1 equal to 1 and a temporal_id equal to i.

max_latency_increase_length[ i ] indicates the length in bits of the syntax element max_latency_increase_[ i ].

- max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] to vps_num_reorder_pics[ i ] + vps_max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When vps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When vps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of vps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive. The length of the max_latency_increase[ i ] syntax element is max_latency_increase_length[ i ] bits.

layer_id_plus1_dim_len[ i ] indicates the length in bits of layer_id_plus1 that is used to signal the scalable identifier associated with the i-th scalability dimension according to Table 4beginning at the MSB for i = 0 or the end of the (i-1)-th signaled scalability dimension for i > 0.

Table 4: Proposed scalability_type mapping table

| scalability_type | Return value of MaxDim(scalability_type) | Scalability dimensions |
| --- | --- | --- |
| 0 | 1 | none (base HEVC) |
| 1 | 2 | spatial and quality scalability |
| 2 | 2 | multiview and depth scalability |
| 3 | 4 | multiview, spatial, quality and depth scalability |
| 4...15 | reserved | reserved |

The scalability types exemplarily defined in Error: Reference source not found represent a set of scalabilities which can be anticipated at this point in time, but may be extended with new dimensions and/or combinations with the progress of HEVC extensions.

Table 5: Proposed VPS extension data syntax.

| vps_extension_data() { | Descriptor |
| --- | --- |
| vps_extension_type | u(2) |
| layer_id_plus1 | u(5) |
| max_temporal_layers_minus1 | u(3) |
| new_profile_level_flag | u(1) |
| if ( new_profile_level_flag == 0 ) { |  |
| profile_reference_layer_id_plus1 | u(5) |
| } |  |
| else { |  |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| for( i = 0; i < max_temporal_layers_minus1; i ++ ) |  |
| level_idc[ i ] | u(8) |
| } |  |
| num_reference_layers | u(5) |
| for( n = 0; n < num_reference_layers; n ++ ) |  |
| direct_coding_dependency_layer_id_plus1[ n ] | u(5) |
| } |  |

vps_extension_type indicates the type of the VPS extension. When the value of vps_extension_type is equal 0, the syntax given above applies.

layer_id_plus1 indicates the layer_id for which the syntax elements signaled in the VPS extension are valid.

max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in NAL units with a layer_id_plus1 equal to the layer_id_plus1 signaled in the VPS.

new_profile_level_flag equal to 0 indicates that the profile and level signaling associated with a value of layer_id_plus1 equal to the values specified by profile_reference_layer_id_plus1 is valid for the NAL unit with layer_id_plus1 specific within the VPS extension. New profile_level_flag equal to 1 indicates that profile and level for the specified value of layer_id_plus1 is signaled explicitly in the VPS extension.

profile_reference_layer_id_plus1 indicates the value of layer_id_plus1 whose associated profile and level signaling is valid for NAL units with a value of layer_id_plus1 equal to the value  of layer_id_plus1 signaled in the VPS extension.

profile_space, profile_idc, constraint_flags, profile_and compatability_flag signal information as specified in the current WD and the indicated values are valid for all NAL units with a layer_id_plus1 equal to the layer_id_plus1 values specified in the VPS extension data.

level_idc[ i ] signals information as specified in the current WD and the indicated values are valid for all NAL units with a layer_id_plus1 equal to layer_id_plus1 values specified in the VPS extension data and a temporal_id equal to i.

num_reference_layers specifies the number of layers, identified by layer_id, to be used as references in the decoding process.

direct_coding_dependency_layer_id_plus1 indicates the layer_id_plus1 of NAL units to be used as references in the decoding process.

Table 6: Proposed Layer Removal SEI.

| layer_removal( payloadSize) { | Descriptor |
| --- | --- |
| num_removed_layers | u(5) |
| for( i = 0; i < num_removed_layers; i++ ) { |  |
| removed_layer_id_plus1[ i ] | u(5) |
| removed_temporal_level [ i ] | u(3) |
| } |  |
| } |  |

num_removed_layers indicates the number of removed layers in the coded video sequence, e.g. due to adaptation of the coded video sequence.

removed_layer_id_plus1[ i ] indicates the value of layer_id_plus1 that is described in the VPS and VPS extension but whose associated NAL units are (potentially partially) removed from the coded video sequence.

removed_temporal_level[ i ] equal to 0 indicates that the NAL units with layer_id_plus1 equal to removed_layer_id_plus1[ i ] are completely removed from the video sequence. max_temporal_level[ i ]  greater than 0 indicates the maximum of (temporal_id - 1) to be signaled in NAL units with a layer_id_plus1 equal to removed_layer_id_plus1[ i ].

# Additional proposal per revision 3

This section presents an additional proposal for high level syntax for 3D: The following bullets summarize the changes with respect to the former proposal:

- The syntax relevant for scalable/multiview extensions is moved behind the base HEVC VPS extention flag.

- layer_id bits are are not associated with a specific scalable identifier.

- Semantics of the scalable identifiers signalled for each value of layer_id are determind via a mapping table and the scalability_type syntax element.

- Start code emulation prevention is achieved through temporal_id_plus1 instead of layer_id_plus1.

- The length of the layer_id syntax element is changed to 6 bits.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| scalability_type | u(4) |
| for( i = 0; i <= max_num_layers_minus1; i++ ) { |  |
| vps_extension_data() |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |
| } |  |

scalability_type indicates the scalability types in use in the coded video sequence and the dimensions signaled through layer_id_plus1 in the NAL unit header. When scalability_type is equal to 0, the coded video sequence conforms to the base HEVC specification, thus layer_id_plus1 of all NAL units is equal to 1 and there are no NAL units belonging to an enhancement layer or view. Higher values of scalability_type have to be interpreted according to Table 7.

Table 7: Proposed scalability_type mapping table

| scalability_type | Return value of MaxDim(scalability_type) | Scalability dimensions |
| --- | --- | --- |
| 0 | 1 | none (base HEVC) |
| 1 | 2 | spatial and quality scalability |
| 2 | 2 | multiview and depth scalability |
| 3 | 4 | multiview, spatial, quality and depth scalability |
| 4...15 | reserved | reserved |

| vps_extension_data() { | Descriptor |
| --- | --- |
| vps_extension_type | u(2) |
| layer_id | u(6) |
| layer_max_sub_layers_minus1 | u(3) |
| new_profile_level_flag | u(1) |
| if ( new_profile_level_flag == 0 ) { |  |
| profile_reference_layer_id | u(6) |
| } |  |
| else { |  |
| profile_level( 1, layer_max_sub_layers_minus1 ) |  |
| } |  |
| for( i = 0; i < MaxDim(scalability_type ); i++ ) { |  |
| scalable_identifier [ i ] | u(6) |
| } |  |
| num_reference_layers | u(6) |
| for( n = 0; n < num_reference_layers; n ++ ) |  |
| direct_coding_dependency_layer_id_plus1[ n ] | u(6) |
| } |  |

vps_extension_type indicates the type of the VPS extension. When the value of vps_extension_type is equal 0, the syntax given above applies.

layer_id indicates the layer_id for which the syntax elements signaled in the VPS extension are valid.

layer_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in NAL units with a layer_id_plus1 equal to the layer_id_plus1 signaled in the VPS.

new_profile_level_flag equal to 0 indicates that the profile and level signaling associated with a value of layer_id_plus1 equal to the values specified by profile_reference_layer_id_plus1 is valid for the NAL unit with layer_id_plus1 specific within the VPS extension. New profile_level_flag equal to 1 indicates that profile and level for the specified value of layer_id_plus1 is signaled explicitly in the VPS extension.

profile_reference_layer_id indicates the value of layer_id_plus1 whose associated profile and level signaling is valid for NAL units with a value of layer_id_plus1 equal to the value  of layer_id_plus1 signaled in the VPS extension.

scalable_identifier [ i ] indicates the i-th scalability identifier according to mapping of scalability_type via the proposed Table 7.

num_reference_layers specifies the number of layers, identified by layer_id, to be used as references in the decoding process.

direct_coding_dependency_layer_id_plus1 indicates the layer_id_plus1 of NAL units to be used as references in the decoding process.

# Additional proposal for combined syntax per revision 4

This section presents a possible straw man design for enabling either partitioning or mapping of values of the reserved_zero_6bits in the NAL unit header via a flag based VPS syntax.

Table 8: Proposed VPS syntax for combined solution.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| scalability_type | u(4) |
| partitioning_flag | u(1) |
| if( partitioning_flag = = 1) { |  |
| for( i = 0; i < MaxDim(scalability_type ); i++ ) { |  |
| layer_id_dim_len[ i ] | u(3) |
| } |  |
| } |  |
| for( i = 0; i <= max_num_layers_minus1; i++ ) { |  |
| vps_extension_data( partitioning_flag ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |
| } |  |

scalability_type indicates the scalability types in use in the coded video sequence and the dimensions signaled through layer_id in the NAL unit header. When scalability_type is equal to 0, the coded video sequence conforms to the base HEVC specification, thus layer_id of all NAL units is equal to 0 and there are no NAL units belonging to an enhancement layer or view. Higher values of scalability_type have to be interpreted according to Table 9.

partitioning_flag indicates whether individual bit ranges of the syntax element layer_id are associated with a specific scalable identifier determind by scalability_type or scalable identifiers are signalled explicitply in vps_extension_data().

layer_id_dim_len[ i ] indicates the length in bits of layer_id that is used to signal the scalable identifier associated with the i-th scalability dimension according to Table 9 beginning at the MSB for i = 0 or the end of the (i-1)-th signaled scalability dimension for i > 0.

Table 9: Proposed scalability_type mapping table

| scalability_type | Return value of MaxDim(scalability_type) | Scalability dimensions/Scalable identifiers |
| --- | --- | --- |
| 0 | 1 | none (base HEVC) |
| 1 | 2 | spatial and quality |
| 2 | 3 | spatial, quality, unspecified |
| 3 | 4 | spatial, quality, unspecified, unspecified |
| 4 | 2 | multiview and depth |
| 5 | 3 | multiview, depth, unspecified |
| 6 | 4 | multiview, depth, unspecified, unspecified |
| 7 | 4 | multiview, spatial, quality and depth |
| 8 | 5 | multiview, spatial, quality, depth, unspecified |
| 9 | 6 | multiview, spatial, quality, depth, unspecified, unspecified |
| 10...15 | reserved | reserved |

Table 10: Proposed Syntax for VPS extension data in a combined solution.

| vps_extension_data( partitioning_flag ) { | Descriptor |
| --- | --- |
| layer_id | u(6) |
| layer_max_sub_layers_minus1 | u(3) |
| new_profile_level_flag | u(1) |
| if ( new_profile_level_flag == 0 ) { |  |
| profile_reference_layer_id | u(6) |
| } |  |
| else { |  |
| profile_level( 1, layer_max_sub_layers_minus1 ) |  |
| } |  |
| if( partitioning_flag = = 0 ) { |  |
| for( i = 0; i < MaxDim(scalability_type ); i++ ) { |  |
| scalable_identifier [ i ] | u(6) |
| } |  |
| } |  |
| num_reference_layers | u(6) |
| for( n = 0; n < num_reference_layers; n ++ ) |  |
| direct_coding_dependency_layer_id[ n ] | u(6) |
| } |  |

layer_id indicates the layer_id for which the syntax elements signaled in the VPS extension are valid.

layer_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in NAL units with a layer_id equal to the layer_id signaled in the VPS extension data.

new_profile_level_flag equal to 0 indicates that the profile and level signaling associated with a value of layer_id equal to the values specified by profile_reference_layer_id is valid for the NAL unit with layer_id specific within the VPS extension. New profile_level_flag equal to 1 indicates that profile and level for the specified value of layer_id is signaled explicitly in the VPS extension.

profile_reference_layer_id indicates the value of layer_id whose associated profile and level signaling is valid for NAL units with a value of layer_id equal to the value  of layer_id signaled in the VPS extension.

scalable_identifier [ i ] indicates the i-th scalability identifier according to mapping of scalability_type via the proposed Table 9 if partitioning is not used.

num_reference_layers specifies the number of layers, identified by layer_id, to be used as references in the decoding process.

direct_coding_dependency_layer_id_plus1 indicates the layer_id of NAL units to be used as references in the decoding process.

# References

- JCT-VC, "High-Efficiency Video Coding (HEVC) text specification Working Draft 7", JCTVC-I1003, May 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).