| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0258 |
| --- | --- |

| Title: | Interlaced coding performance and Chroma consideration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse ATEME 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com, p.larbier@ateme.com, jm.thiesse@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution proposes an assessment of both objective and subjective performance on interlaced material coding with HM 7.0. Objective performances are reported against H.264/AVC while visual degradations on chroma component are highlighted when encoding at low bitrates.

This degradation results from the misalignment of chroma samples locations of top field with regard to bottom field. An update of previous proposal JCTVC-I0502 is consequently evaluated while considering last SEI modification. Objective improvements associated to chroma artifacts correction are reported.

# Introduction

While current stats of HEVC is very promising, it has been reported in previous contribution [1] some lack in HEVC design for encoding legacy interlaced video content. When considering 4:2:0 interlaced material top and bottom fields chroma samples are misaligned leading to inconsistency between the current derivation process of motion vector information and the nature of interlace format.

In this contribution, a first evaluation of H.264/AVC in comparison with HEVC encoding performance when dealing with interlaced material, is provided. Hence previous proposal [1] is updated regarding last SEI Field indication modification and both objective and subjective performance of proposed solution have been evaluated. As expected, important objective and subjective improvements are obtained.

# A first evaluation on interlaced material

## Interlaced material

Three HD (1920x1080/60i) sequences specified in ITU-R BT.1120 [2] have been used:

#13 "Church HD ",

#16 "Whale show HD "

#22 "Marching in HD ".

When performing HEVC encoding (using HM-7.0 software), one field picture in interlaced sequences is considered as one frame and therefore interlaced sequences are encoded as sequences with half height and doubled frame rate. Sequences are encoded under the common condition considering "Random Access High Efficiency" configurations.

## Objective assessment: H.264/AVC vs HEVC

In order to assess current HEVC efficiency against H.264/AVC on the interlaced sequences described above, a benchmark has been performed on interlaced sequences using HM7.0.

ATEME H.264/AVC encoder has been used for this evaluation with all “encoder only” specific tools disabled and representative GOP structure conditions as defined in following table.  H.264/AVC interlaced tools such as MBAFF and PAFF are activated. HEVC based encoding have been performed with a set of quantization parameters which leads to realistic target bitrates (regarding broadcast applications) from 1Mbits/s to 10Mbits/s. Quantization parameters leading to the same range of bitrates have been used with H.264/AVC.

Table 1 Coding configuration.

|  | H.264/AVC | HEVC |
| --- | --- | --- |
| FrameRate | 30 | 60 |
| SourceHeight | 1080 | 544 |
| FrameToBeEncoded | 450 | 900 |
| IntraPeriod | 15 | 32 |
| GOPSize | 8 | 8 |

Average bitrate savings are reported in Table 2. As expected, HEVC outperforms H.264/AVC with 23.0% bitrate savings for Luma and respectively 45.3% and 38.4% for Chroma U and V.

HEVC performance gains on legacy interlaced sequences are smaller than those obtain on progressive sequences since H.264/AVC disposed of specific interlaced tools. One must remark that no rigorously comparable GOP structure have been used to perform this assessment. Hence, for instance “non paired” hierarchical GOP has been considered.

Table 2 Results of BD-Rate comparing HM7.0 to H.264/AVC.

|  | BD-rate (H.264/AVC vs HM7.0) |  |  |
| --- | --- | --- | --- |
| Video Sequences | Y | U | V |
| Church_HD | -30.4 | -30.7 | -17.1 |
| Whale_Show_HD | -23.3 | -50.1 | -47.2 |
| Marching_in_HD | -15.3 | -55.0 | -50.9 |
| Overall | -23.0 | -45.3 | -38.4 |

## Subjective assessment

To fulfill the evaluation, HEVC performance have been subjectively assessed. The same three interlaced HD sequences have been encoded with HM 7.0. Each sequence has been encoded with RA-Main configuration at QP=37 leading to realistic target bitrates for this kind of material for broadcast applications (below 2Mbits/s).

Many artifacts appears on encoded sequences. In particular, main artifacts occur for Chroma component as mentioned in [1] and lead to so-called Chroma leaks, as reported below.

Marching:

Chroma leaks around walking people, stadium lines and flags.

In particular, have a look on:

-frames 115 to 125: around stadium lines.

-frames 145 to 158: below group of people with blue pants.

-frames 303 to 320: Red/Orange leak on the right of first line of athlete; Green leak below border between stadium and green ground.

Church:

Many Chroma leaks (Yellow/Purple) on horizontal window borders and colored window areas.

In particular:

-frames 115 to 125.

Many Green Chroma leaks on the Church stone (frame 317 on the right for instance).

Whale:

-frames 2 to 100: chroma leaks (blue/purple) around the swimming-pool borders

As discussed in [1], the observed artifacts come from chroma misalignment problems between both top and bottom fields when processing interlaced 4:2:0 content. In order to benefit from the HEVC coding efficiency on interlaced content as is the case for progressive content, it is necessary to correct these artifacts. This contribution proposes a solution.

##### Results available

Results of this experiments as well as original sequences are available at the following address:

ftp://mood.ateme.com/JMT_HEVC/Interlaced_ATEME_all.zip

Each test should be respectively downloaded from:

ftp://mood.ateme.com/JMT_HEVC/Interlaced_ATEME_Marching1080i.zip

ftp://mood.ateme.com/JMT_HEVC/Interlaced_ATEME_Church1080i.zip

ftp://mood.ateme.com/JMT_HEVC/Interlaced_ATEME_Whale1080i.zip

# Proposal

## Description

In this contribution, we propose to align both top and bottom fields chroma samples before encoding. The related chroma displacements have then to be signaled in the bitstream, in order, at the decoding side, to realign the chroma samples in a proper way (as in the original source content for instance).

In order to validate our proposal, various alignment policies have been studied and evaluated. Quite simple Lanczos 3 filtering (6 taps), has been used to shift the chroma components. As expected, the best configuration consist in a phase shifting of ¼ pixels down on the top field and of ¼ pixels up on the bottom field.

With such an approach, the encoding input chroma samples have the same relative phase with regard to luma samples, as for progressive material. Moreover, such phasing is fully in line with the so-called “nominal” chroma samples locations in progressive mode knowing.

Thus prior to encode, we recommend to perform a phase shift of ¼ pixels down on the top field and a phase shift of ¼ pixels up on the bottom field when encoding interlaced content with HEVC. Hence, based on the phase information signalled in the bitstream, the decoder side can properly de-shift the chroma samples in a suitable way.

## Signalling method

In the present contribution, we propose to signal the chroma samples shifting information using the SEI Field indication message expanded with complementary syntax elements.

## Syntax changes

It is proposed to insert syntax elements related to chroma samples location in the Field indication SEI message as described in Table 3.

Table 3. Proposed Field Indication SEI Message.

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| progressive_source_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_pic_flag ) |  |
| bottom_field_flag | u(1) |
| else if( !progressive_source_flag ) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit /* equal to 0 */ | u(1) |
| reserved_zero_3bit /* equal to 0 */ | u(3) |
| chroma_sample_loc_delta_present_flag | u(1) |
| if (chroma_sample_loc_delta_present_flag == 1 ) { |  |
| chroma_sample_loc_delta_x | u(3) |
| chroma_sample_loc_delta_y | u(3) |
| reserved_zero_2bit /* equal to 0 */ | u(2) |
| } |  |
| } |  |

Three new syntax elements (in red) are proposed chroma_sample_loc_delta_flag, chroma_sample_loc_delta_x and chroma_sample_loc_delta_y. The semantics of these elements are as follows.

- chroma_sample_loc_delta_flag equal to 1 indicates that chroma_sample_loc_delta_x and chroma_sample_loc_delta_y are present in the field indication SEI message. chroma_loc_info_present_flag equal to 0 indicates that chroma_sample_loc_delta_x and chroma_sample_loc_delta_y are not present in the field indication SEI message.

- chroma_sample_loc_delta_x, specifies the horizontal chroma samples phase shift, in units of fourth luma samples of a frame, for potential use in chroma samples inverse shifting process of the output decoded frame to recover the original source chroma samples location. The value of chroma_sample_loc_delta_x shall be in the range of -3 to +3, inclusive.

- chroma_sample_loc_delta_y, specifies the vertical chroma samples phase shift, in units of fourth luma samples of a frame, for potential use in chroma samples inverse shifting process of the output decoded frame to recover the original source chroma samples location. The value of chroma_sample_loc_delta_y shall be in the range of -3 to +3, inclusive.

- lès:

NOTE – source chroma sample location is indicated by the addition of the location specified by the VUI syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, as described in Section E.2.1 and illustrated in Error: Reference source not found, and the displacement signalled by chroma_sample_loc_delta_x and chroma_sample_loc_delta_y described in this section.

# Proposal assessment

## Proposed vs HM7.0

The proposed solution has been implemented in the current HM7.0 reference software. Before encoding, top and bottom fields of each input sequences are separated. Then, phase shifting of + ¼ pixels on the top field and - ¼ pixels on the bottom field are performed using Lanczos-3 filtering (6 taps). Then, after encoding and decoding with HM 7.0, inverse shifting is performed based on the information provided by the SEI field indication message.

Table 4 Results of BD-Rate comparing the current proposal  to HM7.0.

|  | BD-rate (RA-HE) |  |  |
| --- | --- | --- | --- |
| Video Sequences | Y | U | V |
| Church_HD | -0.01 | -5.5 | -2.8 |
| Whale_Show_HD | -0.10 | -2.7 | -6.0 |
| Marching_in_HD | -0.15 | -14.0 | -17.2 |
| Overall | -0.09 | -7.4 | -8.7 |

Strong improvement in coding efficiency of Chroma U and V components are reported with respectively an average of 7.4% and 8.7% bitrate savings associated to a reduced Luma improvement. Up to 17.2% for chroma V component of sequence Marching_in is reported.

These objective gains are associated to an efficient correction of chroma artifacts as can be observed on subjective assessment available at the following address:
 ftp://mood.ateme.com/JMT_HEVC/Interlaced_ATEME_all.zip (cf. section 2.3).

## Proposed vs H.264/AVC

In regard to table 2, the proposed solution has been evaluated against H.264/AVC. As expected, Chroma improvements are obtained as reported in table 5.

Table 5 Results of BD-Rate comparing the current proposal to H.264/AVC.

|  | BD-rate (H.264/AVC vs HM7.0 modified) |  |  |
| --- | --- | --- | --- |
| Video Sequences | Y | U | V |
| Church_HD | -30.4 | -33.6 | -18.8 |
| Whale_Show_HD | -23.4 | -51.1 | -49.3 |
| Marching_in_HD | -15.3 | -58.5 | -55.6 |
| Overall | -23.0 | -47.7 | -41.2 |

# Conclusion

In this contribution, a first performance assessment of interlaced material coding using both H.264/AVC and HEVC is proposed. Lot of Chroma artifacts are effectively reported when encoding interlaced material with current HM7.0. Most of them are removed with the proposed chroma shifting approach and a strong additional objective gain is reported.

Therefore, we suggest the JCTVC group to consider the current signaling proposal for adoption.

# Reference(s)

[1]	Jérôme Viéron, Jean-Marc Thiesse and Pierre Larbier, “On Chroma considerations for Interlaced material coding”, JCTVC-I0502, May, 2012, Geneva, CH.

[2] Rec. ITU-R BT.1210-3, "Test materials to be used in subjective assessment".

# Patent rights declaration(s)

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).