| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0260 |
| --- | --- |

| Title: | Source code bug fix on the RQ model equation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyomin Choi Junghak Nam and Donggyu Sim | Email: | hyomin06@kw.ac.kr qejixfyza@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University(KWU) |  |  |

_____________________________

# Abstract

This contribution corrects the RQ model equation in the rate control of HM software.

# Problem Statement

The current rate control software in the HM employs a quadratic pixel-based URQ model to obtain QP value. According to the equation, a QP value is derived by quadratic formula. However, there is a simple mistake. Therefore, this contribution proposed to correct the equation properly.

# Proposed Solutions

The quadratic URQ model is specified by

TijNpixels,ij=α×MADpred,ijQsij+β×MADpred,ijQsi2j                    (1)

where Npixels,i(j) is number of pixels in a frame, and Ti(j)/Npixels,i(j) is the average bit rate in bits per pixels (bpp) of the j-th frame in the i-th GOP. Qs indicates the quantization step size and the MAD value is predicted by simple linear model.

To derive Qs value from the equation, it should be modified in accordance with quadratic formula. Let’s assume that 1/Qsi(j) and Ti(j)/(Npixels,i(j)*MADpred,i(j)) are substituted by x and y, respectively. The equation (1) then,

y=β x2+α x                                     (2)

x=4βy+α24β2−α2β                                   (3)

After that, x and y are substituted by original terms, respectively. The equation (3) can be

1Qsi(j)=4β∗Ti(j)Npixels,i(j)∗MADpred,i(j)+α24β2−α2β                      (4)

However, there is a simple mistake in the software as follows,

qStep = 1/( sqrt((bppPerMAD/m_paramHighX1)+((m_paramHighX2*m_paramHighX2)/(4*m_paramHighX1*m_paramHighX1*m_paramHighX1))) - (m_paramHighX2/(2*m_paramHighX1))); qStep = 1/( sqrt((bppPerMAD/m_paramLowX1)+((m_paramLowX2*m_paramLowX2)/(4*m_paramLowX1*m_paramLowX1*m_paramLowX1))) - (m_paramLowX2/(2*m_paramLowX1)));

where, bppperMAD is equal to Ti(j)/(Npixels,i(j)*MADpred,i(j)). m_paramHighX1 and m_paramLowX1 correspond to β. α corresponds to m_paramHighX2 and m_paramLowX2.

Therefore, it should be corrected as follows

qStep = 1/( sqrt((bppPerMAD/m_paramHighX1)+((m_paramHighX2*m_paramHighX2)/(4*m_paramHighX1*m_paramHighX1))) - (m_paramHighX2/(2*m_paramHighX1))); qStep = 1/( sqrt((bppPerMAD/m_paramLowX1)+((m_paramLowX2*m_paramLowX2)/(4*m_paramLowX1*m_paramLowX1))) - (m_paramLowX2/(2*m_paramLowX1)));

# Patent rights declaration(s)

KWU does not have any current or pending patent rights relating to the technology described in this contribution.