| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0261 |
| --- | --- |

| Title: | Signaling of VPS Activation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 JungWon Kang, Hahyun Lee, Jinho Lee, Jin Soo Choi 218, Gajeong-no, Yuseong, Daejeon, Korea | Tel: Email: | - thang@u-aizu.ac.jp {jungwon, hanilee, jinosoul, jschoi}@ etri.re.kr |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

Video parameter set (VPS) can be used to describe essential information of “scalable layers” of a bitstream. However, to know which VPS is active, an extractor must parse slice headers, related PPS and SPS as well. This is obviously a big burden for any extractor. In this contribution, some simple mechanisms for activation signaling of video parameter set using additional messages are proposed.

# Introduction and Problem Statement

In the last meeting, video parameter set (VPS) is adopted to describe essential information of “scalable layers” of a bitstream. However, to know which VPS is active, an extractor must parse slice headers, and related PPS/SPS as well. This means that, an extractor must understand all the syntax elements in parameter sets and slice headers. This is obviously a big burden for any extractor. To solve this problem, we propose some simple mechanisms for activation signaling of video parameter set using additional message.

# Activation Signaling for Video Parameter Set (VPS)

The video parameter set could be sometimes updated. To let an extractor know which VPS is activated without having to parse slice headers and related PPS/SPSs, we propose to include a new element vps_id to indicate the identifier of the video parameter set to be used for the NAL units inside the related access unit. The following proposed options are different in the locations of the vps_id.

In the first option, vps_id element is added to the access unit delimiter. The syntax of the modified access unit delimiter is shown below.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| vps_id_present_flag | u(1) |
| if( VPS_id_present_flag ) |  |
| vps_id | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

- vps_id_present_flag: if equal to 0, indicates that vps_id element is present in the access unit delimiter.

- vps_id: identifies the video parameter set referred by the access unit. The value of vps_id shall be in the range of 0 to X, inclusive.

In the second option, a new SEI message called parameter_set_reference() is used to specifically convey the vps_id element. The syntax of the SEI message is shown below.

| parameter_set_reference( payloadSize ) { | Descriptor |
| --- | --- |
| vps_id | ue(v) |
| } |  |

In the third option, Recovery point SEI message is extended to convey vps_id element. The semantics of  vps_id_present_flag and vps_id is the same as the above.

| recovery_point( payloadSize ) { | Descriptor |
| --- | --- |
| vps_id_present_flag | u(1) |
| if( vps_id_present_flag ) |  |
| vps_id | ue(v) |
| recovery_poc_cnt | ue(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| } |  |

Note that, the above options to convey vps_id value are only used at IDR access unit. To extract a bitstream, an extractor just has to maintain one or more VPS and detects the value of vps_id element if at least one of these options is used in an IDR access unit.

# References

[1] T. C. Thang, J. W. Kang, H. Lee, J. S. Choi, "High-level syntax modifications to support extractor operation" JCTVC-I0252, 9th meeting: Geneva, CH, 27 April – 7 May, 2012.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).