| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0262 |
| --- | --- |

| Title: | Initial QP set for rate control |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyomin Choi Junghak Nam Donggyu Sim and Seoungjun Oh | Email: | hyomin06@kw.ac.kr qejixfyza@kw.ac.kr dgsim@kw.ac.kr sjoh@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

This contribution presents a condition of initial QP setting in beginning of a sequence for the rate control implemented on HM7.0 and complexity of the rate control. The initial QP setting condition was already introduced at the last meeting, JCTVC-I0094 includes identical condition. In this document, the identical condition is implemented in HM7.0 and tested. The initial QP is automatically calculated based on bit per pixel (bpp) with target bitrate. Therefore, the QP value in the configuration file is ignored. Furthermore, this proposal provides complexities by rate control in encoding.

# Introduction

A rate control algorithm for HEVC, JCTVC-H0213, was introduced and adopted to implement in HM at the last San Jose meeting. However, a problem in the rate control is that the QP value for beginning of a sequence is affected by the configuration file setting. For practical rate control algorithm, QP value at the beginning of the sequence is automatically calculated by predefined condition because we do not have an idea how many bits are generated by the manually defined QP value for every time. Therefore, we propose the condition to calculate the initial QP value based on bpp with target bitrate.

# Initial QP setting

At the beginning of a sequence, the frame of the zeroth GOP, initial QP value is derived. For deriving the QP, bppinit should be calculated with input target bitrate by formula (7).

bppinit = R01f×Npixels                                 (7)

where Npixels is number of pixels for a frame, Ri(j) is the available bandwidth and f indicates predefined frame rate. The initial QP is set by the value of bppinit as follows,

QPi1=12 0.7780 <bppinit 17 0.3200 <bppinit ≤ 0.778022 0.1220 <bppinit ≤ 0.320027 0.0469 <bppinit ≤ 0.122032 0.0213 <bppinit ≤ 0.046937 0.0102 <bppinit ≤ 0.021342 0.0049 <bppinit ≤ 0.010247 0.0024 <bppinit ≤ 0.0049                        (8)

Threshold values are statistically obtained and approximated.

# Experiments

For experiments, we provide results with encoding complexity. The hardware and software specification is as follow

Table I. Test platform specification

| Items | Specification |
| --- | --- |
| CPU | Intel Core i7 6-core 3.49GHz |
| Memory | 8GB |
| OS | Window7 64bit |
| compiler | VS2008 |

First, we test initial rate control (RC) implemented in HM7.0 against to HM7.0 without rate control. In this case, the input target bits are assigned as output bitrate by HM7.0. In addition, QP values are manipulated by configuration as well as HM7.0 anchor. The accuracy and complexity of initial RC version are described in Table II.

Table II. Accuracy and complexity of initial RC on HM7.0 compared to initial HM7.0

| Main profile | Random access |  |  | Low delay |  |  | Low delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime |
| ClassA | 1.40% | -1.32 | 102.36 |  |  |  |  |  |  |
| ClassB | 0.91% | -1.31 | 102.32 | 0.20% | -1.02 | 98.66 | 0.22% | -0.95 | 98.79 |
| ClassC | 0.62% | -1.58 | 102.43 | 0.35% | -0.53 | 101.38 | 0.28% | -0.55 | 101.82 |
| ClassD | 0.67% | -1.39 | 102.88 | 0.09% | -0.44 | 102.33 | 0.10% | -0.46 | 103.07 |
| ClassE |  |  |  | 0.36% | -0.83 | 97.09 | 0.36% | -0.75 | 96.68 |
| Avg | 0.90% | -1.40 | 102.50 | 0.25% | -0.70 | 99.87 | 0.24% | -0.68 | 100.09 |

Table III shows the accuracy and complexity of modified RC including initial QP setting for beginning of the sequence compared to the initial HM7.0

Table III. Accuracy and complexity of modified RC on HM7.0 compared to initial HM7.0

| Main profile | Random access |  |  | Low delay |  |  | Low delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime |
| ClassA | 1.16% | -1.45 | 103.16 |  |  |  |  |  |  |
| ClassB | 1.03% | -1.44 | 103.00 | 0.22% | -1.02 | 99.50 | 0.16% | -0.97 | 99.72 |
| ClassC | 0.63% | -1.64 | 101.61 | 0.37% | -0.55 | 101.38 | 0.26% | -0.56 | 101.90 |
| ClassD | 0.65% | -1.38 | 102.55 | 0.09% | -0.46 | 102.36 | 0.10% | -0.49 | 103.04 |
| ClassE |  |  |  | 0.38% | -1.91 | 97.75 | 0.31% | -1.82 | 97.50 |
| Avg | 0.87% | -1.48 | 102.58 | 0.26% | -0.99 | 100.24 | 0.21% | -0.96 | 100.54 |

# Patent rights declaration(s)

KWU does not have any current or pending patent rights relating to the technology described in this contribution.