| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0263 |
| --- | --- |

| Title: | Reports on improvement of the rate control |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyomin Choi Junghak Nam Sunmi Yoo Donggyu Sim (KWU) Yong-Hwan Kim and Jiho Park (KETI) | Email: | hyomin06@kw.ac.kr |
| Source: | Kwangwoon University (KWU) Multimedia IP research center, KETI |  |  |

_____________________________

# Abstract

This contribution presents improvement of rate control algorithm based on JCTVC-H0213 and JCTVC-I0094. This document proposes a scheme of boundary clipping for target bits in the frame level rate control. The boundary for target bits of a frame is used to conform to the hypothetical reference decoder (HRD) requirement, so the target bits are clipped by lower and upper bounds. The lower and upper bounds are adaptively derived from the amount of coded picture buffer (CPB). In addition, this document introduces an adaptive MAD prediction on LCU level regarding to correlation of spatial and temporal MAD cost to improve accuracy of MAD prediction.

# Introduction

A rate control algorithm for HEVC, JCTVC-H0213, was introduced and adopted to implement in HM at the last San Jose meeting. The introduced rate control doesn’t take care about boundary of frame bits. In real time processing, it may or may not cause under- or over-flow of a coded picture buffer (CPB). Therefore, we propose a simple equation that clips target bit for a frame with considering HRD conformance. To improve accuracy of predicted MAD, we propose adaptive MAD prediction on LCU level regarding to correlation of MAD values between either spatial or temporal MAD values.

# Proposed Solutions

## Target bits clipping

The target bit for a frame computed by current methods implemented in SW is clipped as follow

Ti(j) = minUbound,ij,maxLbound,ij,Ti(j)                       (1)

where, T means target bits for the j-th frame of i-th GOP, U and L indicate upper and lower bounds, respectively.

Upper and Lower bound are computed by formula (2) and (3),

Lbound,ij=Fij−ω×Vij j=1                     (2)

Ubound,ij= Cij                                   (3)

where, ω can be 0.9, V indicates the occupancy of virtual buffer at the j-th frame of i-th GOP, remaining bits C at the j-th frame of i-th GOP for a sec according to CPB is computed by (4), F is frame bits computed based on remaining bits in a sec by formula (5)

Cij=Rij×δ if i ==0 Cij−1+Rij×δ else if i×NGOP+j == f          (4)

where, δis set to 0.9, R is target bitrate at the j-th frame of i-th GOP, b is actual generated bits for j-1 th frame of i-th GOP. NGOP indicates number of frames in a GOP and f means pre-defined frame rate.

Fij=Cijf when low delay                (5)

where, Wref,i means weighting factor for reference frames for i-th GOP.

Lower bound is computed on the frame bits based on remaining bits in a sec and adjusted by buffer occupancy V. After coding an entire GOP, the lower bound is re-calculated based on updated F at the 1st frame of next GOP. The upper bound is calculated for every frame with updated remaining bits C for a second. Initial C is equal to 0.9*target bitrate and it is updated in every second. Generally, it is subtracted by actual generated bits. For low delay case, the frame bits based on the C is divided by frame rate. The F is updated at the first frame of new second because C is also updated at that time. On the other hands, the weighting factors Wref,i for reference frames in a GOP are considered by computing F. The Wref,i is computed by

Wref,i=j=1Nrefbij×MADijj=1NGOPbijMADij                                 (6)

## Adaptive MAD prediction

Current rate control SW in HM, predicted MAD cost for a LCU is always MAD value of corresponding LCU to the previous frame. To use always corresponding MAD makes accuracy of predicted MAD poor. Therefore, we propose simple adaptive MAD prediction to improve predicted MAD accuracy against the current scheme.

The figure 1 shows an example of adaptive MAD prediction with LCU blocks position. Let us assume that the current LCU ‘C’ at ‘t’ time will be coded by computed QP value by RQ model. To run the RQ model, we have to get predicted MAD value. The current software always exploits the MAD value of ‘E’ LCU at ‘t-1’ time.

Figure 1. Example of adaptive MAD prediction

The proposed adaptive MAD prediction considers MAD differences in between spatial neighbors and temporal neighbors. To find correlation between temporal neighbors, absolute value of which difference between MAD of ‘A’ LCU and ‘D’ LCU.

MADtemp =MADA−MADD

In addition, absolute value of which difference between MAD of ‘A’ LCU and ‘B’ LCU is exploited by spatial correlation.

MADspat =MADA−MADB

After computing the each MAD absolute difference between temporal and spatial neighbors, the final predicted MAD cost is set to average value depending on either temporal or spatial  which have the smallest difference as follow

MADpred =AVGMADA+MADD if MADtemp≤MADspat

# Experiments

For experiments, we provide results with encoding complexity. The hardware and software specification is as follow

Table I. Test platform specification

| Items | Specification |
| --- | --- |
| CPU | Intel Core i7 6-core 3.49GHz |
| Memory | 8GB |
| OS | Window7 64bit |
| compiler | VS2008 |

Table II shows the accuracy and complexity of proposed RC compared to modified RC by JCTVC-J0262 on HM7.0

Table III. Accuracy and complexity of proposed RC compared to modified RC by JCTVC-J0262 on HM7.0

| Main profile | Random access |  |  | Low delay |  |  | Low delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime | Δkbps | ΔPSNR | encTime |
| ClassA | 0.10% | 0.07 | 99.19 |  |  |  |  |  |  |
| ClassB |  |  |  |  |  |  | 0.13% | 0.24 | 100.06 |
| ClassC | 1.31% | -0.16 | 98.81 | 0.40% | 0.01 | 99.55 | 0.10% | 0.00 | 99.51 |
| ClassD | 0.09% | 0.03 | 93.62 | 0.05% | -0.16 | 99.28 | 0.04% | -0.18 | 99.22 |
| ClassE |  |  |  | 0.30% | 0.00 | 99.79 | 0.28% | -0.03 | 99.77 |
| Avg | 0.50% | -0.02 | 97.21 | 0.25% | -0.05 | 99.54 | 0.14% | 0.01 | 99.64 |

# Patent rights declaration(s)

KWU and KETI do not have any current or pending patent rights relating to the technology described in this contribution.