| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 July – 20 July 2012 | Document: JCTVC-J0264 |
| --- | --- |

| Title: | Dependent slices support in HEVC main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Schierl V. George A. Henkel D. Marpe Fraunhofer HHI Einsteinufer 37 D-10587 Berlin G. Clare F. Henry Orange Labs 4 rue du Clos Courtel 35510 Cesson Sevigne France Kimihiko Kazui Fujitsu Laboratories LTD. 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki Japan Stéphane VALENTE ST-Ericsson 29, boulevard Romain Rolland F-75669 Paris Cedex 14 France | Tel: Email: Tel: Email: Tel: Email: Email: | +49-30-31002-227 thomas.schierl@hhi.fraunhofer.de +33 2 99124558 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com stephane.valente@stericsson.com |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

On the 9th JCT-VC meeting, the dependent slices [JCTVC-I0229] have been added to the standard, while the decision for inclusion in the main profile is pending. As summary, the dependent slice concept allows for information exchange between slices for both the parsing and reconstruction process. The original contribution presented as one benefit the enabling of low delay coding and transmission with wavefront parallel processing (WPP).

In this contribution will give different use cases and explanations why dependent slices introduce general benefit for the standard and therefore request the inclusion into the main profile.

The presented use cases comprise bit stream rewriting, bit stream entry point signalling, low delay HRD operations based on decoding units, as well as a general design for inclusion of the parallelization techniques of entropy slices and WPP.

# Introduction

The dependent slices have been added to the HEVC standard draft 7 [JCTVC-I1003], while the decision for inclusion in the main profile is pending.

Dependent slices give a uniform design for different already introduced techniques in HEVC, such as Entropy Slices and entry point signalling for WPP/tiles.

Furthermore, dependent slice allow for bitstream re-packetization in use cases where error robustness is a minor issue, e.g. in quasi error-free broadcast channels or error-free communication over satellite.

Dependent slices can be also used for a low level bitstream rewriting operation, which allows the re-writing of a single slice per picture bitstream to a WPP supporting bit stream by rewriting the entropy coding only, instead of fully re-encoding the bitstream without the associated coding losses.

Sub-picture CPB operations as defined for decoding units can benefit from using dependent slices in order to operate along decoding unit structures without sacrificing coding efficiency.

The use cases are more detailed in the following subsections.

Due to the mentioned benefits of using dependent slices as a generic tool for different use cases, this contribution requests the inclusion of the dependent slices into the main profile.

It is further proposed to keep slice rates as defined by levels also for dependent slices.

# Dependent slices

Dependent slices describe a part of a normal slice, therefore a dependent slice requires the slice header of a preceding normal slice, with a full slice header, whereas a dependent slice does only provide a subset of a slice header. This slice header includes

- first slice in pic flag

- corresponding Picture Parameter Set id

- slice address

- the entry point table signalling

- as well as the slice extensions.

Dependent slice can coexist with normal slices in the same picture without any issue, while the normal slice keeps it robustness to errors since it is independently coded and provides the full slice header. Actually, the dependent slice is very close to a normal slice, while not breaking entropy and prediction over slice boundaries. The dependent slice allows for fragmentation of slice data at codec level while not sacrificing coding efficiency. Use cases are presented in the following section.

# Dependent slices use cases

## WPP low delay

Low delay applications such as remote surgery, video gaming interactive video conferencing, as well as TV live contribution are getting more interest these days. It is expected that such applications will gain from high resolutions such as 1080p and 2160p. The processing of remote video material, such as coming from endoscopy camera, typically requires the use of parallelization techniques in order to overcome hardware limitation wrt. single-threaded real-time encoding. One technique to overcome these limitations is WPP. Unfortunately, WPP structures the picture into substreams, which have dependencies to each other. Those substreams, e.g. if applied as one substream per ctb row, may be contained in a single slice per picture, but in order to allow an immediate transport after encoding such a substream, each substream would need to be in its own slice. This would result in coding efficiency loss, due to broken dependencies in the reconstruction as well as in the parsing process.

The concept of dependent slices, as proposed in this contribution, allows for information exchange between slices for both the parsing and reconstruction process. This enables low delay coding and transmission with a minimum bit rate overhead for WPP.

To fully make use of dependent slices with WPP, a decoding unit should be assigned to a WPP sub stream.

## Bit stream rewriting

In case a system standard should require a certain NAL unit size for optimal transport or storage of the bit stream, a re-writing device may re-write the slice NAL unit(s) of the existing bit stream into dependent slices of the maximum required size by a single entropy decoding and re-encoding operation, without the need of full re-encoding with the associated coding loss as required for (“independent”) slices. In RTP payload formats, there is typically a Fragmentation Unit mechanism to spread data of a slice to different packets to match MTU restrictions. With dependent slice a fully system standard independent device could apply such an operation to existing bit streams.

On the other hand, a bitstream containing multiple dependent slices per picture can be rewritten to a bitstream with one slice per picture without the need of full re-encoding and the associated coding loss.

## Use dependent slices as entry point markers for WPP and tiles

On the last JCT-VC meeting, it has been decided to remove bit stream markers for entry point signalling, but there was the decision to keep the functionality of markers for, e.g., low delay applications. With applying a single dependent slice per WPP sub stream the functionality of entry points can be achieved implicitly, while using the slice address of the dependent slice header for the association of the dependent slice to a WPP substream. The same method may be used for tiles in an equal manner, where the association of dependent slice also would be based on the tile dimensions and the signalled slice address.

  - Sub picture CPB operations

In case of sub-picture CPB operations, it may be beneficial to decide on the fly, while encoding, to finalize a decoding unit in order to meet CPB timing constraints or decoding unit structures as signalled in the Picture timing SEI message [JCTVC-I1003]. Dependent slices allow for keeping all dependencies in the entropy and reconstruction process in order to not affect coding efficiency, while an insertion of a normal slice for finalizing a decoding unit would sacrifice coding efficiency for some low delay applications not requiring the use of normal slices.

## Generic design for different high level techniques in HEVC

The dependent slices further generalize the use of different tools already contained in HEVC. These tools are WPP, entropy slices, and entry point markers. Furthermore, the dependent slice also gives the ability for fragmentation of pictures on slice level without the need for breaking all dependencies as required by slices. Without dependent slice, such fragmentation was applied on the system or transport layer, without any possible interaction of the codec layer. Furthermore, dependent slices allow, as mentioned before for re-writing of existing bit streams to WPP or a certain MTU size without a full re-encoding process, but with an entropy re-encoding only.

# Coding results

The set of results can be found in the attached Excel sheet.

# Conclusion and further aspects

We request the inclusion of dependent slices in the main profile of HEVC.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FUJITSU LIMITED does not have any current or pending patent rights relating to the technology described in this contribution.

ST-Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).