| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0265 |
| --- | --- |

| Title: | Intra Transform Skipping: Smallest CU and Implicit Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two topics on intra transform skipping. Part 1 proposes to restrict transform skipping for 4x4 intra TUs further to the SCU level. This eliminates needless signalling of the transform_skip_flags for the larger CU sizes and speeds up the encoder in normal mode by about 8% and in fast mode by about 4% without significant coding efficiency loss.

The experiments in part 2 of this proposal indicate that the coding efficiency of chroma 4x4 intra TU transform skipping is a rather modest 2.5% on average for class F, while doubling the encoding time increase of luma transform skipping. Part 2 of this contribution proposes to define the HEVC transform skipping for 4x4 TUs on the fast encoder mode and eliminate signalling of transform_skip_flags for chroma, while implicitly deriving them from the luma flags as is performed in the fast encoder mode.

# Introduction

During the 9th JCT-VC meeting in Geneva, Switzerland, transform skipping for 4x4 intra TUs was adopted. Apart from adding one flag to indicate whether a 4x4 intra TU uses transform skipping or not, there is no change to the prediction, quantization (scaling), in-loop filters and entropy coding modules. It is enabled by the transform_skip_enabled_flag in the SPS and by the transform_skip_flag in the residual coding syntax [1][2].

The transform_skip_flag is signaled for every 4x4 intra TU that is chosen by RQT within an intra-coded coding unit (CU) of any allowed size. However, our experiments in part 1 of this proposal indicate that the coding efficiency from intra transform skipping is achieved when the smallest coding unit (SCU) size is chosen by the HM7.0 encoder. Therefore, the current method has a computational overhead. Part 1 of this contribution proposes to restrict transform skipping for 4x4 intra TUs further to the SCU level. This eliminates needless signalling of the transform_skip_flags for the other CU sizes and speeds up the encoder in normal mode by about 8% and in fast mode by about 4%.

The current transform skip method supports 4x4 intra transform skipping for both luminance and chrominance components. This means that the transform_skip_flag is signaled for both luma and chroma 4x4 intra TUs. The HM7.0 encoder has two modes: “normal” and “fast”. In the normal encoder mode, full RDO is performed for both luma and chroma independently. In the fast encoder mode, the values of the chroma transform_skip_flags are derived from the corresponding values of the luma transform_skip_flags. However, in fast encoder mode the chroma transform_skip_flags are still signalled. In addition, our experiments in part 2 of this proposal indicate that the coding efficiency of chroma 4x4 intra TU transform skipping is a rather modest 2.5% on average for class F, while doubling the encoding time increase of luma transform skipping. Part 2 of this contribution proposes to define the HEVC transform skipping for 4x4 TUs on the fast encoder mode and eliminate signalling of separate transform_skip_flags for chroma, while implicitly deriving them from the luma flags as is performed in the fast encoder mode.

# Part 1: Intra Transform Skipping Restricted to SCU

## Part 1: Description

It is proposed to restrict the signaling of the transform_skip_flag to the SCU size only, which is determined by the SPS syntax parameter log2_min_coding_block_size_minus3. Table 1 illustrates the residual coding syntax change in yellow highlight.

Table 1 Residual coding syntax change to restrict signalling of transform_skip_flag to SCU

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && PredMode = = MODE_INTRA) && (log2CbSize = = Log2MinCbSize) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| …… |  |

## Part 1: Results

### All Intra Test Conditions

Table 2 enumerates BD-rate results for 4x4 intra TU transform skipping in normal encoder mode. The reference is HM7.0 with transform skipping disabled for both Main and HE10. Results for Lowdelay and Random Access test conditions will be provided in Section 2.2.2.

Table 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.03% | -0.04% | -0.13% | 0.03% | 0.03% | -0.03% |
| Class B | 0.04% | -0.02% | -0.15% | 0.03% | -0.04% | -0.09% |
| Class C | -0.05% | -0.08% | -0.16% | -0.06% | -0.03% | -0.09% |
| Class D | 0.00% | -0.05% | -0.23% | -0.03% | -0.01% | -0.14% |
| Class E | 0.09% | 0.00% | -0.34% | 0.08% | 0.00% | -0.31% |
| Class F | -7.54% | -7.41% | -7.98% | -7.45% | -6.46% | -7.32% |
| Overall | -1.24% | -1.27% | -1.49% | -1.24% | -1.09% | -1.32% |
|  | -1.24% | -1.27% | -1.48% | -1.24% | -1.09% | -1.31% |

BD-rate results for the proposed intra transform skipping method restricted to SCU are summarized in Table 3. In this experiment the SCU size is 8x8. The reference is HM7.0 with transform skipping enabled (Table 2). The results indicate that there is a negligible coding efficiency difference compared with the total gain of intra transform skipping reported in Table 2.

Table 3

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | -0.02% | 0.00% | -0.06% | -0.01% |
| Class B | 0.00% | 0.00% | 0.01% | 0.00% | 0.02% | -0.02% |
| Class C | 0.01% | -0.01% | 0.02% | 0.01% | -0.02% | -0.01% |
| Class D | 0.01% | 0.03% | 0.00% | 0.01% | 0.02% | -0.03% |
| Class E | -0.01% | 0.02% | -0.02% | 0.00% | -0.02% | 0.01% |
| Class F | 0.07% | 0.10% | 0.05% | 0.06% | -0.08% | 0.13% |
| Overall | 0.01% | 0.02% | 0.01% | 0.01% | -0.02% | 0.01% |
|  | 0.01% | 0.02% | 0.01% | 0.01% | -0.02% | 0.02% |

The durations of All Intra encodings were measured on a single PC. Table 4 summarizes the timings for transform skipping (TS) in normal and fast encoder modes relative to HM7.0 with TS off. In normal encoder mode the speedup of SCU-level transform skipping (TS-SCU) is about 8%, while in fast encoder mode the speedup is about half or 4%. Relative speedups are similar for Main and HE10 test conditions. Decoding timings for TS-SCU are equal to the HM7.0 decoder timings.

Table 4

| Encoder Mode | TS = off | TS = on | TS-SCU = on |
| --- | --- | --- | --- |
| Normal | 100% | 133% | 125% |
| Fast | 100% | 115% | 111% |

Results for SCU size 16x16 are summarized in Table 5. The reference is HM7.0 intra transform skipping with SCU equal to 16x16. Class B results could not be produced, because the picture height is not a multiple of the SCU size.

Table 5

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.004% | -0.015% | 0.013% | -0.003% | 0.027% | 0.013% |
| Class B |  |  |  |  |  |  |
| Class C | -0.002% | 0.003% | -0.012% | 0.000% | -0.005% | 0.006% |
| Class D | -0.001% | -0.005% | 0.000% | 0.000% | 0.009% | 0.003% |
| Class E | 0.003% | -0.014% | 0.056% | -0.004% | -0.011% | 0.001% |
| Class F | -0.006% | -0.017% | 0.017% | -0.005% | -0.034% | -0.037% |
| Overall | -0.002% | -0.009% | 0.013% | -0.002% | -0.002% | -0.003% |
|  | -0.002% | -0.002% | 0.012% | -0.002% | 0.003% | -0.003% |

In summary, the BD-rate changes in Table 3 and Table 5 corresponding with the proposed SCU restriction are negligible compared with the total BD-rate gains achieved with intra transform skipping under All Intra test conditions, while significantly speeding up the encoder. In the following section, it is demonstrated that this conclusion also holds for the Lowdelay and Random Access test conditions.

### Lowdelay and Random Access Test Conditions

Table 6 is the extension of Table 2 and enumerates the Lowdelay and Random Access BD-rate results for HM7.0 intra transform skipping.

Table 6

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.45% | -0.22% | 0.00% | -0.37% | -0.42% |
| Class B | 0.03% | -0.05% | -0.38% | 0.04% | -0.21% | -0.15% |
| Class C | -0.11% | -0.35% | -0.39% | -0.10% | -0.38% | -0.13% |
| Class D | 0.04% | 0.01% | -0.38% | -0.03% | -0.02% | -0.08% |
| Class E |  |  |  |  |  |  |
| Class F | -5.50% | -6.36% | -6.88% | -5.59% | -6.31% | -6.50% |
| Overall | -1.06% | -1.37% | -1.59% | -1.08% | -1.40% | -1.39% |
|  | -1.05% | -1.37% | -1.64% | -1.08% | -1.37% | -1.40% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | 0.05% | 0.12% | 0.03% | -0.08% | 0.05% |
| Class C | -0.01% | -0.02% | 0.20% | 0.00% | -0.09% | 0.00% |
| Class D | 0.08% | 0.26% | 0.08% | 0.08% | -0.03% | -0.13% |
| Class E | -0.11% | 0.46% | -0.24% | -0.14% | 0.27% | -0.09% |
| Class F | -3.32% | -3.84% | -3.47% | -3.29% | -4.25% | -4.70% |
| Overall | -0.66% | -0.64% | -0.65% | -0.65% | -0.85% | -0.97% |
|  | -0.66% | -0.70% | -0.63% | -0.65% | -0.96% | -1.03% |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.02% | 0.02% | -0.31% | 0.04% | -0.19% | 0.43% |
| Class C | 0.02% | -0.20% | 0.24% | -0.01% | -0.07% | -0.14% |
| Class D | 0.09% | 0.37% | 0.34% | 0.05% | 0.10% | 0.61% |
| Class E | -0.01% | 0.14% | 0.68% | -0.03% | 0.14% | -1.00% |
| Class F | -3.19% | -3.54% | -3.50% | -3.23% | -4.22% | -4.58% |
| Overall | -0.62% | -0.65% | -0.56% | -0.63% | -0.86% | -0.87% |
|  | -0.63% | -0.67% | -0.55% | -0.63% | -0.88% | -1.00% |

Table 7 is the extension of Table 3 and enumerates the BD-rate changes of the proposal (SCU restriction) compared with HM7.0 with intra transform skipping enabled for Main and HE10 (Table 6).

Table 7

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.17% | 0.02% | 0.01% | 0.20% | -0.08% |
| Class B | 0.01% | -0.07% | 0.01% | -0.01% | -0.04% | 0.03% |
| Class C | 0.02% | 0.14% | 0.01% | 0.01% | 0.11% | -0.06% |
| Class D | -0.01% | 0.01% | -0.14% | 0.01% | -0.14% | -0.25% |
| Class E |  |  |  |  |  |  |
| Class F | 0.07% | 0.19% | 0.17% | 0.04% | 0.16% | -0.08% |
| Overall | 0.03% | 0.08% | 0.01% | 0.01% | 0.05% | -0.08% |
|  | 0.03% | 0.06% | 0.03% | 0.01% | 0.02% | -0.06% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | -0.05% | -0.06% | 0.00% | 0.07% | 0.05% |
| Class C | 0.02% | 0.05% | -0.14% | -0.03% | -0.11% | 0.11% |
| Class D | -0.02% | 0.37% | 0.59% | -0.06% | 0.30% | 0.06% |
| Class E | -0.08% | -0.79% | -1.14% | 0.09% | -0.23% | 0.08% |
| Class F | 0.16% | 0.64% | 0.60% | 0.10% | -0.03% | 0.73% |
| Overall | 0.01% | 0.08% | 0.02% | 0.02% | 0.02% | 0.21% |
|  | 0.01% | 0.10% | -0.05% | 0.01% | 0.16% | 0.33% |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.03% | -0.02% | 0.30% | -0.07% |
| Class C | 0.00% | -0.10% | -0.41% | -0.01% | -0.08% | 0.19% |
| Class D | -0.06% | 0.02% | 0.16% | 0.00% | 0.51% | -0.79% |
| Class E | -0.04% | -0.57% | -1.70% | -0.08% | -0.20% | 0.13% |
| Class F | 0.04% | 0.05% | 0.54% | 0.09% | 0.31% | -0.30% |
| Overall | -0.01% | -0.09% | -0.19% | 0.00% | 0.19% | -0.18% |
|  | -0.01% | -0.10% | -0.24% | 0.00% | 0.18% | 0.00% |

# Part 2: Intra Transform Skipping With Implicit Chroma

## Part 2: Description

The current fast encoder mode in HM7.0 derives the values of the chroma transform_skip_flags from the values of the luma transform_skip_flags. If the chroma type is 4:2:0, then 4 luma 4x4 intra TUs correspond with one chroma 4x4 intra TU. The HM7.0 encoder determines the value of the chroma transform_skip_flag for the chroma 4x4 intra TU as follows. If all four luma 4x4 intra TUs have a transform_skip_flag value equal to 1, then the corresponding transform_skip_flag of the chroma 4x4 intra TU is set equal to 1. However, the HM7.0 encoder signals this derived chroma transform_skip_flag. The proposal is to disable signalling of the chroma transform_skip_flag and implicitly assign the value that is derived from the corresponding luma transform_skip_flags. To avoid drift between encoder and decoder, the rule that is employed by the encoder must also be applied by the decoder.

Table 8 Residual coding syntax change to restrict signalling of transform_skip_flag to luma

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && (cIdx == 0) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| ... |  |

## Part 2: Results

### All Intra Test Conditions

Table 9 enumerates BD-rate results for HM7.0 intra transform skipping in fast encoder mode. The reference is HM7.0 with transform skipping disabled for both Main and HE10.

Table 9

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.09% | 0.06% | 0.02% | 0.06% | 0.13% |
| Class B | 0.02% | 0.12% | 0.12% | 0.02% | 0.09% | 0.14% |
| Class C | -0.08% | 0.25% | 0.27% | -0.08% | 0.28% | 0.27% |
| Class D | -0.04% | 0.30% | 0.33% | -0.04% | 0.27% | 0.31% |
| Class E | 0.07% | 0.12% | 0.09% | 0.06% | 0.08% | 0.14% |
| Class F | -7.13% | -5.12% | -5.08% | -7.12% | -4.91% | -5.12% |
| Overall | -1.19% | -0.71% | -0.70% | -1.19% | -0.69% | -0.69% |
|  | -1.19% | -0.69% | -0.69% | -1.19% | -0.67% | -0.67% |

The following Table 10 compares the BD-rates of fast encoder mode with normal encoder mode. For all test sequence classes, the loss of fast encoder mode is about 0.6% on average for chroma. For the class F sequences the loss is about 2.5% on average, which is a rather modest loss for chroma.

Table 10

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | 0.12% | 0.19% | -0.01% | 0.03% | 0.16% |
| Class B | -0.02% | 0.15% | 0.27% | -0.01% | 0.13% | 0.23% |
| Class C | -0.04% | 0.33% | 0.43% | -0.02% | 0.31% | 0.36% |
| Class D | -0.03% | 0.35% | 0.56% | -0.01% | 0.28% | 0.44% |
| Class E | -0.02% | 0.12% | 0.43% | -0.02% | 0.08% | 0.45% |
| Class F | 0.47% | 2.51% | 3.25% | 0.38% | 1.67% | 2.43% |
| Overall | 0.06% | 0.60% | 0.85% | 0.05% | 0.42% | 0.67% |
|  | 0.06% | 0.62% | 0.86% | 0.05% | 0.44% | 0.68% |

Table 11 enumerates BD-rate changes corresponding with the proposed intra transform skipping with implicit chroma. The reference is HM7.0 with transform skipping enabled in fast encoder mode (Table 9).

Table 11

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% | -0.06% |
| Class B | -0.02% | -0.03% | -0.03% | -0.01% | -0.03% | -0.04% |
| Class C | -0.03% | -0.03% | -0.01% | -0.03% | -0.06% | -0.03% |
| Class D | -0.05% | -0.08% | -0.08% | -0.04% | -0.11% | -0.13% |
| Class E | -0.03% | -0.03% | -0.02% | -0.03% | -0.04% | -0.02% |
| Class F | -0.11% | -0.21% | -0.16% | -0.10% | -0.18% | -0.19% |
| Overall | -0.04% | -0.06% | -0.05% | -0.04% | -0.07% | -0.08% |
|  | -0.04% | -0.06% | -0.05% | -0.04% | -0.08% | -0.08% |

In summary, the coding efficiency gains in Table 11 reduce the coding loss somewhat of fast encoder mode compared with normal encoding mode. The BD-rate results in Table 12 compare the proposed implicit chroma intra transform skipping with HM7.0 intra transform skipping in normal encoder mode.  Since the overall coding loss is negligible and the coding loss for class F sequences is acceptable compared with the total gain of intra transform skipping, it is recommended to adopt the proposed intra transform skipping with implicit chroma into HEVC. This conclusion also holds for the BD-rate results for Lowdelay and Random Access test conditions presented in the following Section 3.2.2.

Table 12

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.11% | 0.19% | -0.02% | 0.00% | 0.10% |
| Class B | -0.03% | 0.12% | 0.25% | -0.03% | 0.11% | 0.19% |
| Class C | -0.07% | 0.31% | 0.42% | -0.05% | 0.25% | 0.33% |
| Class D | -0.08% | 0.27% | 0.48% | -0.05% | 0.17% | 0.32% |
| Class E | -0.05% | 0.09% | 0.41% | -0.04% | 0.04% | 0.43% |
| Class F | 0.36% | 2.30% | 3.08% | 0.28% | 1.49% | 2.23% |
| Overall | 0.02% | 0.53% | 0.80% | 0.01% | 0.35% | 0.59% |
|  | 0.02% | 0.56% | 0.80% | 0.01% | 0.36% | 0.60% |

### Lowdelay and Random Access Test Conditions

The following Table 13 is an extension of Table 9. The table enumerates BD-rate results for HM7.0 intra transform skipping in fast encoder mode. The reference is HM7.0 with transform skipping disabled for both Main and HE10.

Table 13

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.27% | -0.19% | 0.02% | -0.25% | -0.07% |
| Class B | 0.04% | 0.11% | 0.05% | 0.01% | 0.04% | 0.13% |
| Class C | -0.12% | 0.22% | 0.06% | -0.09% | 0.20% | 0.22% |
| Class D | -0.02% | 0.46% | 0.05% | -0.02% | 0.44% | 0.10% |
| Class E |  |  |  |  |  |  |
| Class F | -5.17% | -3.31% | -3.48% | -5.25% | -3.55% | -3.40% |
| Overall | -1.00% | -0.53% | -0.67% | -1.02% | -0.59% | -0.57% |
|  | -1.00% | -0.50% | -0.68% | -1.01% | -0.58% | -0.60% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | 0.20% | 0.05% | 0.05% | 0.39% | -0.13% |
| Class C | 0.00% | 0.22% | 0.28% | 0.01% | -0.08% | 0.31% |
| Class D | 0.01% | 0.13% | 0.35% | 0.07% | 0.27% | -0.41% |
| Class E | -0.06% | -0.16% | 0.31% | -0.03% | -0.18% | -1.54% |
| Class F | -2.99% | -2.26% | -1.80% | -2.82% | -3.04% | -3.07% |
| Overall | -0.60% | -0.36% | -0.17% | -0.54% | -0.50% | -0.90% |
|  | -0.60% | -0.44% | -0.18% | -0.54% | -0.59% | -0.90% |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.27% | -0.14% | 0.04% | 0.11% | 0.29% |
| Class C | -0.02% | -0.08% | 0.26% | 0.01% | -0.01% | 0.05% |
| Class D | 0.10% | -0.02% | 0.11% | 0.02% | 0.37% | -0.24% |
| Class E | -0.01% | -1.06% | 0.75% | -0.13% | -0.27% | -2.26% |
| Class F | -2.85% | -2.26% | -1.73% | -2.92% | -3.12% | -3.31% |
| Overall | -0.56% | -0.56% | -0.19% | -0.59% | -0.57% | -0.97% |
|  | -0.56% | -0.58% | -0.19% | -0.59% | -0.61% | -1.01% |

Table 14 enumerates BD-rate changes corresponding with the proposed intra transform skipping with implicit chroma. The reference is HM7.0 with transform skipping enabled in fast encoder mode (Table 13).

Table 14

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | -0.05% | 0.26% | -0.03% | 0.51% | -0.22% |
| Class B | -0.03% | -0.06% | -0.04% | -0.01% | -0.10% | -0.20% |
| Class C | -0.03% | 0.07% | 0.04% | -0.01% | -0.08% | 0.15% |
| Class D | -0.01% | -0.18% | 0.12% | -0.03% | -0.21% | 0.09% |
| Class E |  |  |  |  |  |  |
| Class F | -0.03% | -0.02% | -0.11% | -0.11% | -0.23% | -0.41% |
| Overall | -0.02% | -0.05% | 0.05% | -0.04% | -0.03% | -0.12% |
|  | -0.02% | -0.06% | 0.06% | -0.04% | -0.05% | -0.09% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.02% | -0.07% | 0.02% | -0.03% | -0.36% | 0.11% |
| Class C | -0.04% | -0.12% | -0.33% | -0.05% | -0.16% | -0.18% |
| Class D | -0.01% | 0.20% | -0.56% | 0.01% | 0.43% | 0.21% |
| Class E | -0.02% | -0.36% | -0.40% | -0.06% | 0.69% | 1.02% |
| Class F | 0.07% | -0.29% | -0.33% | 0.00% | 0.01% | 1.11% |
| Overall | 0.00% | -0.11% | -0.30% | -0.03% | 0.07% | 0.41% |
|  | 0.00% | -0.07% | -0.30% | -0.03% | 0.03% | 0.38% |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.04% | 0.00% | -0.21% | -0.17% |
| Class C | -0.01% | -0.07% | -0.40% | 0.01% | -0.16% | -0.01% |
| Class D | -0.05% | 0.34% | 0.06% | -0.01% | 0.02% | -0.06% |
| Class E | -0.03% | 0.23% | -0.27% | 0.22% | 1.03% | 0.46% |
| Class F | 0.06% | 0.28% | -0.39% | 0.04% | 0.02% | 0.31% |
| Overall | -0.01% | 0.14% | -0.18% | 0.04% | 0.08% | 0.07% |
|  | 0.00% | 0.12% | -0.22% | 0.04% | 0.09% | 0.06% |

The BD-rate results in Table 15 are an extension of Table 12.  Table 15 compares the proposed implicit chroma intra transform skipping with HM7.0 intra transform skipping in normal encoder mode.

Table 15

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.03% | 0.14% | 0.29% | -0.01% | 0.63% | 0.13% |
| Class B | -0.02% | 0.10% | 0.40% | -0.03% | 0.15% | 0.09% |
| Class C | -0.04% | 0.64% | 0.49% | 0.00% | 0.50% | 0.51% |
| Class D | -0.07% | 0.27% | 0.56% | -0.03% | 0.25% | 0.27% |
| Class E |  |  |  |  |  |  |
| Class F | 0.32% | 3.28% | 3.65% | 0.25% | 2.74% | 2.97% |
| Overall | 0.04% | 0.85% | 1.04% | 0.03% | 0.82% | 0.76% |
|  | 0.04% | 0.85% | 1.09% | 0.03% | 0.79% | 0.76% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.08% | -0.04% | -0.01% | 0.11% | -0.07% |
| Class C | -0.04% | 0.12% | -0.25% | -0.05% | -0.16% | 0.13% |
| Class D | -0.07% | 0.09% | -0.29% | -0.01% | 0.72% | -0.08% |
| Class E | 0.03% | -0.96% | 0.13% | 0.05% | 0.24% | -0.43% |
| Class F | 0.43% | 1.37% | 1.46% | 0.48% | 1.27% | 2.92% |
| Overall | 0.06% | 0.19% | 0.19% | 0.09% | 0.43% | 0.51% |
|  | 0.06% | 0.20% | 0.17% | 0.09% | 0.42% | 0.55% |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.21% | 0.00% | 0.08% | -0.31% |
| Class C | -0.05% | 0.05% | -0.38% | 0.03% | -0.11% | 0.18% |
| Class D | -0.04% | -0.06% | -0.17% | -0.04% | 0.31% | -0.92% |
| Class E | -0.03% | -0.98% | -0.20% | 0.13% | 0.61% | -0.78% |
| Class F | 0.42% | 1.65% | 1.49% | 0.36% | 1.18% | 1.74% |
| Overall | 0.06% | 0.24% | 0.21% | 0.09% | 0.39% | 0.01% |
|  | 0.06% | 0.23% | 0.17% | 0.09% | 0.37% | 0.08% |

# Conclusions

In part 1, it was demonstrated that the BD-rate changes corresponding with the proposed SCU restriction for intra transform skipping are negligible compared with the total BD-rate gains achieved with intra transform skipping, while significantly speeding up the encoder under All Intra test conditions (8% for normal and 4% for fast encoder modes) and eliminating needless signalling of flags for larger CU sizes. Therefore, it is proposed to adopt the SCU restriction for intra transform skipping.

In part 2, it is demonstrated that for the proposed intra transform skipping with implicit chroma the overall coding loss is negligible and the coding loss for class F sequences is acceptable compared with the total gain of intra transform skipping, while eliminating the signalling of chroma transform_skip_flags and contexts. Therefore, it is recommended to adopt the proposed intra transform skipping with implicit chroma into HEVC. The proposed method additionally simplifies the HEVC draft text by removing chroma contexts.

# Committee Draft Text Changes

## Part 1: Intra Transform Skipping Restricted to SCU

The residual coding syntax changes are highlighted in Table 1 and are based on [2].

## Part 2: Intra Transform Skipping With Implicit Chroma

The residual coding syntax changes are highlighted in Table 8 and are based on [2].

Changes to transform_skip_flag semantics:

“transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated luma transform block or not: The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1, specifies that no transform will be applied to the current luma transform block. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.”

Changes to the “8.6.2 Scaling and transformation process”:

“Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP’C	b		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP’C	r		(8-3)

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, (nW)x(nH) array r is set equal to (nW)x(nH) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the width of the transform unit nW, the height of the transform unit nH, the (nW)x(nH) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ], the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nW)x(nH) array d.

- The transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the width of the transform unit nW, the height of the transform unit nH, the scaled transform coefficient (nW)x(nH) array d, the transform skip flag transform_skip_flag[ xT ][ yT ][ cIdx ] the transSkipFlag and the chroma component variable cIdx as the inputs and the output is a residual samples (nW)x(nH) array r. The value of the transSkipFlag is derived as follows:

  - If cIdx is equal to 0, then  transSkipFlag is set equal to transform_skip_flag[ xT ][ yT ].

  - Otherwise, if cIdx is equal to 1 or 2, then transSkipFlag is set equal to 1 if the transform_skip_flag[ xT ][ yT ] is equal to 1 and transform_skip_flag[ xT+4 ][ yT ] is equal to 1 and transform_skip_flag[ xT ][ yT+4 ] is equal to 1 and transform_skip_flag[ xT+4 ][ yT+4 ] is equal to 1, otherwise, transSkipFlag is set equal to 0.”

Additional simplifications to the committee draft are the removal of the chroma contexts for the transform_skip_flag (see track changes in accompanying committee draft text).

# References

[1] C. Lan (Xidian Univ.), J. Xu, G. J. Sullivan, F. Wu (Microsoft), “Intra transform skipping,” 9th JCT-VC Meeting, Geneva, Switzerland, Apr.-May 2012, Doc. JCTVC-I0408

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” 9th JCT-VC Meeting, Geneva, Switzerland, Apr.-May 2012, Doc. JCTVC-I1003_d5

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).