| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0267_r1 |
| --- | --- |

| Title: | Non-CE1: DCT/DST adaptation for Chroma of 4x4 TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Cheung Auyeung, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal extended DST-DCT selection based on Intra mode in CE1 to Chroma of 4x4 TU. Experimental results report that BD-rate performances for Solution A are under CE1 Test 1 is -0.1% for Y, 1.2% for U and 1.1% for V, under CE1 Test 2 is -0.1% for Y, -0.3% for U and -0.3% for V, and under CE1 Test 3 is -0.1% for Y, 1.1% for U and 0.9% for V. BD-rate performances for Solution B are under CE1 Test 1 is -0.1% for Y, 0.2% for U and 0.0% for V, under CE1 Test 2 is -0.1% for Y, -0.5% for U and -0.6% for V, and under CE1 Test 3 is -0.1% for Y, 0.2% for U and -0.2% for V.

# Introduction

In HM7.0, DST and DCT are selected based on intra prediction mode for row and column transforms of luma in 4x4 TU, which has 4 types of combinations of DST and DCT transforms in 2D. In CE1 [1], simplifications of DST-DCT adaptation are tested. The simplifications are basically selecting DST and DCT on 2D level instead of 1D level in HM7.0.

## Simplification 1 in CE1

The simplification 1 in CE1 selects DCT and DST as follows:

- DST for both horizontal and vertical directions are used for all intra prediction modes of luma intra 4x4 TUs.

## Simplification 2 in CE1

The simplification 2 in CE1 selects DCT and DST as follows:

- For Intra_DC mode of luma intra 4x4 TUs, DCT for both horizontal and vertical directions are used.

- For all other prediction modes of luma intra 4x4 TUs, DST for both horizontal and vertical directions are used.

# Proposed solution

To improve the coding of Chroma, extension of mode dependent transforms for chroma of 4x4 TU is proposed.

## Solution A

- DST for both horizontal and vertical directions are used for all intra prediction modes of intra 4x4 TUs.

## Solution B

For special modes such as LM and DM in chroma, luma intra prediction mode is used. Then DST-DCT selection shares the same logic as simplification 2. The solution can be described as follows:

- For LM and DM modes of chroma, set the mode as intra prediction mode from luma.

- For Intra_DC mode, DCT for both horizontal and vertical directions are used.

- For all other prediction modes, DST for both horizontal and vertical directions are used.

# Experimental results

Proposed algorithm is implemented in HM7.0. The simulation is conducted using CE1 recommended test configurations [1].

## Solution A

### Test 1: Main, All-Intra, Common Conditions QP Range

The summary of results is shown in Table 1. Detailed results can be found in the accompanying excel file.

Table 1 Summary of results for Test 1of Solution A

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.8% | 0.9% |
| Class B | 0.0% | 1.2% | 1.3% |
| Class C | -0.1% | 1.0% | 1.1% |
| Class D | 0.0% | 1.3% | 1.3% |
| Class E | -0.2% | 2.0% | 0.9% |
| Overall | -0.1% | 1.2% | 1.1% |
|  | -0.1% | 1.2% | 1.1% |
| Class F | 0.3% | 1.2% | 1.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

### Test 2: Main, All Intra, Low QP Range: 2, 7, 12, 17

The summary of results is shown in Table 2. Detailed results can be found in the accompanying excel file.

Table 2 Summary of results for Test 2 of Solution A

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.4% | -0.4% |
| Class B | -0.1% | -0.3% | -0.3% |
| Class C | -0.1% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E | -0.1% | -0.5% | -0.6% |
| Overall | -0.1% | -0.3% | -0.3% |
|  | 0.0% | 0.0% | 0.2% |
| Class F | 0.6% | 0.3% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

### Test 3: Main, All Intra, Common Conditions QP Range, Transform Skipping ON.

The summary of results is shown in Table 3, where the anchor is HM7.0. Detailed results can be found in the accompanying excel file.

Table 3 Summary of results for Test 3 of Solution A

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.7% | 0.7% |
| Class B | 0.0% | 1.2% | 1.1% |
| Class C | -0.1% | 0.9% | 1.0% |
| Class D | 0.0% | 1.2% | 1.1% |
| Class E | -0.1% | 2.0% | 0.6% |
| Overall | -0.1% | 1.1% | 0.9% |
|  | -0.1% | 1.1% | 0.9% |
| Class F | -7.3% | -6.2% | -6.8% |
| Enc Time[%] | 129% |  |  |
| Dec Time[%] | 101% |  |  |

Compared to HM7.0 + Transform skipping ON, the results are summarized in Table 4. Detailed results can be found in accompanying excel file. Time information is not accurate.

Table 4 Summary of results compared to HM7.0+TS

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.7% | 0.9% |
| Class B | -0.1% | 1.2% | 1.3% |
| Class C | -0.1% | 1.0% | 1.2% |
| Class D | 0.0% | 1.3% | 1.3% |
| Class E | -0.2% | 2.0% | 0.9% |
| Overall | -0.1% | 1.2% | 1.1% |
|  | -0.1% | 1.1% | 1.1% |
| Class F | 0.2% | 1.3% | 1.3% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

## Solution B

### Test 1: Main, All-Intra, Common Conditions QP Range

The summary of results is shown in Table 5 Summary of results for Test 1. Detailed results can be found in the accompanying excel file.

Table 5 Summary of results for Test 1 of Solution B

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.4% | 0.3% |
| Class C | -0.2% | -0.2% | -0.3% |
| Class D | -0.1% | -0.3% | -0.5% |
| Class E | -0.2% | 1.1% | 0.0% |
| Overall | -0.1% | 0.2% | 0.0% |
|  | -0.1% | 0.2% | 0.0% |
| Class F | 0.1% | 0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

### Test 2: Main, All Intra, Low QP Range: 2, 7, 12, 17

The summary of results is shown in Table 6 Summary of results for Test 2 of Solution B. Detailed results can be found in the accompanying excel file.

Table 6 Summary of results for Test 2 of Solution B

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.5% | -0.5% |
| Class B | -0.1% | -0.5% | -0.5% |
| Class C | -0.1% | -0.4% | -0.5% |
| Class D | -0.1% | -0.5% | -0.5% |
| Class E | -0.2% | -1.1% | -1.3% |
| Overall | -0.1% | -0.5% | -0.6% |
|  | -0.1% | -0.2% | -0.1% |
| Class F | 0.3% | -0.2% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

### Test 3: Main, All Intra, Common Conditions QP Range, Transform Skipping ON.

The summary of results is shown in Table 7 Summary of results for Test 3 of Solution B. Detailed results can be found in the accompanying excel file.

Table 7 Summary of results for Test 3 of Solution B

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.4% | 0.2% |
| Class C | -0.2% | -0.3% | -0.4% |
| Class D | -0.2% | -0.2% | -0.6% |
| Class E | -0.2% | 1.1% | -0.3% |
| Overall | -0.1% | 0.2% | -0.2% |
|  | -0.1% | 0.2% | -0.2% |
| Class F | -7.5% | -7.1% | -7.8% |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 101% |  |  |

Compared to HM7.0 + Transform skipping ON, the results are summarized in. Detailed results can be found in accompanying excel file. Time information is not accurate.

Table 8 Summary results compared to HM7.0+TS

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.4% | 0.3% |
| Class C | -0.2% | -0.2% | -0.2% |
| Class D | -0.2% | -0.2% | -0.4% |
| Class E | -0.2% | 1.1% | 0.1% |
| Overall | -0.1% | 0.2% | 0.0% |
|  | -0.1% | 0.2% | 0.0% |
| Class F | 0.0% | 0.4% | 0.2% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 159% |  |  |

# Conclusion

Extension of DST-DCT adaptation is presented in this proposal. Proposed solution has a harmonized DCT/DST adaptation for 4x4 TU. Experimental results reports BD-rate gains over HM7.0 in all test cases suggested in CE1. It’s recommended to adopt the extension of DST-DCT adaptation to HEVC.

# Reference

- K. Ugur and A. Saxena, “Description of Core Experiment 1: Intra transform mode dependency simplifications”, JCTVC-I1101, Geneva, May 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

All WD text changes from the proposed solution are based on JCTVC-I1003_d4.doc and highlighted in yellow.

## Solution A

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable transSkipFlag specifying whether transform is applied to the current block,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

The variables horizTrType and vertTrType are set equal to 0.

Table 8-9 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Solution B

### 8.6.4 Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable transSkipFlag specifying whether transform is applied to the current block,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

Depending on PredMode, IntraPredMode and IntraPredModeC, the following applies:

- If  any of the following condition is true, the variable intraMode is set equal to IntraPredMode.

  - cIdx is equal to 0.

  - cIdx is not equal to 0 and IntraPredModeC is equal to Intra_FromLuma.

- Otherwise, intraMode is set to IntraPredModeC.

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and intraMode is not equal to INTRA_DC , the variables horizTrType and vertTrType are set equal to 1. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-10 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |