| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0268 |
| --- | --- |

| Title: | AHG6: on SAO signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal is a follow-up study of JCTVC-I0246 under HM7.0. The coding of SAO type is reconfigured to have separate signalling for SAO On/Off, SAO type BO and EO and EO/BO side information (classes or band positions). Grouping both context coded bins and by-pass coded bins is also proposed to improve throughput of CABAC. Using AHG5 template, it is reported that percentages of context coded bins for SAO are reduced from 3.12% to 3.11% for Main and no change for HE10 on average, percentages of by-pass coded bins for SAO are increased from 0.23% to 0.64% for Main and from 0.06% to 0.18% for HE10. Theoretical worse case analysis in AHG5 also reports that the max number of context coded bins for SAO types is reduced from 6 to 2. Experimental results report BD-rate performance as 0.0%/-0.2%/-0.3% for Y/U/V under All Intra Main, 0.0%/-0.3%/-0.4% for Y/U/V under Random Access Main, -0.1%/-0.2%/-0.3% for Y/U/V under Low Delay Main, 0.0%/-0.1%/-0.2% for Y/U/V under All intra HE10, 0.0%/-0.2%/-0.3% for Y/U/V under Random Access HE10, and 0.0%/-0.2%/-0.1% for Y/U/V under Low Delay HE10.

# Problem statement

Current SAO types (in WD 7) consist of SAO on/off, 4 Edge Offset (EO) classes, and one Band Offset (BO). These options (SAO types) are signalled to the decoder using a Unary code with two contexts (for CABAC coding), one on the first bin and one for the rest of the bins. In addition, for the BO type, band position is transmitted using FL codes. To have a more regularized structure, this design can be improved from four different aspects:

1) While there are only two actual types in SAO (namely EO and BO), the SAO types in current design consists of 4 EO classes but only 1 BO. Therefore, the extra context in CABAC mode cannot track the statistics of EO versus BO.

2) The four EO classes correspond to 4 directions but have very different code lengths. Logically, due to the symmetry in a large-enough set of images, there should not be an advantage to a direction compared to the rest. This has been confirmed even on the HEVC test set which is a much smaller test set. The numbers of occurrences of the four directions are almost uniformly distributed hence FL codes are used.

3) Side information is transmitted only in BO. Side information for the EO classes (directions), however, are coded with SAO type and on/off switch.

4) Context coded bins and by-pass coded bins are interleaved because band positions of BO are encoded in by-pass mode before offsets coded with contexts.

# Proposed solution

It’s proposed that SAO type is first signalled and side information i.e. EO classes and BO band position is signalled separately. Such a scheme has balanced structure, reduced branches and improved throughput. All the changes are itemized as follows:

- The SAO type (sao_type_idx) is reduced to indicate only SAO on/off, BO and EO as shown in Table 1. sao_type_idx is binarized using truncated unary and coded with two contexts: one for the first bin and the other for the second bin.

- EO classes are signalled using sao_eo_class as shown in Table 2, which encoded by FLC with by-pass.

- Reordering of syntax to avoid interleaving of context coded bins and by-pass coded bins as described in details in Section .

Table 1 Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - Band offset |
| - 2 | - Edge offset |

Table 2 Specification of the SAO edge offset class

| - sao_eo_class [ cIdx ][ rx ][ ry ] | - SAO edge offset class (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |

These changes can reduce the number of context coded bins for SAO syntax (including SAO type and BO/EO information) in the scenario of theoretical worst cases discussed in AHG5. The max context coded bins per syntax elements are reduced from 6 to 2.

## Interleaving issue

In the current WD7 and JCTVC-I0246, context coded bins and by-pass coded bins are interleaved. This interleaving will limit the throughput of CABAC engine. To resolve the interleaving issue, it is proposed to reorder the syntax to group each category and have context coded bin group before by-pass coded bin group. The solution can apply directly to WD7 or JCTVC-I0246.

### Resolving interleaving in WD7

To group by-pass coded bins in syntax of SAO in WD7, it is proposed to move band position after BO offsets sign if any. This change also saves one branch check for BO type. The related WD changes highlighted by yellow are shown as follows. The same technique has been discovered independently by JCTVC-J0046 [1] and JCTVC-J0054 [2].

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| } |  |

### Resolving interleaving in proposed solution

Similarly, to group by-pass coded bins in the solution proposed, syntax is reordered to avoid interleaving as follows. Complete WD changes are shown in accompanying word files.

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| else |  |
| sao_eo_class[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |

The signalling scheme of the proposed solution is visualized in Figure 1. The flowchart demonstrates that the proposed solution has balanced signalling structure for EO and BO. The context coded syntax and by-pass coded syntax are separated clearly.

Figure 1. Flowchart of proposed solution

# Experimental results

The proposed solution is implemented in HM7.0 as well as AHG5 decoder patch and tested using common test conditions [3]. Encoding and decoding time is not accurate because simulations are running in a heterogeneous cluster. Results are summarized in Table 3. Detailed results are available in accompanying excel files using AHG5 template. The AHG5 template shows that proposed solution reduced context coded bins and increased by-pass coded bins.

Table 3 Summary of results for proposed solution

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.4% | -0.5% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.5% | 0.0% | -0.3% | -0.3% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | -0.3% | -0.3% |
| Class C | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.3% |
| Class F | 0.0% | -0.1% | -0.2% | -0.1% | -0.3% | -0.3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.6% | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | -0.3% | -0.5% | 0.0% | -0.3% | -0.2% |
| Class D | 0.0% | 0.0% | 0.4% | -0.1% | -0.2% | -0.2% |
| Class E | -0.1% | -0.2% | -0.6% | 0.0% | -0.4% | 0.3% |
| Overall | -0.1% | -0.2% | -0.3% | 0.0% | -0.2% | -0.1% |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |
| Class F | -0.2% | -0.5% | -0.1% | 0.1% | -0.4% | -0.1% |

In the AHG5 summary, the changes of context coded bins and by-pass coded bins are shown in Table . It states that context coded bins are reduced and by-pass coded bins increased consistently for all configurations. “Delta rates” in positive values indicate that bins for SAO are increased because of RDO decisions in SAO.

Table 4 AHG5 summary

|  | SAO |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bins relative to total bins |  |  |  | Delta rates % |
|  | Reference |  | Tested |  |  |
|  | ctx | bp | ctx | bp |  |
| AI Main | 1.06% | 0.14% | 1.03% | 0.33% | 6.28% |
| RA Main | 3.57% | 0.18% | 3.64% | 0.51% | 4.71% |
| LB Main | 3.76% | 0.36% | 3.74% | 0.85% | 4.91% |
| LP Main | 4.10% | 0.23% | 4.04% | 0.87% | 2.54% |
| Average | 3.12% | 0.23% | 3.11% | 0.64% | 4.61% |
|  | SAO |  |  |  |  |
|  | Bins relative to total bins |  |  |  | Delta rates % |
|  | Reference |  | Tested |  |  |
|  | ctx | bp | ctx | bp |  |
| AI HE10 | 0.80% | 0.05% | 0.79% | 0.11% | 6.06% |
| RA HE10 | 2.50% | 0.06% | 2.50% | 0.16% | 3.53% |
| LB HE10 | 2.96% | 0.08% | 2.97% | 0.22% | 3.52% |
| LP HE10 | 3.03% | 0.06% | 3.04% | 0.24% | 3.14% |
| Average | 2.32% | 0.06% | 2.32% | 0.18% | 4.06% |

# Conclusion

This proposal changed SAO coding to have regularized structure for signalling SAO information, put more bins in by-pass mode and also group by-pass mode bins to improve throughput. Some BD-rate improvements are reported due to proposed solution. It is recommended to adopt proposed solution into HEVC.

# Reference

- E. Alshina, A. Alshin and J.H. Park, “AHG6: On left band position coding in SAO”, JCTVC-J0046, Stockholm, July 2012.

- J. Sole, I.S. Chong and M. Karczewics, “AHG6: Bypass bins grouping in SAO”, JCTVC-J0046, Stockholm, July 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, May 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

All WD text changes from the proposed solution are based on JCTVC-I1003_d4.doc and highlighted in yellow.

#### Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| else |  |
| sao_eo_class[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

Table 7-3 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - Band offset |
| - 2 | - Edge offset |

sao_eo_class[ cIdx ][ rx ][ ry ] indicates the edge offset type as specified in Table 7-3 of current coding tree block at position rx and ry for the colour component cIdx.

When sao_eo_class [ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_eo_class[ cIdx ][ rx ][ ry ] is set equal to sao_eo_class [ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_eo_class[ cIdx ][ rx ][ ry ] is set equal to sao_eo_class [ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_eo_class [ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-10 – Specification of the SAO edge offset class

| - sao_eo_class [ cIdx ][ rx ][ ry ] | - SAO edge offset class (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the sample range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 1.

…

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 1.

…

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 2 and i is greater than 1,offsetSign is set equal to −1.

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 2 and i is less than 2, offsetSign is set equal to 1.

...

### 8.7.2 Sample adaptive offset process

…

–	Otherwise, if saoTypeIdx is equal to 2, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Table 8-4 based on sao_eo_class[ cIdx ][ rx ][ ry ].

...

- Otherwise (saoTypeIdx is equal to 1), the following ordered steps apply:

...

Table 8-4 – Specification of hPos and vPos according to the sample adaptive offset type

| sao_eo_class[ cIdx ][ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 9-5 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 200 | 157 | 185 | 157 | 160 | 157 |

| Table 9-6 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_type_idx | 0 | TU, cMax = 2 | 1 | Table 9-5 | 0 |
|  | 1 |  | 1 | Table 9-5 | 2 |
|  | 2 |  | 1 | Table 9-5 | 4 |
| sao_eo_class | 0 | FL, cMax = 2 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |