| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0269_r1 |
| --- | --- |

| Title: | AHG6: SAO on/off flags coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal decoupled the SAO on/off from SAO type coding and encoded SAO on/off flags jointly for all color components. Experimental results state BD performance of Y/U/V is 0.1%/0.0%/-0.1% for All Intra Main, 0.1%/0.0%/-0.1% for All Intra HE10, -0.3%/-0.4%/-0.3% for Random Access Main, -0.2%/-0.2%/-0.3% for Random Access HE10, -0.3%/-0.8%/-0.9% for Lowdelay B Main, and -0.3%/-0.4%/-0.4% for Lowdelay B HE10. There is also a combined solution along with JCTVC-J0xxx. The combined solution reports BD performance of Y/U/V is 0.1%/-0.4%/-0.5% for All Intra Main, 0.1%/-0.3%/-0.4% for All Intra HE10, -0.3%/-0.7%/-0.7% for Random Access Main, -0.2%/-0.5%/-0.7% for Random Access HE10, -0.3%/-1.3%/-1.3% for Lowdelay B Main, and -0.3%/-0.7%/-0.6% for Lowdelay B HE10.

# Introduction

In HM7.0, each color component has one SAO type including on/off, EO types and BO. In the 9th meeting, JCTVC-I0193 [1] proposed to decouple the SAO on/off from SAO type and signal all on/off flags together for all color components. When combining SAO flags for luma and chroma, a combination index is calculated as:

Combination index = (lcu_sao_enable_flagY<<2) + (lcu_sao_enable_flagCb<<1) + lcu_sao_enable_flagCr,

where flag value 1 implies SAO on, and flag value 0 implies SAO off for each color component.

Then, a look-up table (Table 1) is used to get code index and the code index is binarized using Unary code with one context shared for all bins in CABAC.

Table 1 Mapping from LCU SAO enable flags to a lcu_sao_enable_idx index.

| LCU SAO enable flag |  |  | Combination index | Code index (lcu_sao_enable_idx) |
| --- | --- | --- | --- | --- |
| Y | Cb | Cr |  |  |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 5 |
| 0 | 1 | 0 | 2 | 6 |
| 0 | 1 | 1 | 3 | 7 |
| 1 | 0 | 0 | 4 | 1 |
| 1 | 0 | 1 | 5 | 2 |
| 1 | 1 | 0 | 6 | 3 |
| 1 | 1 | 1 | 7 | 4 |

The SAO type is changed to Table 2.

Table 2. SAO type table in JCTVC-I0193

| - sao_type_idx | - SAO type |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

# Proposed solution

## Solution A - Simplification of JCTVC-I0193

This proposal simplified JCTVC-I0193 solution in the combined SAO flags with the following changes:

- Remove the look-up table in Table 1 and generate code index with an equation as follows:

code index = (lcu_sao_enable_flagCr<<2) + (lcu_sao_enable_flagCb<<1) + lcu_sao_enable_flagY

- Truncated unary binarization for the code index.

- Only the first bin is context coded and other bins are encoded in by-pass mode.

The type coding in Table 2 and offset coding stay the same as JCTVC-I0193 extended to HM7.0.

Note: Truncated unary binarization can be used to replace the unary code for SAO type in either JCTVC-I0193 or HM7.0 to further remove redundancy.

## Solution B - JCTVC-J0268 + Simplification of JCTVC-I0193

The simplified solution is combined with the solution from JCTVC-J0268 on SAO signalling to have better signalling structure for SAO. There will be a small change in the SAO type due to the decoupling of SAO on/off in Solution A. The SAO signalling of the combined solution is listed in order as follows:

- SAO on/off index for three on/off flags from all color components. The index is binarized using truncated unary code. Only the first bin is context coded with one dedicated context. The rest bins are encoded in by-pass mode. For each color component, if SAO is enabled, go to step 2; Otherwise, finish SAO signalling.

- SAO type as shown in Table 3 is encoded with one context.

Table 3 Specification of the SAO type in Solution B

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Band offset |
| - 1 | - Edge offset |

- Offsets including absolute values and signs are encoded as HM7.0 does.

- If SAO type is EO, EO classes are signalled using sao_eo_class as shown in Table 4 , which encoded by FLC with by-pass.

- If SAO type is BO, band positions are signalled as HM7.0 does.

Table 4 Specification of the SAO edge offset class

| - sao_eo_class [ cIdx ][ rx ][ ry ] | - SAO edge offset class (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |

# Experimental results

The proposed solution is implemented in HM7.0 and tested using common test conditions [2]. Encoding and decoding time is not accurate because simulations are running in a heterogeneous cluster. The summarized results are shown as follows. Detailed results are available in accompanying excel files.

## Solution A

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.2% |
| Class B | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | -0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | -0.1% |
| Overall | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 166% |  |  | 161% |  |  |
| Dec Time[%] | 195% |  |  | 191% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.7% | -0.5% | 0.0% | -0.2% | -0.4% |
| Class B | -0.1% | -0.4% | -0.5% | -0.1% | -0.3% | -0.3% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | -0.5% | -0.4% | -0.2% | -0.4% | -0.3% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.3% |
|  | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.3% |
| Class F | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 110% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.0% | -1.1% | -0.1% | -0.6% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.1% | -0.3% | -0.1% |
| Class D | -0.3% | 0.0% | 0.3% | -0.2% | 0.1% | -0.1% |
| Class E | -0.6% | -2.2% | -2.5% | -0.7% | -0.8% | -0.7% |
| Overall | -0.3% | -0.8% | -0.9% | -0.3% | -0.4% | -0.4% |
|  | -0.3% | -0.7% | -0.8% | -0.3% | -0.4% | -0.3% |
| Class F | -0.3% | -0.9% | -0.6% | -0.1% | -0.7% | -0.3% |
| Enc Time[%] | 94% |  |  | 95% |  |  |
| Dec Time[%] | 113% |  |  | 101% |  |  |

## Solution B

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.4% | -0.5% | 0.1% | -0.3% | -0.4% |
| Class B | 0.1% | -0.3% | -0.5% | 0.1% | -0.3% | -0.5% |
| Class C | 0.1% | -0.1% | -0.2% | 0.1% | -0.2% | -0.3% |
| Class D | 0.1% | -0.3% | -0.4% | 0.0% | -0.1% | -0.2% |
| Class E | 0.1% | -0.7% | -0.9% | 0.1% | -0.4% | -0.6% |
| Overall | 0.1% | -0.4% | -0.5% | 0.1% | -0.3% | -0.4% |
|  | 0.1% | -0.3% | -0.5% | 0.1% | -0.3% | -0.4% |
| Class F | 0.1% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 164% |  |  | #NUM! |  |  |
| Dec Time[%] | 183% |  |  | 521% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -1.2% | -1.1% | -0.1% | -0.6% | -0.8% |
| Class B | -0.1% | -0.7% | -0.8% | -0.1% | -0.5% | -0.7% |
| Class C | -0.3% | -0.4% | -0.5% | -0.2% | -0.3% | -0.5% |
| Class D | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.7% | -0.7% | -0.2% | -0.5% | -0.7% |
|  | -0.3% | -0.7% | -0.7% | -0.2% | -0.5% | -0.7% |
| Class F | -0.2% | -0.3% | -0.5% | -0.3% | -0.5% | -0.8% |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 106% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.5% | -1.6% | -0.2% | -0.7% | -1.1% |
| Class C | -0.2% | -0.5% | -0.7% | -0.2% | -0.7% | -0.4% |
| Class D | -0.4% | -0.3% | 0.1% | -0.3% | -0.2% | -0.1% |
| Class E | -0.6% | -3.0% | -3.2% | -0.7% | -1.1% | -0.6% |
| Overall | -0.3% | -1.3% | -1.3% | -0.3% | -0.7% | -0.6% |
|  | -0.3% | -1.2% | -1.2% | -0.3% | -0.6% | -0.5% |
| Class F | -0.3% | -1.5% | -0.6% | -0.2% | -0.6% | 0.2% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 115% |  |  | 98% |  |  |

# Conclusion

This proposal changed SAO coding to have regularized structure for signalling SAO information. Some BD-rate improvements are reported due to proposed solution. It is recommended to adopt proposed Solution B into HEVC.

# Reference

- W.K, Kim, D.K. Kwon, I.S. Chong and M. Karczewics, “Non-CE1: LCU SAO Enable Flag Coding”, JCTVC-I0193, Geneva, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, May 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

All WD text changes from the proposed solution are based on JCTVC-I1003_d4.doc and highlighted in yellow.

## Solution A

### 7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| if( slice_sample_adaptive_offset_flag[ 0 ]) |  |
| lcu_sao_enable_idx | ae(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] &&LcuSaoEnableFlag[ 0 ][ xCtb ][ yCtb ]) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

#### 7.3.4.1 Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile&& LcuSaoEnableFlag[ cIdx ][ rx - 1][ ry ] ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile&& LcuSaoEnableFlag[ cIdx ][ rx - 1][ ry ] ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag&& LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =4 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 7-5 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |
| - 4 | - Band offset |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 4.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 4.

…

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 4 and i is larger than 1,offsetSign is set to −1.

- Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 4 or i is less than 2), offsetSign is set to 1.

### 7.4.4 Slice data semantics

lcu_sao_enable_idx indicates the LcuSaoEnableFlag values of three color components as specified in Table 7-x.

Table 7-x – Specification of the LcuSaoEnableFlag values of three color components

| - lcu_sao_enable_idx | - LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] |  |  |
| --- | --- | --- | --- |
|  | - cIdx = 0 | - cIdx = 1 | - cIdx = 2 |
| - 0 | - 0 | - 0 | - 0 |
| - 1 | - 1 | - 0 | - 0 |
| - 2 | - 0 | - 1 | - 0 |
| - 3 | - 1 | - 1 | - 0 |
| - 4 | - 0 | - 0 | - 1 |
| - 5 | - 1 | - 0 | - 1 |
| - 6 | - 0 | - 1 | - 1 |
| - 7 | - 1 | - 1 | - 1 |

##### 8.7.2.1 Coding tree block modification process

…

If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] equals 0.

- Otherwise, if saoTypeIdx is equal to one of the values of 0, 1, 2, or 3, the following ordered steps apply:

…

Table 8-6 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ] [ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |

## Solution B

### 7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| if( slice_sample_adaptive_offset_flag[ 0 ]) |  |
| lcu_sao_enable_idx | ae(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] &&LcuSaoEnableFlag[ 0 ][ xCtb ][ yCtb ]) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

#### 7.3.4.1 Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile&& LcuSaoEnableFlag[ cIdx ][ rx - 1][ ry ] ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile&& LcuSaoEnableFlag[ cIdx ][ rx - 1][ ry ] ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag&& LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 0 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| else |  |
| sao_eo_class[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |

Table 7-7 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Band offset |
| - 1 | - Edge offset |

sao_eo_class[ cIdx ][ rx ][ ry ] indicates the edge offset type as specified in Table 7-5 of current coding tree block at position rx and ry for the colour component cIdx.

When sao_eo_class [ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_eo_class[ cIdx ][ rx ][ ry ] is set equal to sao_eo_class [ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_eo_class[ cIdx ][ rx ][ ry ] is set equal to sao_eo_class [ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_eo_class [ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-10 – Specification of the SAO edge offset class

| - sao_eo_class [ cIdx ][ rx ][ ry ] | - SAO edge offset class (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 0.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 0.

…

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 1 and i is greater than 1,offsetSign is set equal to −1.

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 1 and i is less than 2, offsetSign is set equal to 1.

### 7.4.4 Slice data semantics

lcu_sao_enable_idx indicates the LcuSaoEnableFlag values of three color components as specified in Table 7-x.

Table 7-x – Specification of the LcuSaoEnableFlag values of three color components

| - lcu_sao_enable_idx | - LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] |  |  |
| --- | --- | --- | --- |
|  | - cIdx = 0 | - cIdx = 1 | - cIdx = 2 |
| - 0 | - 0 | - 0 | - 0 |
| - 1 | - 1 | - 0 | - 0 |
| - 2 | - 0 | - 1 | - 0 |
| - 3 | - 1 | - 1 | - 0 |
| - 4 | - 0 | - 0 | - 1 |
| - 5 | - 1 | - 0 | - 1 |
| - 6 | - 0 | - 1 | - 1 |
| - 7 | - 1 | - 1 | - 1 |

##### 8.7.2.1 Coding tree block modification process

…

If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] equals 0.

–	Otherwise, if saoTypeIdx is equal to 1, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Table 8-6 based on sao_eo_class[ cIdx ][ rx ][ ry ].

...

- Otherwise (saoTypeIdx is equal to 0), the following ordered steps apply:

...

Table 8-8 – Specification of hPos and vPos according to the sample adaptive offset type

| sao_eo_class[ cIdx ][ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 9-9 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  |  |  |
| initValue | 157 | 157 | 157 |  |  |  |

| Table 9-10 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_type_idx | 0 | U, cMax = 1 | 0 | Table 9-9 | 0 |
|  | 1 |  | 0 | Table 9-9 | 1 |
|  | 2 |  | 0 | Table 9-9 | 2 |
| sao_eo_class | 0 | FL, cMax = 2 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |