| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11–20 July 2012 | Document: JCTVC-J0270 WG11 Number: m25598 |
| --- | --- |

| Title: | HEVC VUI Parameters with Extension Hooks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This contribution presents updates of the current HEVC VUI Parameters Syntax structure to consider the Temporal Scalability extension. These parameters can also be extended to meet future HEVC extensions.

- On HEVC VUI Syntax Structure

The VUI syntax structure in the current Committee Draft [2] does not address certain data elements for the Temporal Scalability extension.  In our contribution, the existing VUI syntax structure is extended to include such new VUI syntax parameters needed for the Temporal Scalability in the base version of HEVC and also it opens the possibility of  newer VUI parameters structure for future HEVC extensions.

It may be noted that most of the AVC VUI syntax parameters are borrowed (JCTVC-F289) to create the HEVC VUI syntax structure in the current CD. In the same breadth, the VUI extension syntax structures  for SVC and MVC are carefully studied [1] and as HEVC baseline specification covers Temporal Scalability, some of these SVC/MVC VUI extension parameters are also applicable for HEVC VUI.

## HEVC VUI Parameters Syntax Structure

The following VUI syntax structure is available in [2], but there is a missing bracket "{" which is corrected below at the location highlighted in red ink. If we keep this editing error resolved as shown below, all the yellow highlighted areas below are re-addressed for the current contribution to include VUI syntax parameters applicable for Temporal Scalability as well as other HEVC Scalability extensions in the future.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) { // ed - missing ? |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

## HEVC VUI Parameters Syntax Structure with Extension Hooks:Option-1

In option-1 of this syntax structure shown below, the field_seq_flag and all syntaxes related to timing_info_present_flag are kept separate for each temporal layer. The yellow highlighted part of the VUI syntax structure shown in the previous section can be replaced by the following syntax structure segment:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ..... edited out .... | ....... |
| vui_num_temporal_layers_minus1 | u(3) |
| for (i=0; i<=vui_num_temporal_layers_minus1; i++ ) { // iterative loop starts |  |
| vui_temporal_id[i] | u(3) |
| field_seq_flag[i] | u(1) |
| timing_info_present_flag[i] | u(1) |
| if( timing_info_present_flag[i] ) { |  |
| num_units_in_tick[i] | u(32) |
| time_scale[i] | u(32) |
| fixed_pic_rate_flag[i] | u(1) |
| } |  |
| nal_hrd_parameters_present_flag[i] | u(1) |
| if( nal_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag[i] | u(1) |
| if( vcl_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag[i] ¦ ¦ vcl_hrd_parameters_present_flag[i] ) { // ed - missing ? |  |
| low_delay_hrd_flag[i] | u(1) |
| sub_pic_cpb_params_present_flag[i] | u(1) |
| if( sub_pic_cpb_params_present_flag[i] ) |  |
| num_units_in_sub_tick[i] | u(32) |
| } |  |
| } // iterative loop ends for temporal layers |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| vui_extension_flag | u(1) |
| if( vui_extension_flag ) { |  |
| while( more_rbsp_data() ) |  |
| vui_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

As highlighted bordered by green regions above, all such VUI parameters above are needed to deal with various temporal layers in case of Temporal Scalability for HEVC VUI.

## HEVC VUI Parameters Syntax Structure with Extension Hooks:Option-2

In option-2 of this syntax structure shown below, the field_seq_flag and all syntaxes related to timing_info_present_flag (sky-blue part of the syntax structure below) are shared among all temporal layers, and thus kept fixed for all temporal layer (outside the iterative loop). The yellow highlighted part of the VUI syntax structure shown in an earlier section (1.1) can be replaced by the following syntax structure segment:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ..... edited out .... | ....... |
| field_seq_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| vui_num_temporal_layers_minus1 | u(3) |
| for (i=0; i<=vui_num_temporal_layers_minus1; i++ ) { // iterative loop starts |  |
| vui_temporal_id[i] | u(3) |
| nal_hrd_parameters_present_flag[i] | u(1) |
| if( nal_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag[i] | u(1) |
| if( vcl_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag[i] ¦ ¦ vcl_hrd_parameters_present_flag[i] ) { // ed - missing ? |  |
| low_delay_hrd_flag[i] | u(1) |
| sub_pic_cpb_params_present_flag[i] | u(1) |
| if( sub_pic_cpb_params_present_flag[i] ) |  |
| num_units_in_sub_tick[i] | u(32) |
| } |  |
| } // iterative loop ends for temporal layers |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| vui_extension_flag | u(1) |
| if( vui_extension_flag ) { |  |
| while( more_rbsp_data() ) |  |
| vui_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

As highlighted bordered by green regions above, all such VUI parameters above are needed to deal with various temporal layers in case of Temporal Scalability for HEVC VUI.

# Conclusion

In this contribution, the HEVC VUI Syntax Structure is enhanced to deal with coded video contents for HEVC Version-1 Temporal Scalability layers and also for the future HEVC Extensions in Scalability and Multi-view coding areas.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Haque, A. Tabatabai, “Extension of HEVC VUI Syntax Structure”, JCTVC-I0263, April-May 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003_d4, Geneva, Switzerland, April-May 2012.