| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11–20 July 2012 | Document: JCTVC-J0272 WG11 Number: m25600 |
| --- | --- |

| Title: | Simplifications of HRD parameters for Temporal Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This document considers the HEVC VUI Parameters Syntax structure with extension hooks for Temporal Scalability. In this modified VUI syntax structure, there are two HRD parameters syntax function calls that exist for each temporal layer.  For multiple temporal layers, such HRD parameters syntax could have redundant parameters. So this is analyzed to find out some common and shared parameters among them so that the size of the overall VUI parameters syntax structure can be reduced.

- HEVC VUI Parameters Syntax Structure with Extension Hooks

In [1], the VUI parameters syntax structure in the current Committee Draft [2] is extended to  address the HEVC extensions of Scalable Video Coding, especially Temporal Scalability for version-1. The corresponding modified syntax structure is shown below.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ..... edited out .... | ....... |
| vui_num_temporal_layers_minus1 | u(3) |
| for (i=0; i<=vui_num_temporal_layers_minus1; i++ ) { // iterative loop starts |  |
| vui_temporal_id[i] | u(3) |
| field_seq_flag[i] | u(1) |
| timing_info_present_flag[i] | u(1) |
| if( timing_info_present_flag[i] ) { |  |
| num_units_in_tick[i] | u(32) |
| time_scale[i] | u(32) |
| fixed_pic_rate_flag[i] | u(1) |
| } |  |
| nal_hrd_parameters_present_flag[i] | u(1) |
| if( nal_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag[i] | u(1) |
| if( vcl_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag[i] ¦ ¦ vcl_hrd_parameters_present_flag[i] ) { // ed - missing ? |  |
| low_delay_hrd_flag[i] | u(1) |
| sub_pic_cpb_params_present_flag[i] | u(1) |
| if( sub_pic_cpb_params_present_flag[i] ) |  |
| num_units_in_sub_tick[i] | u(32) |
| } |  |
| } // iterative loop ends for temporal layers |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| vui_extension_flag | u(1) |
| if( vui_extension_flag ) { |  |
| while( more_rbsp_data() ) |  |
| vui_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

In the above VUI syntax structure, for the iterative loop related to various temporal layers, the function "hrd_parameters()" is called twice (VCL and NAL cases) per iteration or temporal layer. In the following sub-section, this function "hrd_parameters()" is analyzed to find whether some common syntax parameters can be taken outside this iterative loop to reduce the total number of VUI parameters. This is to reduce some parameter redundancies from one temporal layer to next layer, etc.

## HRD Parameters Syntax

| hrd_parameters( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |

The HDR parameters in the syntax structure above are analyzed below to find possible common parameters that can be considered outside the iterative loop, as the same syntax structure being repeated for various temporal layers (temporal scalability).

i) cpb_cnt_minus1, a variable length parameter, specifies number of alternative delivery schedules where each SchedSelIdx (delivery Schedules Selected Idx for Hypothetical Stream Scheduler or HSS) has restricted bit rate and CPB size values. So this parameter cannot be considered as a constant parameter for all temporal layers..

ii) bit_rate_scale and cpb_size_scale: These parameters have fixed-sizes (4-bits each), and these are used in the following equations:

a) BitRate[ SchedSelIdx ] = ( bit_rate_value_minus1[ SchedSelIdx ] + 1 ) * 2(6 + bit_rate_scale)

b) CpbSize[ SchedSelIdx ] = ( cpb_size_value_minus1[ SchedSelIdx ] + 1 ) * 2(4 + cpb_size_scale)

These two parameters can be considered constant values for all temporal layers, as they are used as the power-factors, etc.

iii) initial_cpb_removal_delay_length_minus1, cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1 and time_offset_length: These parameters have fixed-sizes (5-bits each). In the Semantics definition, the following description is noted -

"When the '*'_delay_length_minus1 (or, time_offset_length)  syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the '*'_delay_length_minus1 (or, time_offset_length)  parameters shall be equal in both hrd_parameters( ) syntax structures. When the '*'_delay_length_minus1 (or, time_offset_length)  syntax element is not present, it is inferred to be equal to 23 (24 for time_offset_length)."

Here "both hrd_parameters" refer to NAL and VCL HRD parameters.

Given the above semantics definition, we may consider all these four syntax parameters to remain constant for all temporal-layers (similar buffer properties for all layers).

So the above HRD parameters syntax structure can be split up into two parts:

a) hrd_parameters_fixed() - it will be the same for all temporal layers with constant parameters

| hrd_parameters_fixed( ) { | Descriptor |
| --- | --- |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |

There could be 2 options for this hrd_parameters_fixed() structure:

Option-1: A single hrd_parameters_fixed() structure for both NAL and VCL HRD cases

Option-2: Two separate hrd_parameters_fixed() structures for NAL and VCL HRD cases -

hrd_parameters_fixed_nal() and  hrd_parameters_fixed_vcl(). Both these syntax structures contain identical syntax parameters as in hrd_parameters_fixed(), but with different values perhaps.

b) hrd_parameters_var() - it will be changing according to each temporal layer requirements.

| hrd_parameters_var( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| } |  |

## Modified HEVC VUI Parameters Syntax Structure with Extension Hooks

Option-1: A single hrd_parameters_fixed( ) structure for both NAL and VCL HRDs.

The final version of the modified VUI syntax structure with extension for Temporal Scalability is shown below.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ..... edited out .... | ....... |
| vui_num_temporal_layers_minus1 | u(3) |
| hrd_parameters_fixed( ) // Fixed HRD parameters for all temporal layers |  |
| for (i=0; i<=vui_num_temporal_layers_minus1; i++ ) { // iterative loop starts |  |
| vui_temporal_id[i] | u(3) |
| field_seq_flag[i] | u(1) |
| timing_info_present_flag[i] | u(1) |
| if( timing_info_present_flag[i] ) { |  |
| num_units_in_tick[i] | u(32) |
| time_scale[i] | u(32) |
| fixed_pic_rate_flag[i] | u(1) |
| } |  |
| nal_hrd_parameters_present_flag[i] | u(1) |
| if( nal_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters_var( ) |  |
| vcl_hrd_parameters_present_flag[i] | u(1) |
| if( vcl_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters_var( ) |  |
| if( nal_hrd_parameters_present_flag[i] ¦ ¦ vcl_hrd_parameters_present_flag[i] ) { // ed - missing ? |  |
| low_delay_hrd_flag[i] | u(1) |
| sub_pic_cpb_params_present_flag[i] | u(1) |
| if( sub_pic_cpb_params_present_flag[i] ) |  |
| num_units_in_sub_tick[i] | u(32) |
| } |  |
| } // iterative loop ends for temporal layers |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| vui_extension_flag | u(1) |
| if( vui_extension_flag ) { |  |
| while( more_rbsp_data() ) |  |
| vui_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

Option-2: Two separate hrd_parameters_fixed( ) structures for NAL and VCL HRD parameters

The final version of the modified VUI syntax structure with extension for Temporal Scalability is shown below.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ..... edited out .... | ....... |
| vui_num_temporal_layers_minus1 | u(3) |
| hrd_parameters_fixed_nal( ) // Fixed HRD parameters for all temporal layers |  |
| hrd_parameters_fixed_vcl( ) // Fixed HRD parameters for all temporal layers |  |
| for (i=0; i<=vui_num_temporal_layers_minus1; i++ ) { // iterative loop starts |  |
| vui_temporal_id[i] | u(3) |
| field_seq_flag[i] | u(1) |
| timing_info_present_flag[i] | u(1) |
| if( timing_info_present_flag[i] ) { |  |
| num_units_in_tick[i] | u(32) |
| time_scale[i] | u(32) |
| fixed_pic_rate_flag[i] | u(1) |
| } |  |
| nal_hrd_parameters_present_flag[i] | u(1) |
| if( nal_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters_var( ) |  |
| vcl_hrd_parameters_present_flag[i] | u(1) |
| if( vcl_hrd_parameters_present_flag[i] ) |  |
| hrd_parameters_var( ) |  |
| if( nal_hrd_parameters_present_flag[i] ¦ ¦ vcl_hrd_parameters_present_flag[i] ) { // ed - missing ? |  |
| low_delay_hrd_flag[i] | u(1) |
| sub_pic_cpb_params_present_flag[i] | u(1) |
| if( sub_pic_cpb_params_present_flag[i] ) |  |
| num_units_in_sub_tick[i] | u(32) |
| } |  |
| } // iterative loop ends for temporal layers |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| vui_extension_flag | u(1) |
| if( vui_extension_flag ) { |  |
| while( more_rbsp_data() ) |  |
| vui_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

# Conclusion

In this contribution, the HEVC VUI Syntax Structure with Extension hooks is simplified by removing some of the redundant parameters from the iterative loop of temporal layers. This enables a more compact VUI syntax structure in terms of the total VUI parameters size.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Haque, et.al., “HEVC VUI Parameters with Extension Hooks”, JCTVC-J0270, July 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003_d4, Geneva, Switzerland, April-May 2012.