| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11–20 July 2012 | Document: JCTVC-J0273 WG11 Number: m25601 |
| --- | --- |

| Title: | A simple ordering issue for VUI parameters syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This document shows an ordering issue on some HEVC VUI Parameters in the corresponding Syntax structure and a possible fix for this ordering problem.

- HEVC VUI Syntax Structure

An edited version of the VUI syntax structure in the current Committee Draft [1] is shown below.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ... edited out ... |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) { // Is this bracket missing? |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| ... edited out ... |  |
| } |  |

In the above syntax flow, there is possibly an editing problem with a missing bracket "{" in the line

" if( nal_hrd_parameters_present_flag  | |  vcl_hrd_parameters_present_flag )", or the 2nd last bracket "}" is wrong.

Besides the above editing issue, there is also an "ordering problem" in the placement of the  syntax parameter, low_delay_hrd_flag. This flag is present in the syntax flow order after 2 probable calls to "hrd_parameters()" syntax structure which is shown below:

| hrd_parameters( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |

The value of low_delay_hrd_flag parameter of "vui_parameters()" influences the syntax element cpb_cnt_minus1 in "hrd_parameters()" syntax structure, where the semantics definition of cpb_cnt_minus1 is as follows: "When low_delay_hrd_flag is equal to 1, cpb_cnt_minus1 shall be equal to 0".

However, as indicated above, the placement of low_delay_hrd_flag happens after the "hrd_parameters()" syntax structure calls inside the "vui_parameters()" syntax structure. In AVC specification also, the same ordering issue is noted. To fix this issue, it is more logical to have low_delay_hrd_flag available in the "vui_parameters()" syntax flow before calls to "hrd_parameters()" are made, as follows:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ... edited out ... |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) { // Is this bracket missing? |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| ... edited out ... |  |
| } |  |

# Conclusion

In this contribution, the HEVC VUI parameters Syntax Structure is studied with two possible fixes, one related to an editing issue, and the other is related to the placement of some syntax parameters in the VUI parameters Syntax flow. Both such fixes are found to be obviously simple ones.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003_d4, Geneva, Switzerland, April-May 2012.