| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0274 |
| --- | --- |

| Title: | Parity Inference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Wang Dake He Xiang Yu Gaelle Martin-Cocher | Tel: Email: | +1-519-597-5923 {wangj, dhe, xiayu, gmartincocher@rim.com} |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This proposal describes a technique to infer the parity of the first quantized coefficient in a coefficient group. In common test conditions, experimental results based on HM7.0 reference software show that the average Luma BD-rates are (-0.28%, -0.31%, -0.30%, -0.58%, -0.41%, -0.31%) in (AI-Main, RA-Main, LD-Main, AI-HE10, RA-HE10, LD-HE10) settings with RDOQ off, and (-0.22%, -0.10%, -0.10%, -0.62%, -0.50%, -0.65%) with RDOQ on. Note that the results include rotation of residual block in transform skipping as detailed in JCTVC-J0093, which affects HE10 test conditions. The method also reduces the maximum number of context-coded bins in a coefficient group from 25 to 24.

# Introduction

In HM7.0, a technique called sign bit hiding (SBH) [1], [2] was used to infer the sign bit of the first nonzero coefficient in a coefficient group under certain conditions. Specifically, the sign information is inferred from the parity of the sum of the quantized coefficients in a coefficient group (CG).

In this proposal, a technique to infer the parity of the first quantized coefficient in a CG is proposed. The proposed method provides coding gain over all test settings under common test conditions, and also reduces the maximum number of context-coded bins in a coefficient group from 25 to 24.

# Proposed method

## Inference Criterion

For each CG, the following criterion is tested to determine whether parity inference is applied or not, depending on numNonZero, which is the number of non-zero coefficients in the CG excluding the first coefficient, and lastNZPos, which is the position of the last significant coefficient in the scan order.

if (the current CG is the last significant CG)

parityHidden = lastNZPos >= 4

else

parityHidden = numNonZero >= 1

If parityHidden is true, the first coefficient in the CG is decoded by using the proposed parity inference method described in Sec. 2.2; otherwise, parityHidden is false, and the first coefficient is decoded in the conventional way.

## Parsing with Parity Inference

When parity inference is enabled, the parity of the first quantized coefficient in a CG, x, is inferred from the parity of the sum of the absolute value (sumAbs) of all remaining quantized coefficients in that CG. The following predefined convention is used

if sumAbs is odd

The parity of x is inferred to be even

else

The parity of x is inferred to be odd

After determining the parity of the first quantized coefficient, the following decoding process is used to decode the significant coefficient flag, greater-than-one flag, greater-than-two flag, remaining level, and the sign of x:

if x is even

{

decode( |x| > 0 )        // significance flag of |x|

if(|x| > 0 )

{

decode( |x| > 2 )      // greaterTwo flag of |x|

if( |x| > 2 )

{

Decode( (|x| - 4)/2 )   // remaining level of |x|

}

Decode( sign(x) )     // sign of x

}

else

{

Decode( |x| > 1 )       // greaterOne flag of |x|

if( |x| > 1 )

{

Decode( (|x| - 3)/2 )    // remaining level of |x|

}

Decode( sign(x) )     // sign of x

}

}

## Reduction of Maximum Number of Context-coded Bins

In HM7.0, the worst-case number of context-coded bins in parsing one CG is given by

16 significant coefficient flags + 8 greater-than-one flags + 1 greater-than-two flag = 25

With the proposed parity inference method, the following changes are proposed:

- The greater-than-one flag of the first quantized coefficient in a CG is always context- encoded/decoded if its parity is not inferred

- The maximum number of coded greater-than-one flags of the remaining coefficients in the CG is limited to be 7

The proposed method reduces the worst-case number of context-coded bins from 25 to 24 as shown in the following analysis.

Case 1: When parity inference is used and the first quantized coefficient is even, the worst case is

16 significant coefficient flags + 7 greater-than-one flags + 1 greater-than-two flag = 24.

Case 2: When parity inference is used and the first quantized coefficient is odd, the worst case is

15 significant coefficient flags + 8 greater-than-one flags (1 for the upper left coefficient and a maximum of 7 in the remainder of the CG) + 1 greater-than-two flag = 24.

Case 3: When parity inference is not used under the criterion in Sec. 2.1, the total number of non-zero coefficients in the CG is no greater than two. The worst-case number is thus less than 24.

## Additional contexts for coding the significant coefficient flag

Note that when the parity is inferred and the coefficient is even, the significant coefficient flag being 1 implies the coefficient is greater than or equal to 2. As a result, the probability distribution of the values of the significant coefficient flag is different from the normal case. Therefore, the proposed method introduces four additional contexts (two for luma and two for chroma) for coding the significant coefficient flag of the first quantized coefficient of one CG when its parity is inferred and the coefficient is even. The contexts are derived based on the coefficient position (posX, posY) in the TU as follows.

If the parity of the first quantized coefficient is inferred and the coefficient is even

If posX + posY == 0

ctxInc = isLuma ? 24 : 21

Else

ctxInc = isLuma ? 25 : 22

Else

Use normal context derivation

# Experimental results

The performance of the proposed parity inference method compared against the HM7.0 anchor. Note that the results with RDOQ on include rotation of residual data [3] which affects HE10 test conditions.

Table 1 BD-rates agains HM 7.0 Anchor (RDOQ ON)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.30% | -0.26% | -0.30% | -0.48% | -1.34% | -1.37% |
| Class B | -0.22% | -0.17% | -0.25% | -0.49% | -1.39% | -1.52% |
| Class C | -0.22% | -0.32% | -0.35% | -0.78% | -1.64% | -1.99% |
| Class D | -0.30% | -0.33% | -0.47% | -0.97% | -1.95% | -2.12% |
| Class E | -0.32% | -0.18% | -0.16% | -0.59% | -1.34% | -1.45% |
| Overall | -0.22% | -0.22% | -0.28% | -0.62% | -1.52% | -1.65% |
|  | -0.22% | -0.22% | -0.26% | -0.63% | -1.50% | -1.62% |
| Class F | -0.17% | -0.20% | -0.33% | -1.61% | -2.66% | -2.66% |
| Enc Time[%] | 116% |  |  | 113% |  |  |
| Dec Time[%] | 110% |  |  | 105% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.12% | 0.08% | -0.10% | -0.42% | -0.82% | -1.20% |
| Class B | -0.13% | 0.32% | -0.09% | -0.44% | -1.23% | -1.34% |
| Class C | -0.12% | 0.17% | -0.11% | -0.62% | -1.21% | -1.74% |
| Class D | -0.06% | 0.32% | -0.18% | -0.71% | -1.46% | -2.01% |
| Class E |  |  |  |  |  |  |
| Overall | -0.10% | 0.14% | -0.08% | -0.50% | -1.32% | -1.72% |
|  | -0.09% | 0.12% | -0.13% | -0.58% | -1.34% | -1.75% |
| Class F | -0.02% | 0.03% | -0.01% | -1.20% | -1.58% | -2.18% |
| Enc Time[%] | 108% |  |  | 109% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.90% | 0.42% | -0.66% | -0.35% | -0.89% |
| Class C | -0.06% | 0.42% | 0.49% | -0.71% | -0.59% | -0.87% |
| Class D | -0.01% | 1.24% | 0.78% | -0.63% | -0.37% | -0.91% |
| Class E | -0.30% | -0.75% | -0.20% | -0.59% | -1.23% | -0.18% |
| Overall | -0.10% | 0.56% | 0.41% | -0.65% | -0.58% | -0.76% |
|  | -0.10% | 0.53% | 0.32% | -0.67% | -0.45% | -0.62% |
| Class F | -0.21% | -0.01% | 0.72% | -0.90% | -0.96% | -0.45% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

The performance of the proposed parity inference method compared against the HM7.0 anchor with RDOQ off.

Table 2 BD-rates agains HM 7.0 Anchor (RDOQ OFF)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.41% | -0.30% | -0.28% | -0.55% | -0.45% | -0.41% |
| Class B | -0.32% | -0.08% | -0.08% | -0.44% | -0.26% | -0.34% |
| Class C | -0.29% | -0.26% | -0.24% | -0.73% | -0.96% | -1.10% |
| Class D | -0.31% | -0.30% | -0.27% | -0.91% | -1.21% | -1.39% |
| Class E | -0.34% | -0.11% | -0.21% | -0.62% | -0.48% | -0.52% |
| Overall | -0.28% | -0.18% | -0.19% | -0.58% | -0.59% | -0.70% |
|  | -0.28% | -0.18% | -0.19% | -0.58% | -0.58% | -0.69% |
| Class F | -0.15% | -0.05% | -0.06% | -1.32% | -1.75% | -1.67% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.33% | -0.29% | -0.15% | -0.36% | -0.12% | -0.37% |
| Class B | -0.34% | 0.01% | 0.01% | -0.37% | -0.28% | -0.08% |
| Class C | -0.37% | -0.11% | -0.04% | -0.52% | -0.65% | -0.82% |
| Class D | -0.33% | 0.16% | -0.40% | -0.61% | -0.93% | -1.13% |
| Class E |  |  |  |  |  |  |
| Overall | -0.31% | -0.12% | -0.11% | -0.41% | -0.51% | -0.44% |
|  | -0.33% | -0.12% | -0.10% | -0.42% | -0.53% | -0.45% |
| Class F | -0.15% | -0.30% | -0.04% | -0.96% | -1.25% | -1.29% |
| Enc Time[%] | 104% |  |  | 112% |  |  |
| Dec Time[%] | 105% |  |  | 113% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.34% | 0.59% | 0.67% | -0.35% | 0.66% | 0.04% |
| Class C | -0.37% | 0.36% | 0.28% | -0.40% | 0.28% | 0.16% |
| Class D | -0.25% | 0.51% | 1.21% | -0.20% | 0.86% | 0.18% |
| Class E | -0.20% | -0.06% | 0.64% | -0.26% | 1.04% | -0.57% |
| Overall | -0.30% | 0.39% | 0.70% | -0.31% | 0.68% | -0.01% |
|  | -0.29% | 0.34% | 0.71% | -0.30% | 0.62% | 0.05% |
| Class F | -0.52% | -0.44% | -0.53% | -0.78% | -0.82% | -0.37% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |

Note that the timing results are not reliable as they were obtained in a mixed computing cluster.

# Conclusions

This proposal describes a vector quantization method call parity inference that restricts the parity of the absolute sum of all quantization coefficients in a coefficient group to be odd. Experimental results based on HM7.0 reference software show that this method improves coding efficiency in all settings in common test conditions, and is particularly effective when RDOQ is turned off. Furthermore, the parity constraint enforced in this method leads to reduction of the maximum number of context-coded bins in a coefficient group from 25 to 24, which has practical implications. It is recommended that the technique be adopted into HEVC working draft.

# References

[1] G. Clare, F. Henry, and J. Jung, “Sign data hiding”, JCTVC-G271.

[2] X. Yu, J. Wang, D. He, G. Martin-Cocher, and S. Campbell, “Multiple sign bits hiding,” JCTVC-H0481.

[3] D. He, J. Wang, and G. Martin-Cocher, “Rotation of residual block for transform skipping,” JCTVC-J0093.

# Proposed syntax for working draft

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| // decode last significant coefficient information |  |
| // decode significant-coefficient group flag |  |
| // decode significant-coefficient flags for n = 15 to 1, i.e. excluding first position in CG |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 1; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 7 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| } |  |
| } |  |
| lastCoeffPos = numCoeff – 1– (numLastSubset << 4) |  |
| parityHidden = ( i < numLastSubset ? ( numSigCoeff >= 1 ) : ( lastCoeffPos >= 4 ) |  |
| // decode gr2 flag (if there was a gr1 flag = 1 in first fifteen coefficients) |  |
| // decode signs for n = 15 to 1 |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( n == 0 && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| firstParity = ( ( sumAbs%2 == 1 ) ? 0 : 1 ) |  |
| if( offset < (numCoeff-1) && numSigCoeff > 0 && ( !(data_hiding_flag && parityHidden) ¦¦ firstParity == 0 ) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( (!data_hiding_flag ¦¦ !parityHidden ¦¦ firstParity == 1) ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) { |  |
| if( !(data_hiding_flag && parityHidden) ¦¦ firstParity == 0 ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| } |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| } |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 7 ¦¦ n == 0) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| if( n == 0 && data_hiding_flag && parityHidden ) { |  |
| coeff_abs_level_remaining[ n ] *= 2 |  |
| baseLevel += ( (baseLevel % 2 != firstParity) ? 1 : 0 ) |  |
| } |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && parityHidden ) |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

significant_coeff_flag[ xC ][ yC ] specifies for the transform coefficient position ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at location ( xC, yC ) is non-zero as follows.

- If significant_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at location ( xC, yC ) is set equal to 0.

- Otherwise (significant_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at location ( xC, yC ) has a non-zero value.

When significant_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows.

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or both of the following conditions are true, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 1

- ( xC, yC ) is equal to ( xCG << 2, yCG << 2 )

- implicitNonZeroCoeff is equal to 1

- Otherwise, if ( xC, yC ) is equal to ( xCG << 2, yCG << 2 ) and all of the following conditions are true, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 1

- data_hiding_flag is equal to 1

- parityHidden is equal to 1

- firstParity is equal to 1

- Otherwise, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

coeff_abs_level_greater1_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 1.

When coeff_abs_level_greater1_flag[ n ] is not present, it is inferred as follows.

- If ( xC, yC ) is equal to ( xCG << 2, yCG << 2 ) and all of the following conditions are true, coeff_abs_level_greater1_flag[ xC ][ yC ] is inferred to be equal to 1

- numSigCoeff < 7

- data_hiding_flag is equal to 1

- parityHidden is equal to 1

- firstParity is equal to 0

- Otherwise, coeff_abs_level_greater1_flag[ xC ][ yC ] is inferred to be equal to 0.

##### 9.2.3.1.4. Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the current coefficient position in the scan order, n, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If parityHidden is equal to 1, sigCtx is derived as follows

- If xC + yC is equal to 0,

sigCtx  = ( cIdx > 0) ? 24 : 21

- Otherwise, if n is equal to 0,

sigCtx  =  ( cIdx > 0) ? 25 : 22

- Otherwise,

…    // current sigCtx derivation

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).