| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0275 WG11 Number: m25603 |
| --- | --- |

| Title: | Non-CE1: DST quantization matrix syntax for CE1 Simplification 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

Core Experiment 1 (CE1) is testing two simplifications: Simplification 1 uses the 2-D DST for 4x4 luma Intra transform block, and Simplification 2 is the same except that the 2-D DCT is used when the Intra prediction mode is DC. The purpose of this contribution is to re-propose the syntax in JCTVC-I0419, which supports an additional scaling list for 4x4 luma Intra transform blocks, in case Simplification 2 of CE1 is adopted. This syntax supports two default scaling lists for 4x4 luma Intra transform blocks: one associated with the DCT and the other associated with the DST. It is reported that because Simplification 2 of CE1 does not apply the DCT and DST together in the same 4x4 transform block, it is not necessary to switch between the two scaling lists in a transform block.

# Introduction

In Simplification 1 of Core Experiment 1 (CE1) [1], the DST is used horizontally and vertically for all 4x4 luma Intra transform blocks. Simplification 2 is similar, except when the Intra prediction mode is DC. In that case, the DCT is used horizontally and vertically. Table 7-4 of the HEVC text specification [2] defines one default scaling list or quantization matrix for 4x4 luma Intra transform blocks. If Simplification 1 of CE1 is adopted, then this scaling list would always be applied with the DST. If, however, Simplification 2 of CE1 is adopted, then that scaling list would be applied both when the DST or the DCT is used. In the previous JCT-VC meeting, syntax for supporting both one and three additional scaling lists for use with 4x4 Intra luma transform blocks was proposed in JCTVC-I0419 [3]. Values for the scaling lists were additionally proposed in JCTVC-I0268 [4].

The purpose of this proposal is to re-propose the syntax for supporting one additional scaling list, in case Simplification 2 of CE1 is adopted. Values for the actual scaling list are not proposed here, as our intent is to enable this support in the syntax so implementers and content creators can signal their own default matrices.

Figure 1 shows that the additional scaling list is always associated with the 2-D DST, and the other scaling list is always associated with the 2-D DCT. Because transform types are not mixed horizontally and vertically within a block, there is no need to switch among sets of scaling lists designed for each DCT/DST permutation. In an implementation, the scaling list and transform could be combined.

Scaling lists not switched between transforms

Scaling List for DST

2-D Inv DST

Scaling List for DCT

2-D Inv DCT

2-D Inv DST

2-D Inv DCT

Inverse Quantization

IntraPredMode == DC?

Reconstruction

yes

no

Figure 1. Scaling lsits for 2-D DST and 2-D DCT for 4x4 luma Intra transform blocks

# Changes to the HEVC text specification

Additions are indicated with yellow highlighting.

Deletions are indicated with blue highlighting.

#### 7.3.2.3   Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 67; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

Table 7-3 – Specification of MatrixID according to SizeID, prediction type and colour component

| - SizeID | - Prediction type | - Colour component | - MatrixID |
| --- | --- | --- | --- |
| - 0, 1, 2 | - Intra | - Y | - 0 |
| - 0, 1, 2 | - Intra | - Cb | - 1 |
| - 0, 1, 2 | - Intra | - Cr | - 2 |
| - 0, 1, 2 | - Inter | - Y | - 3 |
| - 0, 1, 2 | - Inter | - Cb | - 4 |
| - 0, 1, 2 | - Inter | - Cr | - 5 |
| - 0 | - Intra | - Y (DST) | - 6 |
| - 3 | - Intra | - Y | - 0 |
| - 3 | - Inter | - Y | - 1 |

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |
| - ScalingList[0][6][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |

# Alternative changes

The above syntax can support any corresponding proposed scaling list. For example, the scaling list from JCTVC-I0268 could be incorporated as follows:

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |
| - ScalingList[0][6][i] | - 8 | - 8 | - 7 | - 8 | - 13 | - 13 | - 12 | - 14 | - 19 | - 21 | - 29 | - 36 | - 25 | - 30 | - 47 | - 78 |

# References

- JCTVC-I1101, “Description of Core Experiment 1: Intra transform mode dependency simplifications,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, May 2012.

- B. Bross et al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003_d5, Geneva, CH, May 2012.

- R. Cohen and A. Vetro, “Syntax for supporting 4x4 DST quantization matrices,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0419, Geneva, CH, May 2012.

- J. Lou, Y. Yu, and L. Wang, “Quantization matrices for 4x4 DSTs in HEVC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0268, Geneva, CH, May 2012

# Patent rights declaration

Mitsubishi Electric Research laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).