| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0276 |
| --- | --- |

| Title: | CE1: Crosscheck of Nokia’s results on intra transform mode dependency simplifications (JCTVC-J0035) for low QPs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan L Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Nokia’s proposal on intra transform mode dependency simplifications (JCTVC-J0035) for low QPs. Two simplifications were tested. Simplification 1 proposes always using DST for intra luma 4×4 TUs. Simplification 2 proposes using DST for all intra prediction modes except DC for luma 4×4 TUs. The BD-rate results matched those reported by Nokia for low QP range. The average luma BD-rate difference for AI-main configuration was +0.1% for both simplification 1 and 2. A higher loss was observed for class F sequences (+0.8% for simplification 1, +0.5% for simplification 2).

# Introduction

This contribution provides a summary of a cross-check performed by Qualcomm of Nokia’s proposal on intra transform mode dependency simplifications (JCTVC-J0035) for low QPs. Two simplifications were tested. Simplification 1 proposes always using DST for intra luma 4×4 TUs. Simplification 2 proposes using DST for all intra prediction modes except DC for luma 4×4 TUs.

# Results

Results for AI-main configuration were crosschecked for low QP range (2, 7, 12, 17) for simplification 1 and simplification 2. Table 1 shows a summary of the BD-rate results for simplification 1. Table 2 shows a summary of the BD-rate results for simplification 2. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU.

Table 1: BD-rate results for low QP range for simplification 1

Table 2: BD-rate results for low QP range for simplification 2

# Conclusions

We have verified results reported in JCTVC-J0035 for low QPs. It proposes 2 simplifications. Simplification 1 always uses DST for intra luma 4×4 TUs. Simplification 2 uses DST for all intra prediction modes except DC for luma 4×4 TUs. The BD-rate results matched those reported by Nokia for low QP range. The average luma BD-rate difference for AI-main configuration was +0.1% for both simplification 1 and 2. A higher loss was observed for class F sequences (+0.8% for simplification 1, +0.5% for simplification 2). Encoding times matched very closely with those reported by the proponents.