| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0277 |
| --- | --- |

| Title: | Interaction between pcm_flag and cu_transquant_bypass_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L Joshi Geert Van Der Auwera Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com +1-858-658-5134 geertv@qualcomm.com +1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In the current HEVC text specification, the pcm_flag takes precedence over the cu_transquant_bypass_flag when both flags are one. It is proposed that when cu_transquant_bypass_flag is 1, the pcm_flag is not signalled and is inferred to be 0.

# Introduction

In the current HEVC draft, if for a particular CU, pcm_flag is 1, the PCM mode is used irrespective of the value of the cu_transquant_bypass_flag. Thus, signalling of cu_transquant_bypass_flag can be considered redundant when the block uses PCM mode.

# Proposal

We propose that the pcm_flag is not signalled when cu_transquant_bypass is 1. This preserves all of the existing functionality of the PCM mode as well as lossless mode while reducing the number of times pcm_flag needs to be signalled. If a CU desires to use the PCM mode, cu_transquant_bypass should be set to 0.

The only change in the current HEVC draft is that in the syntax table 7.3.7 “prediction unit syntax”, a check for zero value of cu_transquant_bypass_flag is added to decide whether pcm_flag should be read.

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize && !cu_transquant_bypass_flag) |  |
| pcm_flag | ae(v) |
| … |  |

The change is shown in red.

# Conclusions

It is proposed that pcm_flag is signalled only when the cu_transquant_bypass_flag is 0. This addresses the current situation that the cu_transquant_bypass_flag is redundant when the CU uses PCM mode. We request the adoption of this change into HEVC DIS.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).