## CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.

Figure 9-1 – Spatial neighbour T that is used to invoke the coding tree block availability derivation process relative to the current coding tree block (informative)

The mininum coding block address of the coding tree block containing the spatial neighbor block T (Figure 9-1), ctbMinCbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows.

x = x0 + 2 << Log2CtbSize − 1
y = y0 − 1
ctbMinCbAddrT = MinCbAddrZS[ x >> Log2MinCbSize ][ y >> Log2MinCbSize ]	(9-1)

The variable availableFlagT is obtained by invoking the coding block availability derivation process as specified in subclause Error: Reference source not found with ctbMinCbAddrT as input.

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found, , the following ordered steps apply.

- The arithmetic decoding engine is initialised as follows.

  - If CtbAddrRS is equal to slice_address, dependent_slice_flag is equal to 1 and entropy_coding_reset_flag is equal to 0, the following applies.

  - The synchronization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.3 with TableStateIdxDS and TableMPSValDS as input.

  - The decoding process for binary decisions before termination as specified in subclause 9.3.3.2.4 is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause 9.3.1.4.

  - Otherwise if tiles_or_entropy_coding_sync_idc is equal to 2, and CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

  - When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.3 with TableStateIdxWPP and TableMPSValWPP as input.

  - The decoding process for binary decisions before termination as specified in subclause 9.3.3.2.4 is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause 9.3.1.4.

- When cabac_independent_flag is equal to 0 and dependent_slice_flag is equal to 1, or when tiles_or_entropy_coding_sync_idc is equal to 2, the memorization process is applied as follows.

  - When tiles_or_entropy_coding_sync_idc is equal to 2 and CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.2 with TableStateIdxWPP and TableMPSValWPP as output.

  - When cabac_independent_flag is equal to 0, dependent_slice_flag is equal to 1, and end_of_slice_flag is equal to 1, the memorization process of the CABAC parsing process is invoked as specified in subclause 9.3.1.2 with TableStateIdxDS and TableMPSValDS as output.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause 9.3.2.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.2.8.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause 9.3.3.1.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause 9.3.3.2.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element pcm-flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.3.1.2.

### Initialization process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses 9.3.1.1 and 9.3.1.2 are invoked when starting the parsing of the slice data of a slice in subclause Error: Reference source not found or when starting the parsing of the data of a coding tree in subclause 7.3.5 and the coding tree is the first coding tree in a tile.

#### Initialization process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdxTable and ctxIdx.

Table 9-7 to Table 9-30 contain the values of the 8 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause 9.3.3.2.

From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and intersecIdx are derived according to the following pseudo-code process:

slopeIdx = initValue >> 4
intersecIdx = initValue & 15

Slope m and Intersec n are derived from the indices as follows:

m = slopeIdx*5 − 45
n = ( intersecIdx << 3 ) − 16

The two values assigned to pStateIdx and valMPS for the initialization are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the variable m and n, the initialization is specified by the following pseudo-code process:

preCtxState = Clip3( 1, 126, ( ( m * Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )
valMPS = ( preCtxState  <=  63) ? 0 : 1
pStateIdx = valMPS ? (preCtxState − 64) : (63 – preCtxState)	(9-2)

The variable initType in  is derived as follows:

if( slice_type = = I )
	initType = 0
else if(slice_type = = P )
	initType = cabac_init_flag ? 2 : 1
else
	initType = cabac_init_flag ? 1 : 2

Table 9-1 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao_param() | sao_merge_left_flag | Table 9-2 | 0..2 | 3..5 | 6..8 |
|  | sao_merge_up_flag | Table 9-3 | 0 | 1 | 2 |
|  | sao_type_idx | Table 9-4 | 0..1 | 2..3 | 4..5 |
|  | sao_offset_abs | Table 9-5 | 0..1 | 2..3 | 4..5 |
| alf_cu_control_param() | alf_cu_flag | Table 9-6 | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Table 9-7 | 0..3 | 4..7 | 8..11 |
| coding_unit() | cu_transquant_bypass_flag | Table 9-8 | 0 | 1 | 2 |
|  | skip_flag | Table 9-9 |  | 0..2 | 3..5 |
|  | cu_qp_delta | Table 9-10 | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Table 9-11 |  | 0 | 1 |
|  | part_mode | Table 9-11 | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Table 9-12 | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Table 9-13 | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Table 9-14 |  | 0 | 1 |
|  | merge_idx | Table 9-15 |  | 0 | 1 |
|  | inter_pred_idc | Table 9-16 |  | 0..4 | 5..9 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-17 |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Table 9-18 |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Table 9-18 |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Table 9-19 |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Table 9-20 |  | 0 | 1 |
|  | split_transform_flag | Table 9-21 | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Table 9-22 | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Table 9-23 | 0..2 | 3..5 | 6..8 |
| residual_coding() | transform_skip_flag[ ][ ][ 0 ] | Table 9-24 | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9-24 | 3 | 4 | 5 |
|  | last_significant_coeff_x_prefix | Table 9-25 | 0..17 | 18..35 | 36..54 |
|  | last_significant_coeff_y_prefix | Table 9-26 | 0..17 | 18..35 | 36..54 |
|  | significant_coeff_group_flag | Table 9-27 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-28 | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Table 9-29 | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag | Table 9-30 | 0..5 | 6..11 | 12..17 |

NOTE 2 – ctxIdxTable equal to 0 and ctxIdx equal to 0 are associated with the end_of_slice_flag. The decoding process specified in subclause 9.3.3.2.4 applies to ctxIdxTable equal to 0 and ctxIdx equal to 0. This decoding process, however, may also be implemented by using the decoding process specified in subclause 9.3.3.2.1. In this case, the initial values associated with ctxIdxTable equal to 0 and ctxIdx equal to 0 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

Table 9-2 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 | 118 | 102 | 102 | 102 | 102 | 102 | 102 |

Table 9-3 – Values of variable initValue for sao_merge_up_flag ctxIdx

| Initialization variable | sao_merge_up_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-4 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 61 | 104 | 168 | 120 | 184 | 120 |

Table 9-5 – Values of variable initValue for sao_offset_abs ctxIdx

| Initialization variable | sao_offset_abs ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 171 | 119 | 199 | 169 | 119 | 151 |

Table 9-6 – Values of variable initValue for alf_cu_flag ctxIdx

| Initialization variable | alf_cu_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-7 – Values of variable initValue for split_coding_unit_flag ctxIdx

| Initialization variable | split_coding_unit_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 167 | 143 | 153 | 141 | 15 | 124 | 137 | 153 | 15 | 124 | 137 | 153 |

Table 9-8 – Values of variable initValue for cu_transquant_bypass_flag ctxIdx

| Initialization variable | cu_transquant_bypass_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-9 – Values of variable initValue for skip_flag ctxIdx

| Initialization variable | skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 165 | 168 | 154 | 165 | 168 | 154 |

Table 9-10 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 |

Table 9-11 – Values of variable initValue for pred_mode and part_mode

| Initialization variable | pred_type ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 114 | 98 | 167 | 119 | 87 | 119 | 119 | 87 | 119 |

Table 9-12 – Values of variable initValue for prev_intra_luma_pred_flag ctxIdx

| Initialization variable | prev_intra_luma_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 167 | 119 | 150 |

Table 9-13 – Values of variable initValue for intra_chroma_pred_mode ctxIdx

| Initialization variable | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 53 | 103 | 85 | 87 | 101 | 87 |

Table 9-14 – Value of variable initValue for merge_flag ctxIdx

| Initialization variable | merge_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 72 | 119 |

Table 9-15 – Values of variable initValue for merge_idx ctxIdx

| Initialisation variable | merge_idx ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 100 | 116 |

Table 9-16 – Values of variable initValue for inter_pred_idc ctxIdx

| Initialisation variable | inter_pred_idc ctxIdx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 95 | 79 | 63 | 31 | 31 | 95 | 79 | 63 | 31 | 31 |

Table 9-17 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-18 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialization variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 120 | 166 | 135 | 166 |

Table 9-19 – Values of variable initValue for mvp_l0_flag, mvp_l1_flag ctxIdx

| Initialization variable | mvp_l0_flag, mvp_l1_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 134 | 134 |

Table 9-20 – Values of variable initValue for no_residual_data_flag ctxIdx

| Initialization variable | no_residual_data_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 39 | 39 |

Table 9-21 – Values of variable initValue for split_transform_flag ctxIdx

| Initialization variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 119 | 162 | 148 | 100 | 119 | 71 | 86 | 55 | 119 | 102 | 86 | 86 |

Table 9-22 – Values of variable initValue for cbf_luma ctxIdx

| Initialization variable | cbf_luma ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 73 | 74 | 102 | 89 | 102 | 89 |

Table 9-23 – Values of variable initValue for cbf_cb and cbf_cr ctxIdx

| Initialization variable | cbf_cb ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 55 | 86 | 133 | 114 | 84 | 117 | 114 | 68 | 117 |

Table 9-24 – Values of variable initValue for transform_skip_flag ctxIdx

| Initialization variable | transform_skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 109 | 102 | 102 | 102 | 102 | 102 |

Table 9-25 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-26 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 9-27 – Values of variable initValue for significant_coeff_group_flag ctxIdx

| Initialization variable | significant_coeff_group_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 83 | 122 | 98 | 121 | 99 | 120 | 67 | 119 | 99 | 120 | 67 | 119 |

Table 9-28 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialization variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 | 88 | 103 | 71 | 53 | 87 | 134 | 86 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 84 | 70 | 68 | 89 | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 149 | 70 | 52 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 118 | 133 | 116 | 114 | 129 | 132 | 162 | 115 | 51 | 115 | 66 | 120 | 74 | 115 | 87 | 89 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 118 | 134 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue | 100 | 68 | 67 | 136 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 70 | 53 | 67 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 117 | 102 | 117 | 1158 | 114 | 84 | 115 | 99 | 100 | 83 | 114 | 152 | 168 | 131 | 150 | 120 |
|  | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 | 103 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initValue | 116 | 68 | 67 | 152 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 86 | 84 | 67 |
|  | 128 | 129 | 130 | 131 | 132 | 133 | 134 |  |  |  |  |  |  |  |  |  |
| initValue | 117 | 102 | 117 | 115 | 99 | 100 | 115 |  |  |  |  |  |  |  |  |  |

Table 9-29 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 102 | 66 | 114 | 68 | 87 | 84 | 100 | 101 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 104130 | 130 | 147 | 149 | 104 | 196 | 100 | 165 | 119 | 179 | 179 | 164 | 119 | 85 | 117 | 149 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 116 | 135 | 146 | 147 | 164 | 119 | 148 | 116 | 133 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 100 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |  |  |  |  |  |  |  |  |
| initValue | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |  |  |  |  |  |  |  |  |

Table 9-30 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 86 | 102 | 115 | 117 | 101 | 101 | 84 | 117 | 83 | 100 | 84 | 117 |
|  | 12 | 13 | 14 | 15 | 16 | 17 |  |  |  |  |  |  |
| initValue | 84 | 117 | 83 | 84 | 84 | 117 |  |  |  |  |  |  |

#### Memorization process for context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the initialization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found and Error: Reference source not found except for the end_of_slice_flag.

For each context variable, the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMPS.

#### Synchronization process for context variables

Inputs of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMPS used in the memorization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found and Error: Reference source not found except for the end_of_slice_flag.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries pStateIdx and valMPS of tables tableStateSync and tableMPSSync.

#### Initialization process for the arithmetic decoding engine

This process is invoked before decoding the first coding tree block of a slice.

Outputs of this process are the initialised decoding engine registers codIRange and codIOffset both in 16 bit register precision.

The status of the arithmetic decoding engine is represented by the variables codIRange and codIOffset. In the initialization procedure of the arithmetic decoding process, codIRange is set equal to 510 and codIOffset is set equal to the value returned from read_bits( 9 ) interpreted as a 9 bit binary representation of an unsigned integer with most significant bit written first.

The bitstream shall not contain data that result in a value of codIOffset being equal to 510 or 511.

NOTE – The description of the arithmetic decoding engine in this Recommendation | International Standard utilizes 16 bit register precision. However, a minimum register precision of 9 bits is required for storing the values of the variables codIRange and codIOffset after invocation of the arithmetic decoding process (DecodeBin) as specified in subclause 9.3.3.2. The arithmetic decoding process for a binary decision (DecodeDecision) as specified in subclause 9.3.3.2.1 and the decoding process for a binary decision before termination (DecodeTerminate) as specified in subclause 9.3.3.2.4 require a minimum register precision of 9 bits for the variables codIRange and codIOffset. The bypass decoding process for binary decisions (DecodeBypass) as specified in subclause 9.3.3.2.3 requires a minimum register precision of 10 bits for the variable codIOffset and a minimum register precision of 9 bits for the variable codIRange.

### Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Table 9-31 specifies the type of binarization process, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset associated with each syntax element.

The specification of the unary (U) binarization process, the truncated unary (TU) binarization process, the truncated Rice (TR), the k-th order Exp-Golomb (EGk) binarization process, and the fixed-length (FL) binarization process are given in subclauses 9.3.2.1 to 9.3.2.5, respectively. Other binarizations are specified in subclauses 9.3.2.6 to 9.3.2.9.

The binarizations for the syntax element coeff_abs_level_remaining as specified in subclause 9.3.2.8 consist of bin strings given by a concatenation of prefix and suffix bit strings. For these binarization processes, the prefix and the suffix bit string are separately indexed using the binIdx variable as specified further in subclause 9.3.3. The two sets of prefix bit strings and suffix bit strings are referred to as the binarization prefix part and the binarization suffix part, respectively.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index table (ctxIdxTable) variable and the related context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table 9-31. When two values for each of these variables are specified for one syntax element in Table 9-31, the value in the upper row is related to the prefix part while the value in the lower row is related to the suffix part of the binarization of the corresponding syntax element.

The use of the DecodeBypass process and the variable bypassFlag is derived as follows.

–	If no value is assigned to ctxIdxOffset for the corresponding binarization or binarization part in Table 9-31 labelled as "na", all bins of the bit strings of the corresponding binarization or of the binarization prefix/suffix part are decoded by invoking the DecodeBypass process as specified in subclause 9.3.3.2.3. In such a case, bypassFlag is set equal to 1, where bypassFlag is used to indicate that for parsing the value of the bin from the bitstream the DecodeBypass process is applied.

–	Otherwise, for each possible value of binIdx up to the specified value of maxBinIdxCtx given in Table 9-31, a specific value of the variable ctxIdx is further specified in subclause 9.3.3. In such a case, bypassFlag is set equal to 0.

The possible values of the context index ctxIdx vary depending on the value of ctxIdxTable. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

ctxIdxTable = 0 and ctxIdx = ctxIdxOffset = 0 are assigned to the syntax element end_of_slice_flag as further specified in subclause 9.3.3.1. For parsing the value of the corresponding bin from the bitstream, the arithmetic decoding process for decisions before termination (DecodeTerminate) as specified in subclause 9.3.3.2.4 is applied.

| Table 9-31 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 | Table 9-2 | 0 |
|  | 1 |  | 0 | Table 9-2 | 3 |
|  | 2 |  | 0 | Table 9-2 | 6 |
| sao_merge_up_flag | 0 | FL, cMax = 1 | 0 | Table 9-3 | 0 |
|  | 1 |  | 0 | Table 9-3 | 1 |
|  | 2 |  | 0 | Table 9-3 | 2 |
| sao_type_idx | 0 | U | 1 | Table 9-4 | 0 |
|  | 1 |  | 1 | Table 9-4 | 2 |
|  | 2 |  | 1 | Table 9-4 | 4 |
| sao_band_position | 0 | FL, cMax = 5 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |
| sao_offset_abs | 0 | TU, cMax = ( 1<< (Min(bitDepth, 10) − 5)) − 1 | 1 | Table 9-5 | 0 |
|  | 1 |  | 1 | Table 9-5 | 2 |
|  | 2 |  | 1 | Table 9-5 | 4 |
| sao_offset_sign | 0 | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |
| alf_cu_flag | 0 | Fl, cMax = 1 | 0 | Table 9-6 | 0 |
|  | 1 |  | 0 | Table 9-6 | 1 |
|  | 2 |  | 0 | Table 9-6 | 2 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| split_coding_unit_flag | 0 | FL, cMax = 1 | 0 | Table 9-7 | 0 |
|  | 1 |  | 0 | Table 9-7 | 4 |
|  | 2 |  | 0 | Table 9-7 | 8 |
| cu_transquant_bypass_flag | 0 | FL, cMax = 1 | 0 | Table 9-8 | 0 |
|  | 1 |  | 0 | Table 9-8 | 1 |
|  | 2 |  | 0 | Table 9-8 | 2 |
| skip_flag | 1 | FL, cMax = 1 | 0 | Table 9-9 | 0 |
|  | 2 |  | 0 | Table 9-9 | 3 |
| cu_qp_delta | 0 | as specified in subclause 9.3.2.6 | 2 | Table 9-10 | 0 |
|  | 1 |  | 2 | Table 9-10 | 3 |
|  | 2 |  | 2 | Table 9-10 | 6 |
| pred_mode_flag | 1 | FL, cMax = 1 | 0 | Table 9-11 | 0 |
|  | 2 |  | 0 | Table 9-11 | 1 |
| part_mode | 0 | as specified in subclause 9.3.2.7 | 0 | Table 9-11 | 2 |
|  | 1 |  | 3 | Table 9-11 | 3 |
|  | 2 |  | 3 | Table 9-11 | 6 |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | 0 | FL, cMax = 1 | 0 | Table 9-12 | 0 |
|  | 1 |  | 0 | Table 9-12 | 1 |
|  | 2 |  | 0 | Table 9-12 | 2 |
| mpm_idx | all | TU, cMax = 2 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | FL, cMax = 31 | na | na | na, (uses Decode Bypass) |
| intra_chroma_pred_mode | 0 | prefix and suffix as specified in subclause 9.3.2.9 | prefix: 1 suffix: na | prefix: Table 9-13 suffix: na | prefix: 0 suffix: na, (uses Decode Bypass) |
|  | 1 |  | prefix: 1 suffix: na | prefix: Table 9-13 suffix: na | 2 suffix: na, (uses Decode Bypass) |
|  | 2 |  | prefix: 1 suffix: na | prefix: Table 9-13 suffix: na | 4 suffix: na, (uses Decode Bypass) |
| merge_flag | 1 | FL, cMax = 1 | 0 | Table 9-14 | 0 |
|  | 2 |  | 0 | Table 9-14 | 1 |
| merge_idx | 1 | TU, cMax = MaxNumMergeCand − 1 | 0 | Table 9-15 | 0 |
|  | 2 |  | 0 | Table 9-15 | 1 |
| inter_pred_idc | 1 | as specified in subclause 9.3.2.10 | 1 | Table 9-16 | 0 |
|  | 2 |  | 1 | Table 9-16 | 5 |
| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 2 | Table 9-17 | 0 |
|  | 2 |  | 2 | Table 9-17 | 3 |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Table 9-17 | 0 |
|  | 2 |  | 2 | Table 9-17 | 3 |
| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-18 | 0 |
|  | 2 |  | 0 | Table 9-18 | 1 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-18 | 2 |
|  | 2 |  | 0 | Table 9-18 | 3 |
| abs_mvd_minus2[ ] | 1/2 | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | 1/2 | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_l0_flag | 1 | FL, cMax = 1 | 0 | Table 9-19 | 0 |
|  | 2 |  | 0 | Table 9-19 | 1 |
| mvp_l1_flag | 1 | FL, cMax = 1 | 0 | Table 9-19 | 0 |
|  | 2 |  | 0 | Table 9-19 | 1 |
| no_residual_data_flag | 1 | FL, cMax = 1 | 0 | Table 9-20 | 0 |
|  | 2 |  | 0 | Table 9-20 | 1 |
| split_transform_flag | 0 | FL, cMax = 1 | 0 | Table 9-21 | 0 |
|  | 1 |  | 0 | Table 9-21 | 4 |
|  | 2 |  | 0 | Table 9-21 | 8 |
| cbf_luma | 0 | FL, cMax = 1 | 0 | Table 9-22 | 0 |
|  | 1 |  | 0 | Table 9-22 | 2 |
|  | 2 |  | 0 | Table 9-22 | 4 |
| cbf_cb, cbf_cr | 0 | FL, cMax = 1 | 0 | Table 9-23 | 0 |
|  | 1 |  | 0 | Table 9-23 | 3 |
|  | 2 |  | 0 | Table 9-23 | 6 |
| transform_skip_flag[ ][ ][ 0 ] | 0 | FL, cMax = 1 | 0 | Table 9-24 | 0 |
|  | 1 |  | 0 | Table 9-24 | 1 |
|  | 2 |  | 0 | Table 9-24 | 2 |
| transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | 0 | FL, cMax = 1 | 0 | Table 9-24 | 3 |
|  | 1 |  | 0 | Table 9-24 | 4 |
|  | 2 |  | 0 | Table 9-24 | 5 |
| last_significant_coeff_x_prefix | 0 | TU, cMax = ( log2TrafoWidth << 1 ) − 1 | 8 | Table 9-25 | 0 |
|  | 1 |  | 8 | Table 9-25 | 18 |
|  | 2 |  | 8 | Table 9-25 | 36 |
| last_significant_coeff_y_prefix | 0 | TU, cMax = ( log2TrafoHeight << 1 ) − 1 | 8 | Table 9-26 | 0 |
|  | 1 |  | 8 | Table 9-26 | 18 |
|  | 2 |  | 8 | Table 9-26 | 36 |
| last_significant_coeff_x_suffix | all | FL, cMax = (last_significant_coeff_x_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_y_suffix | all | FL, cMax = (last_significant_coeff_y_prefix >> 1) − 1 | na | na | na, (uses Decode Bypass) |
| significant_coeff_group_flag | 0 | FL, cMax = 1 | 0 | Table 9-27 | 0 |
|  | 1 |  | 0 | Table 9-27 | 4 |
|  | 2 |  | 0 | Table 9-27 | 8 |
| significant_coeff_flag | 0 | FL, cMax = 1 | 0 | Table 9-28 | 0 |
|  | 1 |  | 0 | Table 9-28 | 48 |
|  | 2 |  | 0 | Table 9-28 | 96 |
| coeff_abs_level_greater1_flag | 0 | FL, cMax = 1 | 0 | Table 9-29 | 0 |
|  | 1 |  | 0 | Table 9-29 | 24 |
|  | 2 |  | 0 | Table 9-29 | 48 |
| coeff_abs_level_greater2_flag | 0 | FL, cMax = 1 | 0 | Table 9-30 | 0 |
|  | 1 |  | 0 | Table 9-30 | 6 |
|  | 2 |  | 0 | Table 9-30 | 12 |
| coeff_abs_level_remaining | all | prefix and suffix as specified in subclause 9.3.2.8 | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

#### Unary (U) binarization process

Input to this process is a request for a U binarization for a syntax element.

Output of this process is the U binarization of the syntax element.

The bin string of a syntax element having (unsigned integer) value synElVal is a bit string of length synElVal + 1 indexed by binIdx. The bins for binIdx less than synElVal are equal to 1. The bin with binIdx equal to synElVal is equal to 0.

Table 9-32 illustrates the bin strings of the unary binarization for a syntax element.

Table 9-32 – Bin string of the unary binarization (informative)

| Value of syntax element | Bin string |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (I_NxN) | 0 |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |
| 4 | 1 | 1 | 1 | 1 | 0 |  |
| 5 | 1 | 1 | 1 | 1 | 1 | 0 |
| … |  |  |  |  |  |  |
| binIdx | 0 | 1 | 2 | 3 | 4 | 5 |

#### Truncated unary (TU) binarization process

Input to this process is a request for a TU binarization for a syntax element and cMax.

Output of this process is the TU binarization of the syntax element.

For syntax element (unsigned integer) values less than cMax, the U binarization process as specified in subclause 9.3.2.1 is invoked. For the syntax element value equal to cMax the bin string is a bit string of length cMax with all bins being equal to 1.

NOTE – TU binarization is always invoked with a cMax value equal to the largest possible value of the syntax element being decoded.

#### Truncated Rice (TR) binarization process

Input to this process is a request for a TR binarization for a syntax element, cRiceParam and cTRMax.

Output of this process is the TR binarization of the syntax element.

A TR bin string is a concatenation of a prefix bit string and (when present) a suffix bit string. The prefix of the binarization is specified by invoking the TU binarization process for the prefix part of the value specified by synElVal >> cRiceParam with cMax = cTRMax >> cRiceParam. When cTRMax is greater than synElVal, the suffix of the TR bin string is present and is specified by the binary representation of synElVal – ( ( synElVal >> cRiceParam ) << cRiceParam ).

NOTE – For the input parameter cRiceParam = 0 the TR binarization is exactly the TU binarization.

#### k-th order Exp-Golomb (EGk) binarization process

Input to this process is a request for an EGk binarization for a syntax element.

Output of this process is the EGk binarization of the syntax element.

The bin string of the EGk binarization process of a syntax element synVal is specified by a process equivalent to the following pseudo-code:

absV = Abs( synVal )
stopLoop = 0
do {
	if( absV  >=  ( 1 << k ) ) {
		put( 1 )
		absV = absV − ( 1 << k )
		k++
	} else {
		put( 0 ) 																				(9-3)
		while( k− − ) 
			put( ( absV >> k )  &  1 )
		stopLoop = 1
	}
} while( !stopLoop )

NOTE 1 – The specification for the k-th order Exp-Golomb (EGk) code uses 1's and 0's in reverse meaning for the unary part of the Exp-Golomb code of 0-th order as specified in subclause Error: Reference source not found.

#### Fixed-length (FL) binarization process

Input to this process is a request for a FL binarization for a syntax element and cMax.

Output of this process is the FL binarization of the syntax element.

FL binarization is constructed by using a fixedLength-bit unsigned integer bin string of the syntax element value, where fixedLength = Ceil( Log2( cMax + 1 ) ). The indexing of bins for the FL binarization is such that the binIdx = 0 relates to the most significant bit with increasing values of binIdx towards the least significant bit.

#### Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		cNum = absV − 2
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}

#### Binarization process for part_mode

Input to this process is a request for a binarization for the syntax element part_mode and a variable cLog2CbSize specifying the current CU size.

Output of this process is the binarization of the syntax element.

The binarization for part_mode is given by Table 9-33 depending on PredMode and cLog2CbSize.

Table 9-33 – Binarization for part_mode

| PredMode | Value of part_mode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | cLog2CbSize > Log2MinCbSize | cLog2CbSize = = Log2MinCbSize |  |
|  |  |  |  | cLog2CbSize = = 3 | cLog2CbSize > 3 |
| MODE_INTRA | 0 | PART_2Nx2N | - | 1 | 1 |
|  | 1 | PART_NxN | - | 0 | 0 |
| MODE_INTER | 0 | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | PART_2NxN | 011 | 01 | 01 |
|  | 2 | PART_Nx2N | 001 | 00 | 001 |
|  | 3 | PART_NxN | - | - | 000 |
|  | 4 | PART_2NxnU | 0100 | - | - |
|  | 5 | PART_2NxnD | 0101 | - | - |
|  | 6 | PART_nLx2N | 0000 | - | - |
|  | 7 | PART_nRx2N | 0001 | - | - |

#### Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], and baseLevel.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows.

- If n is equal to 15, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is less than 15 ), cLastSE is set equal to baseLevel + coeff_abs_level_remaining[ n − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >= ( 3 * ( 1 << cLastRiceParam ) ) ) ? 1 : 0, 4 )	(9-4)

The variable cTRMax is derived from cRiceParam as:

cTRMax = 9 << cRiceParam	(9-5)

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause 9.3.2.3 for the syntax element coeff_abs_level_remaining[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived using the EGk binarization as specified in subclause 9.3.2.4 with the Exp-Golomb order k set equal to cRiceParam + 1.

#### Binarization process for intra_chroma_pred_mode

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode and the chroma_pred_from_luma_enabled_flag.

Output of this process is the binarization of the syntax element.

The binarization of the syntax element intra_chroma_pred_mode consists of a prefix part and (when present) a suffix part. Table 9-34 specifies the binarization of the prefix part and the suffix part.

Table 9-34 – Specifcation of intra_chroma_pred_mode depending on chroma_pred_from_luma_enabled_flag

| Value of intra_chroma_pred_mode | chroma_pred_from_luma_enabled_flag = 1 |  | chroma_pred_from_luma_enabled_flag = 0 |  |
| --- | --- | --- | --- | --- |
|  | prefix | suffix | prefix | suffix |
| 5 | 0 | n/a | n/a | n/a |
| 4 | 10 | n/a | 0 | n/a |
| 0 | 11 | 00 | 1 | 00 |
| 1 | 11 | 01 | 1 | 01 |
| 2 | 11 | 10 | 1 | 10 |
| 3 | 11 | 11 | 1 | 11 |

#### Binarization process for inter_pred_idc

Input to this process is a request for a binarization for the syntax element inter_pred_idc.

Output of this process is the binarization of the syntax element.

The binarization for inter_pred_idc is given by Table 9-35.

Table 9-35 – Binarization for inter_pred_idc

| Value of inter_pred_idc | Name of inter_pred_idc | Bin string |
| --- | --- | --- |
| 1 | Pred_L0 | 00 |
| 2 | Pred_L1 | 01 |
| 3 | Pred_BI | 1 |

### Decoding process flow

Input to this process is a binarization of the requested syntax element, maxBinIdxCtx, bypassFlag, ctxIdxTable and ctxIdxOffset as specified in subclause 9.3.2.

Output of this process is the value of the syntax element.

This process specifies how each bit of a bit string is parsed for each syntax element.

After parsing each bit, the resulting bit string is compared to all bin strings of the binarization of the syntax element and the following applies.

–	If the bit string is equal to one of the bin strings, the corresponding value of the syntax element is the output.

–	Otherwise (the bit string is not equal to one of the bin strings), the next bit is parsed.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

When the binarization of the corresponding syntax element consists of a prefix and a suffix binarization part, the variable binIdx is set equal to 0 for the first bin of each part of the bin string (prefix part or suffix part). In this case, after parsing the prefix bit string, the parsing process of the suffix bit string related to the binarizations specified in subclauses 9.3.2.3 and 9.3.2.8 is invoked depending on the resulting prefix bit string as specified in subclauses 9.3.2.3 and 9.3.2.8.

Depending on the variable bypassFlag, the following applies.

–	If bypassFlag is equal to 1, the bypass decoding process as specified in subclause 9.3.3.2.3 is applied for parsing the value of the bins from the bitstream.

–	Otherwise (bypassFlag is equal to 0), the parsing of each bin is specified by the following two ordered steps:

1.	Given binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset, ctxIdx is derived as specified in subclause 9.3.3.1.

2.	Given ctxIdxTable and ctxIdx, the value of the bin from the bitstream as specified in subclause 9.3.3.2 is decoded.

#### Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset.

Output of this process is ctxIdx.

Table 9-36 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxTable and ctxIdxOffset.

The ctxIdx to be used with a specific binIdx is specified by first determining the ctxIdxTable and ctxIdxOffset associated with the given bin string or part thereof. The ctxIdxTable and the ctxIdxOffset are listed in Table 9-36, the ctxIdx for a binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Table 9-36. When more than one value is listed in Table 9-36 for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All bins with binIdx greater than maxBinIdxCtx are parsed using the value of ctxIdx being assigned to binIdx equal to maxBinIdxCtx.

All entries in Table 9-36 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

| Table 9-36 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| sao_merge_left_flag | Table 9-2 | 0 | cIdx | na | na | na | na |
|  |  | 3 | cIdx | na | na | na | na |
|  |  | 6 | cIdx | na | na | na | na |
| sao_merge_up_flag | Table 9-3 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| sao_type_idx | Table 9-4 | 0 | 0 | 1 | 1 | 1 | 1 |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |
| sao_offset_abs | Table 9-5 | 0 | 0 | 1 | 1 | 1 | 1 |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |
| alf_cu_flag | Table 9-6 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| split_coding_unit_flag | Table 9-7 | 0 | 0,1,2,3 (subclause 9.3.3.1.1.1) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause 9.3.3.1.1.1) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause 9.3.3.1.1.1) | na | na | na | na |
| cu_transquant_bypass_flag | Table 9-8 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| skip_flag | Table 9-9 | 0 | 0,1,2 (subclause 9.3.3.1.1.1) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause 9.3.3.1.1.1) | na | na | na | na |
| cu_qp_delta | Table 9-10 | 0 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
|  |  | 3 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
|  |  | 6 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
| pred_mode_flag | Table 9-11 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| part_mode | Table 9-11 | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | 1 | 2 | na (uses Decode Bypass) | na |
|  |  | 6 | 0 | 1 | 2 | na (uses Decode Bypass) | na |
| part_mode (cLog2CbSize = = Log2MinCbSize && cLog2CbSize > 3) | Table 9-11 | 3 | 0 | 1 | 2 | na | na |
|  |  | 6 | 0 | 1 | 2 | na | na |
| prev_intra_luma_pred_flag | Table 9-12 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| intra_chroma_pred_mode (chroma_pred_from_luma_enabled_flag == 1) | Table 9-13 | 0 | 0 | 1 | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
|  |  | 2 | 0 | 1 | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
|  |  | 4 | 0 | 1 | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
| intra_chroma_pred_mode (chroma_pred_from_luma_enabled_flag == 0) | Table 9-13 | 0 | 0 | na (uses Decode Bypass) | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
|  |  | 2 | 0 | na (uses Decode Bypass) | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
|  |  | 4 | 0 | na (uses Decode Bypass) | na (uses Decode Bypass) | na (uses Decode Bypass) | na |
| merge_flag | Table 9-14 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| merge_idx | Table 9-15 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| inter_pred_idc | Table 9-16 | 0 | ctDepth | 4 | na | na | na |
|  |  | 5 | ctDepth | 4 | na | na | na |
| ref_idx_l0 | Table 9-17 | 0 | 0 | 1 | 2 | 2 | 2 |
| ref_idx_l0, ref_idx_l1 | Table 9-17 | 3 | 0 | 1 | 2 | 2 | 2 |
| abs_mvd_greater0_flag[ ] | Table 9-18 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Table 9-18 | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | na | na | na | na |
| mvp_l0_flag | Table 9-19 | 0 | 0 | na | na | na | na |
| mvp_l0_flag, mvp_l1_flag | Table 9-19 | 1 | 0 | na | na | na | na |
| no_residual_data_flag | Table 9-20 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| split_transform_flag | Table 9-21 | 0 | ctDepth + trafoDepth | na | na | na | na |
|  |  | 4 | ctDepth + trafoDepth | na | na | na | na |
|  |  | 8 | ctDepth + trafoDepth | na | na | na | na |
| cbf_luma | Table 9-22 | 0 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
|  |  | 2 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
|  |  | 4 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
| cbf_cb, cbf_cr | Table 9-23 | 0 | trafoDepth | na | na | na | na |
|  |  | 3 | trafoDepth | na | na | na | na |
|  |  | 6 | trafoDepth | na | na | na | na |
| transform_skip_flag[ ][ ][ 0 ] | Table 9-24 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9-24 | 3 | 0 | na | na | na | na |
|  |  | 4 | 0 | na | na | na | na |
|  |  | 5 | 0 | na | na | na | na |
| last_significant_coeff_x_prefix | Table 9-25 | 0 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
|  |  | 18 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
|  |  | 36 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
| last_significant_coeff_y_prefix | Table 9-26 | 0 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
|  |  | 18 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
|  |  | 36 | 0..17 (subclause 9.3.3.1.2) |  |  |  |  |
| significant_coeff_group_flag | Table 9-27 | 0 | 0..3 (subclause 9.3.3.1.3) | na | na | na | na |
|  |  | 4 | 0..3 (subclause 9.3.3.1.3) | na | na | na | na |
|  |  | 8 | 0..3 (subclause 9.3.3.1.3) | na | na | na | na |
| significant_coeff_flag | Table 9-28 | 0 | 0..47 (subclause 9.3.3.1.4) | na | na | na | na |
|  |  | 48 | 48..95 (subclause 9.3.3.1.4) | na | na | na | na |
|  |  | 96 | 96..143 (subclause 9.3.3.1.4) | na | na | na | na |
| coeff_abs_level_greater1_flag | Table 9-29 | 0 | 0..23 (subclause 9.3.3.1.5) | na | na | na | na |
|  |  | 24 | 0..47 (subclause 9.3.3.1.5) | na | na | na | na |
|  |  | 48 | 0..71 (subclause 9.3.3.1.5) | na | na | na | na |
| coeff_abs_level_greater2_flag | Table 9-30 | 0 | 0..5 (subclause 9.3.3.1.6) | na | na | na | na |
|  |  | 6 | 0..11 (subclause 9.3.3.1.6) | na | na | na | na |
|  |  | 12 | 0..17 (subclause 9.3.3.1.6) | na | na | na | na |

##### Assignment process of ctxIdxInc using neighbouring syntax elements

Subclause 9.3.3.1.1.1 specifies the derivation process of ctxIdxInc for the syntax elements split_coding_unit_flag and skip_flag.

###### Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows. [Ed. (BB): Define MinPuSize in the SPS but the derivation should depend on the use of an AMP flag ]

- If the prediction unit covering location ( xL, yL ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize where MinPuSize = 1 << ( Log2MinCbSize − 1). The  variable availableA is derived as follows.

- If the prediction unit covering location ( xA, yA ) is available and the coding tree block address of coding tree block containing the luma location ( xA, yA) is equal to the coding tree block address of coding tree block containing the luma location ( xP, yP )[Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], availableA is set equal to 1.

- Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements split_coding_unit_flag and skip_flag is specified in Table 9-37.

Table 9-37 – Specifcation of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| split_coding_unit_flag | ctDepth[ xL ][ yL ] > ctDepth[ xP ][ yP ] | 2 * (ctDepth[xP][yP] / 2) | ( condL && availableL ) + condA |
| skip_flag | skip_flag[ xL ][ yL ] | skip_flag[ xA ][ yA ] | ( condL && availableL ) + ( availableA ? condA : ( condL && availableL ) ) |

##### Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable log2Size is derived as follows.

- If the syntax element is last_significant_coeff_x_prefix, log2Size is set equal to log2TrafoWidth.

- Otherwise (the syntax element is last_significant_coeff_y_prefix), log2Size is set equal to log2TrafoHeight.

The variables ctxOffset and ctxShift are derived as follows.

- If cIdx is equal to 0, ctxOffset is set equal to 3 * (log2Size − 2) + ((log2Size − 1) >> 2) and ctxShift is set equal to (log2Size + 1) >> 2.

- Otherwise (cIdx is greater than 0), ctxOffset is set equal to 15 and ctxShift is set equal to log2Size − 2.

The variable ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >> ctxShift)+ ctxOffset		(9-6)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

##### Derivation process of ctxIdxInc for the syntax element significant_coeff_group_flag

Inputs to this process are the colour component index cIdx, the current coefficient group scan position ( xCG, yCG ), the previously decoded bins of the syntax element significant_coeff_group_flag, the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable ctxCG is derived using the current position ( xCG, yCG ), two previously decoded bins of the syntax element significant_coeff_group_flag in scan order, and the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight as follows.

- ctxCG is initialized with 0 as follows.

ctxCG  =  0		(9-7)

- When xCG is less than ( 1 << ( log2TrafoWidth − 2 ) ) − 1, ctxCG is modified as follows.

ctxCG  +=  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-8)

- When yCG is less than ( 1 << ( log2TrafoHeight − 2 ) ) − 1, ctxCG is modified as follows.

ctxCG  +=  significant_coeff_group_flag[ xCG ][ yCG + 1 ]	(9-9)

The context index increment ctxIdxInc is derived using the colour component index cIdx and ctxCG as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  = min(ctxCG, 1)		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  2  + min(ctxCG, 1)		(9-11)

##### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_group_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-38 as follows..

sigCtx  =  ctxIdxMap[ (yC << 2) + xC ]	(9-12)

- Otherwise, if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap[ ] specified in Table 9-38 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 0 : ( 9 + ctxIdxMap[ ((yC >> 1 ) << 2) + (xC >> 1) ] )	(9-13)

- Otherwise, if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  0		(9-14)

- Otherwise, sigCtx is derived using previously decoded bins of the syntax element significant_coeff_group_flag as follows.

- The horizontal and vertical coefficient group positions xCG and yCG are set equal to (xC >> 2) and (yC >> 2), respectively.

- The variable prevSig is set equal to 0.

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

prevSig  +=  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-15)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

prevSig  +=  (significant_coeff_group_flag[ xCG ][ yCG + 1 ] << 1)	(9-16)

- The variable sigCtx is derived as follows.

  - If prevSig is equal to 0, the the following applies.

sigCtx  =  ( xC − (xCG << 2) ) + ( yC − (yCG << 2) ) <= 2 ? 1 : 0	(9 26)

  - Otherwise, if prevSig is equal to 1, the following applies.

sigCtx  =  ( yC − (yCG << 2) ) <= 1 ? 1 : 0	(9 27)

  - Otherwise, if prevSig is equal to 2, the following applies.

sigCtx  =  ( xC − (xCG << 2) ) <= 1 ? 1 : 0	(9 28)

  - Otherwise ( prevSig is equal to 3 ), the following applies.

sigCtx  =  ( xC − (xCG << 2) ) + ( yC − (yCG << 2) ) <= 4 ? 2 : 1	(9 29)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx += 21		(9-17)

  - Otherwise, the following applies.

sigCtx += 18	(9-18)

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-19)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  24  +  sigCtx		(9-20)

Table 9-38 – Specifcation of ctxIdxMap[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

##### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the colour component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-21)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-22)

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause 9.3.3.1.6 for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-23)

- The variable greater1Ctx is set equal to 1.

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater1Ctx is set equal to the variable greater1Ctx that has been derived during the last invocation of this subclause.

- When greater1Ctx is greater than 0, the variable lastGreater1Flag is set equal to the syntax element coeff_abs_level_greater1_flag that has been used during the last invocation of this subclause and greater1Ctx is modifed as follows.

- If lastGreater1Flag is equal to 1, greater1Ctx is set equal to 0.

- Otherwise (lastGreater1Flag is equal to 0), greater1Ctx is incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )	(9-24)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  16		(9-25)

##### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the colour component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntex elements coeff_abs_level_greater2_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-26)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-27)

- The variable numGreater1 is set equal to 0.

ctxIdxInc  =  ctxSet		(9-28)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  4		(9-29)

#### Arithmetic decoding process

Inputs to this process are the bypassFlag, ctxIdxTable, and ctxIdx as derived in subclause 9.3.3.1, and the state variables codIRange and codIOffset of the arithmetic decoding engine.

Output of this process is the value of the bin.

Figure 9-2 illustrates the whole arithmetic decoding process for a single bin. For decoding the value of a bin, the context index table ctxIdxTable and the ctxIdx is passed to the arithmetic decoding process DecodeBin(ctxIdxTable, ctxIdx), which is specified as follows.

–	If bypassFlag is equal to 1, DecodeBypass( ) as specified in subclause 9.3.3.2.3 is invoked.

–	Otherwise, if bypassFlag is equal to 0, ctxIdxTable is equal  0 and ctxIdx is equal to 0, DecodeTerminate( ) as specified in subclause 9.3.3.2.4 is invoked.

–	Otherwise (bypassFlag is equal to 0, ctxIdxTable is not equal to 0 and ctxIdx is not equal to 0), DecodeDecision( ) as specified in subclause 9.3.3.2.1 is applied.

Figure 9-2 – Overview of the arithmetic decoding process for a single bin (informative)

NOTE – Arithmetic coding is based on the principle of recursive interval subdivision. Given a probability estimation p( 0 ) and p( 1 ) = 1  p( 0 ) of a binary decision ( 0, 1 ), an initially given code sub-interval with the range codIRange will be subdivided into two sub-intervals having range p( 0 ) * codIRange and codIRange  p( 0 ) * codIRange, respectively. Depending on the decision, which has been observed, the corresponding sub-interval will be chosen as the new code interval, and a binary code string pointing into that interval will represent the sequence of observed binary decisions. It is useful to distinguish between the most probable symbol (MPS) and the least probable symbol (LPS), so that binary decisions have to be identified as either MPS or LPS, rather than 0 or 1. Given this terminology, each context is specified by the probability pLPS of the LPS and the value of MPS (valMPS), which is either 0 or 1.

The arithmetic core engine in this Recommendation | International Standard has three distinct properties:

–	The probability estimation is performed by means of a finite-state machine with a table-based transition process between 64 different representative probability states { pLPS(pStateIdx) | 0 <= pStateIdx < 64 } for the LPS probability pLPS. The numbering of the states is arranged in such a way that the probability state with index pStateIdx = 0 corresponds to an LPS probability value of 0.5, with decreasing LPS probability towards higher state indices.

–	The range codIRange representing the state of the coding engine is quantized to a small set {Q1,…,Q4} of pre-set quantization values prior to the calculation of the new interval range. Storing a table containing all 64x4 pre-computed product values of Qi * pLPS(pStateIdx) allows a multiplication-free approximation of the product codIRange * pLPS(pStateIdx).

–	For syntax elements or parts thereof for which an approximately uniform probability distribution is assumed to be given a separate simplified encoding and decoding bypass process is used.

##### Arithmetic decoding process for a binary decision

Inputs to this process are ctxIdxTable, ctxIdx, codIRange, and codIOffset.

Outputs of this process are the decoded value binVal, and the updated variables codIRange and codIOffset.

Figure 9-3 shows the flowchart for decoding a single decision (DecodeDecision):

- The value of the variable codIRangeLPS is derived as follows.

–	Given the current value of codIRange, the variable qCodIRangeIdx is derived by

qCodIRangeIdx =( codIRange  >>  6 ) & 3		(9-30)

–	Given qCodIRangeIdx and pStateIdx associated with ctxIdxTable and ctxIdx, the value of the variable rangeTabLPS as specified in Table 9-39 is assigned to codIRangeLPS:

codIRangeLPS = rangeTabLPS[ pStateIdx ][ qCodIRangeIdx ]	(9-31)

- The variable codIRange is set equal to codIRange  codIRangeLPS and the following applies.

–	If codIOffset is greater than or equal to codIRange, the variable binVal is set equal to 1 − valMPS, codIOffset is decremented by codIRange, and codIRange is set equal to codIRangeLPS.

–	Otherwise, the variable binVal is set equal to valMPS.

Given the value of binVal, the state transition is performed as specified in subclause 9.3.3.2.1.1. Depending on the current value of codIRange, renormalization is performed as specified in subclause 9.3.3.2.2.

###### State transition process

Inputs to this process are the current pStateIdx, the decoded value binVal and valMPS values of the context variable associated with ctxIdxTable and ctxIdx.

Outputs of this process are the updated pStateIdx and valMPS of the context variable associated with ctxIdx.

Depending on the decoded value binVal, the update of the two variables pStateIdx and valMPS associated with ctxIdx is derived as specified by the following pseudo-code:

if( binVal  = =  valMPS ) 
	pStateIdx = transIdxMPS( pStateIdx )
else {				(9-32)
	if( pStateIdx  = =  0 )
		valMPS = 1  valMPS
	pStateIdx = transIdxLPS( pStateIdx )
}

Table 9-40 specifies the transition rules transIdxMPS( ) and transIdxLPS( ) after decoding the value of valMPS and 1  valMPS, respectively.

Figure 9-3 – Flowchart for decoding a decision

[Ed. (BB): add ctxIdxTable to the figure]

Table 9-39 – Specification of rangeTabLPS depending on pStateIdx and qCodIRangeIdx

| pStateIdx | qCodIRangeIdx |  |  |  | pStateIdx | qCodIRangeIdx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  | 0 | 1 | 2 | 3 |
| 0 | 128 | 176 | 208 | 240 | 32 | 27 | 33 | 39 | 45 |
| 1 | 128 | 167 | 197 | 227 | 33 | 26 | 31 | 37 | 43 |
| 2 | 128 | 158 | 187 | 216 | 34 | 24 | 30 | 35 | 41 |
| 3 | 123 | 150 | 178 | 205 | 35 | 23 | 28 | 33 | 39 |
| 4 | 116 | 142 | 169 | 195 | 36 | 22 | 27 | 32 | 37 |
| 5 | 111 | 135 | 160 | 185 | 37 | 21 | 26 | 30 | 35 |
| 6 | 105 | 128 | 152 | 175 | 38 | 20 | 24 | 29 | 33 |
| 7 | 100 | 122 | 144 | 166 | 39 | 19 | 23 | 27 | 31 |
| 8 | 95 | 116 | 137 | 158 | 40 | 18 | 22 | 26 | 30 |
| 9 | 90 | 110 | 130 | 150 | 41 | 17 | 21 | 25 | 28 |
| 10 | 85 | 104 | 123 | 142 | 42 | 16 | 20 | 23 | 27 |
| 11 | 81 | 99 | 117 | 135 | 43 | 15 | 19 | 22 | 25 |
| 12 | 77 | 94 | 111 | 128 | 44 | 14 | 18 | 21 | 24 |
| 13 | 73 | 89 | 105 | 122 | 45 | 14 | 17 | 20 | 23 |
| 14 | 69 | 85 | 100 | 116 | 46 | 13 | 16 | 19 | 22 |
| 15 | 66 | 80 | 95 | 110 | 47 | 12 | 15 | 18 | 21 |
| 16 | 62 | 76 | 90 | 104 | 48 | 12 | 14 | 17 | 20 |
| 17 | 59 | 72 | 86 | 99 | 49 | 11 | 14 | 16 | 19 |
| 18 | 56 | 69 | 81 | 94 | 50 | 11 | 13 | 15 | 18 |
| 19 | 53 | 65 | 77 | 89 | 51 | 10 | 12 | 15 | 17 |
| 20 | 51 | 62 | 73 | 85 | 52 | 10 | 12 | 14 | 16 |
| 21 | 48 | 59 | 69 | 80 | 53 | 9 | 11 | 13 | 15 |
| 22 | 46 | 56 | 66 | 76 | 54 | 9 | 11 | 12 | 14 |
| 23 | 43 | 53 | 63 | 72 | 55 | 8 | 10 | 12 | 14 |
| 24 | 41 | 50 | 59 | 69 | 56 | 8 | 9 | 11 | 13 |
| 25 | 39 | 48 | 56 | 65 | 57 | 7 | 9 | 11 | 12 |
| 26 | 37 | 45 | 54 | 62 | 58 | 7 | 9 | 10 | 12 |
| 27 | 35 | 43 | 51 | 59 | 59 | 7 | 8 | 10 | 11 |
| 28 | 33 | 41 | 48 | 56 | 60 | 6 | 8 | 9 | 11 |
| 29 | 32 | 39 | 46 | 53 | 61 | 6 | 7 | 9 | 10 |
| 30 | 30 | 37 | 43 | 50 | 62 | 6 | 7 | 8 | 9 |
| 31 | 29 | 35 | 41 | 48 | 63 | 2 | 2 | 2 | 2 |

Table 9-40 – State transition table

| pStateIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| transIdxLPS | 0 | 0 | 1 | 2 | 2 | 4 | 4 | 5 | 6 | 7 | 8 | 9 | 9 | 11 | 11 | 12 |
| transIdxMPS | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| pStateIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| transIdxLPS | 13 | 13 | 15 | 15 | 16 | 16 | 18 | 18 | 19 | 19 | 21 | 21 | 22 | 22 | 23 | 24 |
| transIdxMPS | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| pStateIdx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| transIdxLPS | 24 | 25 | 26 | 26 | 27 | 27 | 28 | 29 | 29 | 30 | 30 | 30 | 31 | 32 | 32 | 33 |
| transIdxMPS | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| pStateIdx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| transIdxLPS | 33 | 33 | 34 | 34 | 35 | 35 | 35 | 36 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 63 |
| transIdxMPS | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 62 | 63 |

##### Renormalization process in the arithmetic decoding engine

Inputs to this process are bits from slice data and the variables codIRange and codIOffset.

Outputs of this process are the updated variables codIRange and codIOffset.

A flowchart of the renormalization is shown in Figure 9-4. The current value of codIRange is first compared to 256 and further steps are specified as follows.

–	If codIRange is greater than or equal to 256, no renormalization is needed and the RenormD process is finished;

–	Otherwise (codIRange is less than 256), the renormalization loop is entered. Within this loop, the value of codIRange is doubled, i.e., left-shifted by 1 and a single bit is shifted into codIOffset by using read_bits( 1 ).

The bitstream shall not contain data that result in a value of codIOffset being greater than or equal to codIRange upon completion of this process.

Figure 9-4 – Flowchart of renormalization

##### Bypass decoding process for binary decisions

Inputs to this process are bits from slice data and the variables codIRange and codIOffset.

Outputs of this process are the updated variable codIOffset and the decoded value binVal.

The bypass decoding process is invoked when bypassFlag is equal to 1. Figure 9-5 shows a flowchart of the corresponding process.

First, the value of codIOffset is doubled, i.e., left-shifted by 1 and a single bit is shifted into codIOffset by using read_bits( 1 ). Then, the value of codIOffset is compared to the value of codIRange and further steps are specified as follows.

–	If codIOffset is greater than or equal to codIRange, the variable binVal is set equal to 1 and codIOffset is decremented by codIRange.

–	Otherwise (codIOffset is less than codIRange), the variable binVal is set equal to 0.

The bitstream shall not contain data that result in a value of codIOffset being greater than or equal to codIRange upon completion of this process.

Figure 9-5 – Flowchart of bypass decoding process

##### Decoding process for binary decisions before termination

Inputs to this process are bits from slice data and the variables codIRange and codIOffset.

Outputs of this process are the updated variables codIRange and codIOffset, and the decoded value binVal.

This special decoding routine applies to decoding of end_of_slice_flag and pcm_flag corresponding to ctxIdxTable equal to 0 and ctxIdx equal to 0. Figure 9-6 shows the flowchart of the corresponding decoding process, which is specified as follows.

First, the value of codIRange is decremented by 2. Then, the value of codIOffset is compared to the value of codIRange and further steps are specified as follows.

–	If codIOffset is greater than or equal to codIRange, the variable binVal is set equal to 1, no renormalization is carried out, and CABAC decoding is terminated. The last bit inserted in register codIOffset is equal to 1. When decoding end_of_slice_flag, this last bit inserted in register codIOffset is interpreted as rbsp_stop_one_bit.

–	Otherwise (codIOffset is less than codIRange), the variable binVal is set equal to 0 and renormalization is performed as specified in subclause 9.3.3.2.2.

NOTE – This procedure may also be implemented using DecodeDecision(ctxIdxTable, ctxIdx) with ctxIdxTable = 0 and ctxIdx = 0. In the case where the decoded value is equal to 1, seven more bits would be read by DecodeDecision(ctxIdxTable, ctxIdx) and a decoding process would have to adjust its bitstream pointer accordingly to properly decode following syntax elements.

Figure 9-6 – Flowchart of decoding a decision before termination

### Arithmetic encoding process (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are decisions that are to be encoded and written.

Outputs of this process are bits that are written to the RBSP.

This informative subclause describes an arithmetic encoding engine that matches the arithmetic decoding engine described in subclause 9.3.3.2. The encoding engine is essentially symmetric with the decoding engine, i.e., procedures are called in the same order. The following procedures are described in this section: InitEncoder, EncodeDecision, EncodeBypass, EncodeTerminate, which correspond to InitDecoder, DecodeDecision, DecodeBypass, and DecodeTerminate, respectively. The state of the arithmetic encoding engine is represented by a value of the variable codILow pointing to the lower end of a sub-interval and a value of the variable codIRange specifying the corresponding range of that sub-interval.

#### Initialization process for the arithmetic encoding engine (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

This process is invoked before encoding the first coding block of a slice, and after encoding any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data for a coding unit of type I_PCM.

Outputs of this process are the values codILow, codIRange, firstBitFlag, bitsOutstanding, and BinCountsInNALunits of the arithmetic encoding engine.

In the initialization procedure of the encoder, codILow is set equal to 0, and codIRange is set equal to 510. Furthermore, firstBitFlag is set equal to 1 and the counter bitsOutstanding is set equal to 0.

Depending on whether the current slice is the first slice of a coded picture, the following applies.

–	If the current slice is the first slice of a coded picture, the counter BinCountsInNALunits is set equal to 0.

–	Otherwise (the current slice is not the first slice of a coded picture), the counter BinCountsInNALunits is not modified. The value of BinCountsInNALunits is the result of encoding all the slices of a coded picture that precede the current slice in decoding order. After initialising for the first slice of a coded picture as specified in this subclause, BinCountsInNALunits is incremented as specified in subclauses 9.3.4.2, 9.3.4.4, and 9.3.4.5.

NOTE – The minimum register precision required for storing the values of the variables codILow and codIRange after invocation of any of the arithmetic encoding processes specified in subclauses 9.3.4.2, 9.3.4.4, and 9.3.4.5 is 10 bits and 9 bits, respectively. The encoding process for a binary decision (EncodeDecision) as specified in subclause 9.3.4.2 and the encoding process for a binary decision before termination (EncodeTerminate) as specified in subclause 9.3.4.5 require a minimum register precision of 10 bits for the variable codILow and a minimum register precision of 9 bits for the variable codIRange. The bypass encoding process for binary decisions (EncodeBypass) as specified in subclause 9.3.4.4 requires a minimum register precision of 11 bits for the variable codILow and a minimum register precision of 9 bits for the variable codIRange. The precision required for the counters bitsOutstanding and BinCountsInNALunits should be sufficiently large to prevent overflow of the related registers. When maxBinCountInSlice denotes the maximum total number of binary decisions to encode in one slice and maxBinCountInPic denotes the maximum total number of binary decisions to encode a picture, the minimum register precision required for the variables bitsOutstanding and BinCountsInNALunits is given by Ceil( Log2( maxBinCountInSlice + 1 ) ) and Ceil( Log2( maxBinCountInPic + 1 ) ), respectively.

#### Encoding process for a binary decision (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are the context index ctxIdx, the value of binVal to be encoded, and the variables codIRange, codILow and BinCountsInNALunits.

Outputs of this process are the variables codIRange, codILow, and BinCountsInNALunits.

Figure 9-7 shows the flowchart for encoding a single decision. In a first step, the variable codIRangeLPS is derived as follows.

Given the current value of codIRange, codIRange is mapped to the index qCodIRangeIdx of a quantized value of codIRange by using Equation 9-30. The value of qCodIRangeIdx and the value of pStateIdx associated with ctxIdx are used to determine the value of the variable rangeTabLPS as specified in Table 9-39, which is assigned to codIRangeLPS. The value of codIRange − codIRangeLPS is assigned to codIRange.

In a second step, the value of binVal is compared to valMPS associated with ctxIdx. When binVal is different from valMPS, codIRange is added to codILow and codIRange is set equal to the value codIRangeLPS. Given the encoded decision, the state transition is performed as specified in subclause 9.3.3.2.1.1. Depending on the current value of codIRange, renormalization is performed as specified in subclause 9.3.4.3. Finally, the variable BinCountsInNALunits is incremented by 1.

Figure 9-7 – Flowchart for encoding a decision

#### Renormalization process in the arithmetic encoding engine (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are the variables codIRange, codILow, firstBitFlag, and bitsOutstanding.

Outputs of this process are zero or more bits written to the RBSP and the updated variables codIRange, codILow, firstBitFlag, and bitsOutstanding.

Renormalization is illustrated in Figure 9-8.

Figure 9-8 – Flowchart of renormalization in the encoder

The PutBit( ) procedure described in Figure 9-9 provides carry over control. It uses the function WriteBits( B, N ) that writes N bits with value B to the bitstream and advances the bitstream pointer by N bit positions. This function assumes the existence of a bitstream pointer with an indication of the position of the next bit to be written to the bitstream by the encoding process.

Figure 9-9 – Flowchart of PutBit(B)

#### Bypass encoding process for binary decisions (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are the variables binVal, codILow, codIRange, bitsOutstanding, and BinCountsInNALunits.

Output of this process is a bit written to the RBSP and the updated variables codILow, bitsOutstanding, and BinCountsInNALunits.

This encoding process applies to all binary decisions with bypassFlag equal to 1. Renormalization is included in the specification of this process as given in Figure 9-10.

Figure 9-10 – Flowchart of encoding bypass

#### Encoding process for a binary decision before termination (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are the variables binVal, codIRange, codILow, bitsOutstanding, and BinCountsInNALunits.

Outputs of this process are zero or more bits written to the RBSP and the updated variables codILow, codIRange, bitsOutstanding, and BinCountsInNALunits.

This encoding routine shown in Figure 9-11 applies to encoding of the end_of_slice_flag and pcm_flag indicating the I_PCM mode both associated with ctxIdx equal to 276.

Figure 9-11 – Flowchart of encoding a decision before termination

When the value of binVal to encode is equal to 1, CABAC encoding is terminated and the flushing procedure shown in Figure 9-12 is applied. In this flushing procedure, the last bit written by WriteBits( B, N )  is equal to 1. When encoding end_of_slice_flag, this last bit is interpreted as the rbsp_stop_one_bit.

Figure 9-12 – Flowchart of flushing at termination

#### Byte stuffing process (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

This process is invoked after encoding the last coding block of the last slice of a picture and after encapsulation.

Inputs to this process are the number of bytes NumBytesInVclNALunits of all VCL NAL units of a picture, the number of minimum CUs PicSizeInMinCbs in the picture, and the number of binary symbols BinCountsInNALunits resulting from encoding the contents of all VCL NAL units of the picture.

NOTE – The value of BinCountsInNALunits is the result of encoding all slices of a coded picture. After initialising for the first slice of a coded picture as specified in subclause 9.3.4.1, BinCountsInNALunits is incremented as specified in subclauses 9.3.4.2, 9.3.4.4, and 9.3.4.5.

Outputs of this process are zero or more bytes appended to the NAL unit.

Let the variable k be set equal to Ceil( ( Ceil( 3 * ( 32 * BinCountsInNALunits − RawMinCUBits * PicSizeInMinCbs ) ÷ 1024 ) − NumBytesInVclNALunits ) ÷ 3 ). Depending on the variable k the following applies.

–	If k is less than or equal to 0, no cabac_zero_word is appended to the NAL unit.

–	Otherwise (k is greater than 0), the 3-byte sequence 0x000003 is appended k times to the NAL unit after encapsulation, where the first two bytes 0x0000 represent a cabac_zero_word and the third byte 0x03 represents an emulation_prevention_three_byte.