| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0281 |
| --- | --- |

| Title: | Additional horizontal and vertical scan for transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two proposals to add horizontal and vertical scans to HM7 to improve coding efficiency. In the proposal A, horizontal and vertical scan is added to the coding of 8x8 chroma transform coefficients. This required one character change in the HM7 source code, and it resulted in -0.2% to intra main chroma BDR and -0.1% to intra HE10 chroma BDR. In proposal B, the horizontal and vertical scan is added to all transform sizes. It resulted in BDR of -0.1% for luma intra main and luma intra HE10. It also resulted in BDR or -0.3% to -0.4% for chroma intra main and chroma intra HE10.

# Introduction

In HM7, horizontal and vertical scan are not applicable to 16x16 and 32x32 intra blocks. They were also not applicable to 8x8 or larger chroma blocks. One of the reasons is that the inclusion of horizontal scan and vertical scan in HM6 are in conflict with parallel context selection of significant_coeff_flag by using the template based method. This situation is changed in HM7, the template based method for the context selection of significant_coeff_flag was replaced by position based method [3]. Parallel context selection of significant_coeff_flag is no longer an issue with horizontal and vertical scan.

As shown in Table 1, HM7 has three scanning pattern for the coefficients of square transform block. It has diagonal scan (D), horizontal scan (H), and vertical scan (V). The horizontal and vertical scans are not applicable to 8x8 chroma blocks. They are also not applicable to luma and chroma blocks with block size larger than 8x8.

This contribution consists of two proposals to improve coding efficiency by means of additional horizontal and vertical scan. In proposal A, the horizontal and vertical scans are added to 8x8 chroma block. In proposal B, the horizontal and vertical scans are extended to cover all square block sizes of luma and chroma transform coefficients.

|  | Transform Size |  |  |  |
| --- | --- | --- | --- | --- |
| Color | 4x4 | 8x8 | 16x16 | 32x32 |
| Luma | H, V, D | H, V, D | D | D |
| Chroma | H, V, D | D | D | D |

Table 1: Scan patterns in HM7

# Proposal A

The proposal A proposes to add horizontal scan (H) and vertical scan (V) to chroma 8x8 transform block as shown in Table 2. This requires one line change in the WD7 and a one character change in the HM7 source code. As a result, the same mode dependent coefficient scan for luma intra 8x8 transform blocks is applied to the chroma intra 8x8 transform block.

The proposal A was integrated in HM7 and tested according to common test conditions [1]. As shown in Table 3, it resulted in chroma BDR of -0.2% for Intra Main and -0.1% for Intra HE10.

|  | Transform Size |  |  |  |
| --- | --- | --- | --- | --- |
| Color | 4x4 | 8x8 | 16x16 | 32x32 |
| Luma | H, V, D | H, V, D | D | D |
| Chroma | H, V, D | H, V, D | D | D |

Table 2: Proposal A added horizontal and vertical scan to chroma 8x8 block.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.23% | -0.23% | -0.01% | -0.11% | -0.14% |
| Class B | 0.00% | -0.35% | -0.40% | 0.00% | -0.23% | -0.26% |
| Class C | -0.01% | -0.18% | -0.17% | 0.00% | -0.08% | -0.08% |
| Class D | 0.00% | -0.12% | -0.13% | 0.00% | -0.10% | -0.11% |
| Class E | 0.00% | -0.23% | -0.16% | 0.00% | -0.13% | -0.08% |
| Overall | 0.00% | -0.23% | -0.23% | 0.00% | -0.14% | -0.14% |
|  | 0.00% | -0.23% | -0.22% | 0.00% | -0.14% | -0.14% |
| Class F | 0.00% | -0.15% | -0.06% | -0.03% | -0.10% | -0.04% |
| Enc Time[%] | 100.37% |  |  | 100.25% |  |  |
| Dec Time[%] | 100.25% |  |  | 99.55% |  |  |

Table 3: BDR of proposal A.

# Proposal B

The proposal B proposes to extend horizontal scan (H) and vertical scan (V) to cover all transform block size as shown in Table 4. Since the horizontal scan and the vertical scan pattern is already defined in HM7 and WD7, this proposal requires the following changes in HM7 and WD7:

- Extend the mode dependent coefficient scan (MDCS) to cover all square transform sizes.

- Added horizontal and vertical scan coefficient groups to 16x16 and 32x32 transform coefficients. Select horizontal/vertical/diagonal scan coefficient group as defined by MDCS.

- Added context selection of the significant_coeff_flag coding in horizontal and vertical coefficient groups.

|  | Transform Size |  |  |  |
| --- | --- | --- | --- | --- |
| Color | 4x4 | 8x8 | 16x16 | 32x32 |
| Luma | H, V, D | H, V, D | H, V, D | H, V, D |
| Chroma | H, V, D | H, V, D | H, V, D | H, V, D |

Table 4: Proposal B added horizontal and vertical scan to cover all square transform sizes.

## Mode dependent coefficient scan

In proposal B, mode dependent coefficient scan is applied to transform coefficients of all square transform sizes. In particular, proposal B modified the MDCS lookup table for the intra coded square transform blocks as in Table 5 for luma and Table 6 for chroma. There is no change in MDCS for non-intra coded blocks with respect to HM7.

| Luma ScanType | IntraPredMode range for 4x4 and 8x8 luma transform block | IntraPredMode range for 16x16 and 32x32 luma transform block |
| --- | --- | --- |
| Horizontal (H) | ¦ IntraPredMode – 26 ¦ < 5 | ¦ IntraPredMode – 26 ¦ < 2 |
| Vertical (V) | ¦ IntraPredMode – 10 ¦ < 5 | ¦ IntraPredMode – 10 ¦ < 2 |
| Diagional (D) | otherwise | otherwise |

Table 5: Luma ScanType is selected based on the size of the luma transform block and the range that the luma IntraPredMode is located.

| Chroma ScanType | IntraPredMode range for 4x4 chroma transform block | IntraPredMode range for 8x8 and 16x16 chroma transform block |
| --- | --- | --- |
| Horizontal (H) | ¦ IntraPredMode – 26 ¦ < 5 | ¦ IntraPredMode – 26 ¦ < 2 |
| Vertical (V) | ¦ IntraPredMode – 10 ¦ < 5 | ¦ IntraPredMode – 10 ¦ < 2 |
| Diagional (D) | otherwise | otherwise |

Table 6: Chroma ScanType is selected based on the size of the chroma transform block and the range that the chroma IntraPredMode is located.

## Horizontal and Vertical Coefficient Group

In the residual coding of HM7, the transform coefficients are encoded in a group of 16 consecutive transform coefficients in the mode dependent coefficient scan order. Figure 2 and Figure 3 shows the arrangement of the horizontal coefficient group and vertical coefficient group with respect to the arrangement of coefficients in the square transforms in Figure 1.

Figure 1: Square transform sizes in HM7.

Figure 2: Horizontal coefficient groups for horizontal scan.

Figure 3: Vertical coefficient groups for vertical scan.

## Context selection of significant_coeff_flag

In HM7, the context selection of a significant_coeff_flag in a coefficient group depends on the position of the flag in its coefficient group and the significance of the right neighboring coefficient group and the significance of the bottom neighboring coefficient groups. If the neighboring coefficient group is not available, it is assumed to be not significant and it is not coded with any transform coefficient.

Figure 4 and Figure 5 shows the context index increment of a coefficient within a coefficient group in condition on the significance of the two adjacent coefficient groups.

Similar to context index increment of a diagonally scan coefficient group, the context index increment has a value of 0, 1, and 2. The value of the context index increment determines the context needed for CABAC as defined in HM7. For example, if a coefficient belongs to a horizontal coefficient group with no coded coefficient in the two adjacent coefficient groups, then the context index increment is classify as Case 0 in Figure 4. Furthermore, if the coefficient lies in the sub region labeled 1 in Case 0, the coefficient shall be assigned with a context index increment of 1.

Figure 4: Four cases of context index increment assignment for horizontal coefficient groups. Context index increment of a coefficient in a horizontal coefficient group in green depends on its position in is coefficient group and the significance of the two adjacent coefficient group in blue.

Figure 5: Four cases of context index increment assignment for vertical coefficient groups. Context index increment of a coefficient in a vertical coefficient group in green depends on its position in is coefficient group and the significance of the two adjacent coefficient group in blue.

## Results

The proposal B was integrated in HM7 and it was tested according to common test conditions [1]. As shown in Table 7, it resulted in luma BDR of -0.1 and chroma BDR of -0.3% to -0.4% for Intra Main and Intra HE10.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.03% | -0.22% | -0.22% | 0.02% | -0.11% | -0.21% |
| Class B | -0.11% | -0.72% | -0.77% | -0.13% | -0.66% | -0.69% |
| Class C | -0.07% | -0.27% | -0.24% | -0.08% | -0.22% | -0.22% |
| Class D | 0.01% | -0.25% | -0.18% | 0.01% | -0.26% | -0.20% |
| Class E | -0.14% | -0.50% | -0.50% | -0.13% | -0.45% | -0.46% |
| Overall | -0.06% | -0.40% | -0.40% | -0.06% | -0.35% | -0.37% |
|  | -0.06% | -0.41% | -0.40% | -0.06% | -0.35% | -0.36% |
| Class F | -0.17% | -0.34% | -0.29% | -0.20% | -0.46% | -0.17% |
| Enc Time[%] | 100.49% |  |  | 100.29% |  |  |
| Dec Time[%] | 102.27% |  |  | 100.72% |  |  |

Table 7: BDR of proposal B.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d4, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Toru Fumakura and Shigeru Fukushima, “Non-CE3: Simplified context derivation for significance map”, JCTVC-I0296, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposal A WD text

The proposed WD is based on JCTVC-I003_d4 [2]. Only the differences from JCTVC-I003_d4 is reported here. The modifications are highlighted in yellow.

7.3.9 Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ max(0,log2TrafoSize – 3) ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |

# Proposal B WD text

The proposed WD is based on JCTVC-I003_d4 [2]. Only the differences from JCTVC-I003_d4 is reported here. The modifications are highlighted in yellow.

7.3.9 Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ 1 + max(0,log2TrafoSize – 3) ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |

7.3.10 	Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = log2TrafoHeight && log2TrafoWidth < 5 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = log2TrafoHeight && log2TrafoWidth < 5) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else if( scanIdx = = 1 && log2TrafoWidth = = log2TrafoHeight && log2TrafoWidth = = 5 ) { |  |
| xCG = i % 2 |  |
| yCG = i >> 1 |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = log2TrafoHeight && log2TrafoWidth = = 5) { |  |
| xCG = i >> 1 |  |
| yCG = i % 2 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

7.4.9 	Transform coefficient semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes,  log2TrafoSize,  and luma intra prediction modes, IntraPredMode, is derived as specified in Table 7-8.

The value of ScanType[ 1 + max(0, log2TrafoSize – 3) ][ IntraPredModeC ] also specifies the chroma scanning order for the chroma intra prediction modes,  IntraPredModeC, and the chroma transform block size corresponding to the luma transform block size log2TraFoSize.

Table 7-8 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 8 | 2 | 2 | 0 | 0 |
| 9 – 11 | 2 | 2 | 2 | 2 |
| 12 – 14 | 2 | 2 | 0 | 0 |
| 15 – 21 | 0 | 0 | 0 | 0 |
| 22 – 24 | 1 | 1 | 0 | 0 |
| 25 – 27 | 1 | 1 | 1 | 1 |
| 28 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

7.4.10 Residual coding semantics

significant_coeff_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero transform coefficient level. A coefficient group at location ( xCG, yCG ) is an array of 16 transform coefficient levels at locations ( xC, yC ).

- If significant_coeff_group_flag[ xCG ][ yCG ] is equal to 0, the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) are inferred to be equal to 0;

- Otherwise (significant_coeff_group_flag[ xCG ][ yCG ] is equal to 1), the following applies.

- If significant_coeff_group_flag[ xCG ][ yCG ] is present or if one of the following condition is true

  - scanIdx is equal to 0 and ( xCG << 2, yCG << 2 ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ),

  - scanIdx is equal to 1 and log2TrafoWidth is equal to 3 and ( xCG , yCG<<1 ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ),

  - scanIdx is equal to 1 and log2TrafoWidth is larger than 3 and ( xCG << 4, yCG ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ),

  - scanIdx is equal to 2 and log2TrafoWidth is equal to 3 and ( xCG << 1 , yCG  ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ),

  - scanIdx is equal to 2 and log2TrafoWidth is larger than 3 and ( xCG , yCG << 4 ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ),

at least one of the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) has a non zero value.

- Otherwise, at least one of the 16 significant_coeff_flag syntax elements is present for the coefficient group at location ( xCG, yCG )

When significant_coeff_group_flag[ xCG ][ yCG ] is not present, it is inferred as follows.

- If one or more of the following conditions are true, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 1.

- scanIdx is equal to 0 and ( xCG, yCG ) is equal to the last coefficient group position ( LastSignificantCoeffX >> 2, LastSignificantCoeffY >> 2 )

- scanIdx is equal to 1 and log2TrafoWidth is equal to 3 and ( xCG, yCG ) is equal to the last coefficient group position ( 0, LastSignificantCoeffY >> 1  )

- scanIdx is equal to 1 and log2TrafoWidth is larger than 3 and ( xCG, yCG ) is equal to the last coefficient group position ( LastSignificantCoeffX >> 4, LastSignificantCoeffY  )

- scanIdx is equal to 2 and log2TrafoWidth is equal to 3 and ( xCG, yCG ) is equal to the last coefficient group position  ( LastSignificantCoeffX >> 1, 0 )

- scanIdx is equal to 2 and log2TrafoWidth is larger than 3 and ( xCG, yCG ) is equal to the last coefficient group position  ( LastSignificantCoeffX , LastSignificantCoeffY >> 4 )

- ( xCG, yCG ) is equal to ( 0, 0 )

- Otherwise, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 0.

9.2.3.1.4 	Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

The JCTVC-I1003_d4  WD text  was not updated to include the corresponding decoding process in HM7. This proposed WD text is based on the WD text from JCTVC-I0296-v5. The modification are highlighted in yellow.

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the previously decoded bins of the syntax element significant_coeff_group_flag, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight and the scanning order scanIdx. The array index scanIdx equal to 0 specifies an up-right diagonal scan, scanIdx equal to 1 specifies a horizontal scan, and scanIdx equal to 2 specifies a vertical scan.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_group. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Table 9-9 as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Table 9-10 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise, sigCtx is derived using previously decoded bins of the syntax element significant_coeff_group_flag and scanIdx as follows.

- The variable xCG is derived as follows.

xCG  =  (scanIdx==0)  ? xC >> 2 : (scanIdx==1) ? xC >> 4 : xC ;	(9-21)

- The variable yCG is derived as follows.

yCG  =  (scanIdx==0)  ? yC >> 2 : scanIdx==1) ? yC  : yC >> 4 ;	(9-22)

- The variable cntCG is initialized as follows.

cntCG  =  0		(9-23)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-24)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG ][ yCG + 1 ] << 1	(9-25)

- If cntCG is equal to 0, the the following applies.

  - If scanIdex is equal to 0,

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 2 ? 1 : 0	(9-26a)

  - Else if scanIdex is equal to 1,

sigCtx  =   ( xC - (xCG << 4) ) < 4 ? 1 : 0	(9-26b)

  - Otherwise

sigCtx  =   ( yC - (yCG << 4) ) < 4 ? 1 : 0	(9-26c)

- Otherwise, if cntCG is equal to 1, the following applies.

  - If scanIdex is equal to 0,

sigCtx  =   ( yC - (yCG << 2) ) <= 1 ? 1 : 0	                                                                          (9-27a)

  - Else if scanIdex is equal to 1,

sigCtx  =   ( xC - (xCG << 4) ) < 8 ? 1 : 0	                                                                          (9-27a)

  - Otherwise,

sigCtx  =   ( yC - (yCG << 4) ) < 8 ? 1 : 0                                                                   (9-27a)

- Otherwise, if cntCG is equal to 2, the following applies.

  - If scanIdex is equal to 0,

sigCtx  =   ( xC - (xCG << 2) ) <= 1 ? 1 : 0	(9-28a)

  - Else if scanIdex is equal to 1,

sigCtx  =   ( xC - (xCG << 4) ) < 4 ? 2 : 1	(9-28b)

  - Otherwise,

sigCtx  =   ( yC - (yCG << 4) ) < 4 ? 2 : 1	(9-28c)

- Otherwise ( cntCG is equal to 3 ), the following applies.

  - If scanIdex is equal to 0,

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 4 ? 2 : 1    	(9-29a)

  - Else if scanIdex is equal to 1,

sigCtx  =   ( xC - (xCG << 4) ) < 8 ? 2 : 1   	(9-29b)

  - Otherwise,

sigCtx  =   ( yC - (yCG << 4) ) < 8 ? 2 : 1    	(9-29c)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = sigCtx + 24		(9-30)

  - Otherwise, the following applies.

sigCtx = sigCtx+ ( (cIdx > 0) ? 18 : 21 )	(9-31)

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-32)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-33)

Table 9-9 – Specifcation of ctxIdxMap4x4[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap4x4[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |
| i | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| ctxIdxMap4x4[ i ] | 0 | 1 | 2 | 4 | 1 | 1 | 2 | 4 | 3 | 3 | 5 | 5 | 4 | 4 | 5 |

Table 9-10 – Specifcation of ctxIdxMap8x8[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap8x8[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 8 | 6 | 6 | 7 | 9 | 9 | 7 | 7 |