| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0282 |
| --- | --- |

| Title: | AHG4: On mandatory parallelism |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Worrall, A. Wise Aspex Semiconductor, Rapid House, 40 Oxford Road, High Wycombe, HP11 2EE | Tel: Email: | +44 1494 558121 stewart.worrall@aspex-semi.com |
|  | J. Viéron ATEME, 26 Burospace, Route de Gisy, 91570 Bièvres, France | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com |
|  | P. Andrivon P. Bordes Technicolor, 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33-2-99-27-38-07 pierre.andrivon@technicolor.com philippe.bordes@technicolor.com |
| Source: | Aspex Semiconductor, ATEME, Technicolor |  |  |

_____________________________

# Abstract

A number of contributions were submitted to the 9th JCT meeting, which argued for a mandatory minimum number of tiles to be included for a particular level. Authors recommend that a minimum number of tiles or sub-pictures are mandated only when encoding is performed using tiles. It is also recommended that no constraints should be placed on maximum slice size. Finally, the authors advocate that the standard should not mandate parallelism usage for any level.

# Introduction

A number of proposals were presented at the 9th JCT meeting, which proposed tile constraints linked to levels [1][2][3]. The main arguments made for introducing such constraints are as follows:

- Enabling decoder parallelism [1]

- Reducing time/effort needed for hardware verification of parallel decoders [1]

- Reducing line buffer size requirements [1][2][3]

Most of these proposals seem only to argue for such restrictions when tiles are used. However, JCTVC-I0118 argues for the use of sub-pictures, where WPP or tiles can be used in sub-pictures. This proposal would certainly have the first two benefits listed above. However, JCTVC-I0118 also mandates prediction breaks for pictures at resolutions greater than 1080p. We will argue in the following sections that this is undesirable.

# Prediction Breaks in HEVC

The deblocking filter in HEVC is highly effective in removing many of the artefacts caused by prediction breaks at tile boundaries. However, it is almost impossible to design a filter that can accurately detect and remove boundary artefacts when pixel magnitude differences are small (e.g. 1-2 pixel magnitude luma difference). Although such differences might seem small, the effects can be amplified by the gamma correction curves of some display devices. The visibility of such artefacts will therefore depend upon the characteristics of the display, and the nature of the content. Content with slowly varying gradients (e.g. blue sky) is therefore likely to suffer the most from perceptual issues where there are prediction breaks.

Some of the tile boundary artefacts are similar to the artefacts generated by slices. Therefore, a constraint on the minimum number of slices per picture is also not desirable for the HEVC Main Profile. No constraints are currently imposed by the Main Profile, and we believe this situation should be maintained.

A further point to note is that, generally speaking, vertical boundary artefacts tend to be more visually significant than horizontal boundary artefacts for the Human Visual System. HEVC applications may mandate constraints that enable a certain degree of parallelism in order to help technologies deployment.For instance, Blu-ray Disc Association (BDA) mandates four slices encoding at least per picture in Blu-ray video specification [4]. The authors believe it is important that the HEVC standard avoids mandating prediction breaks in the level and profile definitions i.e. imposing parallelism constraints for every upcoming application of HEVC.

# Implementation Issues

In AVC, restrictions on the maximum number of macroblocks in a slice were introduced for some profiles. The intention of this was to make implementation of decoders easier. However, it can be argued that such restrictions are no longer necessary for AVC (or HEVC). Building an AVC decoder capable of handling single slice pictures is no longer a serious issue for many application domains.

It can also be argued that building an HEVC decoder is not as serious a challenge in 2012 as building an AVC decoder in 2003. Given expected improvements in hardware performance in the coming years, we do not see a need for maximum slice size constraints that can be justified by implementation concerns.

Finally, it has been argued that some architectures will require parallel decoding features. However, we believe this should be achieved by limiting the imposition of prediction breaks to the case where tiles or independent slices are used. This will allow encoders to potentially produce bitstreams without prediction breaks.

# Conclusions

The authors recommend that no mandatory parallelization specifications are included in the draft standard, which require encoding using a particular parallelization tool. In other words, the Main Profile should allow the syntax element tiles_or_entropy_coding_sync to be set to 0. The profile specification discussed at the 9th JCT meeting is satisfactory in that respect, as it allows encoders to produce bitstreams without prediction breaks.

We expect the issue of prediction breaks to become a more significant issue as displays improve in quality (4k, wide colour gamut and 10-bit panels…). Implementation issues driving calls for mandatory parallelism constraints should not be considered in the HEVC standard but addressed by third party organisms in charge of deploying the technology for a particular application, as those issues will become even less significant during the lifetime of HEVC.

# References

- M. Zhou, JCTVC-I0118, “AHG4: Enable parallel decoding with tiles”, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- A. Norkin, R. Sjöberg, JCTVC-I0255, “Level restrictions on tiles for line buffer reduction”, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- S. Kumar, G. Van der Auwera, M. Coban, Y.-K. Wang, M. Karczewicz, JCTVC-I0362, “AHG4: Virtual line buffer model and restriction on asymmetric tile configuration”, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- Blu-ray Disc Association, System Description Blu-ray Disc Read-Only, Part 3: Audio Visual Basic Specifications, version 2.5 for 2D, June 2011.

# Patent rights declarations

Aspex Semiconductor does not have any current or pending patent rights relating to the technology described in this contribution.

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.