| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11– 20 July 2012 | Document: JCTVC-J0284 |
| --- | --- |

| Title: | Tiles and Wavefront Parallel Processing Restrictions For The Main Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Worrall, A. Wise Rapid House, 40 Oxford Road, High Wycombe, HP11 2EE | Tel: Email: | +44 1494 558121 stewart.worrall@aspex-semi.com |
| Source: | Aspex Semiconductor |  |  |

_____________________________

# Abstract

This contribution proposes some restrictions for the Main Profile of HEVC. The author argues that the proposed restrictions will reduce the conformance checking burden for implementers, and make parallel decoding easier to achieve. It is proposed that two different sets of restrictions be included in the Main Profile for Wavefront Parallel Processing (WPP) and tiles respectively. The proposed restrictions are independent of level, although restricting the use of parallel processing tools to levels greater than 3.1 is recommended. A spreadsheet has been provided to demonstrate the tile configurations specified by the recommended constraints.

Version information:

- Version 2: added excel spreadsheet with tile constraint calculation example

- Version 3: minor correction in main document, included proposed text for draft

# Introduction

At the 9th JCT-VC meeting, it was decided to include three possible parallel processing configurations: no parallel processing, WPP, and tiles. The concept of “opportunistic” parallel decoding was discussed, where decoders try and take advantage of parallelism wherever possible. As it stands, there are a wide variety of possible parallel processing configurations available, specifically for tiles. This contribution proposes restrictions on the parallel processing options. There are two goals:

- Simplification of decoder implementation, by restricting the number of parallel processing configurations that need to be supported.

- Making parallel decoding easier to achieve, by restricting the kinds of bitstream that a decoder might expect to receive.

# Options for Parallel Processing Tools Restrictions

For WPP, the restrictions are straightforward, and are described in section 3.

For tiles, a number of restrictions were proposed during the 9th JCT meeting. I0118 presents the concept of sub-pictures, which includes some level based restrictions to allow a 4K decoder to be constructed from four 1080p decoders [1]. During the meeting, some discussions were held as to whether the same concept could be achieved using restrictions on tile boundaries, which would prevent the need to introduce an additional parallel processing concept. I0462 proposes level independent restrictions on tile sizes, where tile sizes are made to be small enough to allow efficient encoding and decoding with different numbers of cores [2]. It should be noted that the restrictions on I0462 can also be used to build a 4K decoder from four ~1080p decoders, provided tiles are used.

# Restrictions for WPP

At the 9th JCT-VC meeting, a decision was taken to use one substream per Code Tree Block (CTB) row. The only necessary restriction required to enable parallel decoding of WPP bitstreams is to enforce the inclusion of entry point indicators for every substream. We also proposed an additional restriction: WPP shall only be used for Levels higher than or equal to 3.1. This is because parallel processing is unlikely to be required for levels less than 3.1.

# Restrictions for Tiles

## Tile Restriction Issues

The existing restrictions for tiles make parallel decoding impractical, and decoder implementation challenging. For 1080p content, a decoder can expect to receive anything in the range 1-340 tiles in a picture. Tighter restrictions need to be considered. However, when choosing the restrictions, it is necessary to consider a number of potential problems:

- MTU size matching

- Parallel encoding/decoding flexibility

- Compression efficiency

- Decoder conformance testing

- Level based settings

- Line buffer issues

MTU Size Matching Issues

To match MTU sizes, tiles can be grouped together within a slice, or multiple slices can be placed within a tile. However, MTU size matching is difficult to do if tile sizes are not carefully chosen. Depending on the QP used, larger tiles require that slices are used within tiles, while smaller tiles mean that tile grouping is the most effective option. To prevent fragmentation of NALs into separate IP packets, it may be necessary to use a tile size estimation algorithm if some flexibility in tile size choice is not allowed.

Parallel Encoding/Decoding Flexibility

The best encoder/decoder parallel processing flexibility can be achieved by using small tiles. This enables encoders and decoders to group tiles together for processing. Providing more tiles allows encoders and decoders to respectively exploit different, arbitrary numbers of cores, in an efficient manner. Figure 1 shows an example of how nine tiles can be mapped to four cores. These nine tiles can also be mapped to two, three, five, and nine cores. Mapping tiles to an arbitrary number of cores is discussed and evaluated in more detail in JCTVC-I0153 [5].

Figure 1: Example of how nine tiles may be mapped for processing on four cores.

Compression Efficiency Considerations

Using very small tiles degrades compression efficiency significantly. However, it may also grant encoders more cycles per CTB, allowing them to perform better RD optimisation. Some flexibility is needed to allow a good trade-off.

Decoder Conformance Testing

Restricting the possible tile configurations has the significant advantage of cutting down on the tool combinations that need to be checked during conformance testing. This will make it easier to produce reliable compliant decoders.

Level Based Settings

It is preferable to avoid setting minimum tile sizes/numbers with direct links to levels, as the level settings specify maximum values. If implementers choose to work around the profiles and levels, then unexpected (but still legal) combinations of resolution and tile numbers may be encountered.

Line Buffer Issues

It has been stated in other contributions that line buffer sizes can be reduced by constraining tiles [3] [4]. However, this will not be considered in this contribution because it is highly implementation dependent. In addition, decoder implementers will have to build for “non-tiles” encoded bitstreams, meaning that any potential savings may not be realised.

Summary of Issues

Many of the problems can be minimised using small tiles. However, MTU size matching, and RD optimisation will work best if a degree of flexibility in tile size is allowed. It is also good to allow some variation to allow more efficient mapping of tile numbers to the number of cores available to an encoder or decoder. Tiles should not be specified to be too small however, as this will degrade compression efficiency significantly. To make conformance testing easier, evenly spaced tiles should be used, but some limited variability in tile size should be allowed.

## Proposed Tile Restrictions

In addition to using evenly spaced tiles, the proposed restrictions for the Main Profile are as follows:

ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

num_tile_columns_minus1 = PicWidthInCtbs/(6 * 26-Log2CtbSize )  - 1

The above restrictions constrain the tile width, now we will constrain the tile height to get a similar number of tiles compared to the number of 64 CTB rows in a picture, while permitting some variation in the number of tiles. The number of tiles that this generates will allow the encoder and decoder to decode with different numbers of tiles with reasonable parallel processing efficiency, as discussed in [5].

BaseTileRows shall be derived as follows:

TileConstraintA = Ceil(PicHeightInCtbs/26-Log2CtbSize / (num_tile_columns_minus1 + 1))

TileConstraintB = Floor( PicHeightInCtbs / ( 26-Log2CtbSize * 3 ) )

BaseTileRows = Min( TileConstraintA, TileConstraintB )

The following constraints shall be obeyed:

BaseTileRows - 2 < num_tile_rows_minus1 < BaseTileRows + 2

This has the effect of allowing three different tile configurations for any particular picture resolution.

In addition to these level independent constraints, the following constraint should be set: tiles shall only be used for Levels higher than or equal to 3.1.

Example tile column and rows specified by these constraints are shown in Table 1.

Table 1: Example tile columns and rows specified by the proposed restrictions. The number of tiles given by the three different configurations are also shown.

| Class | LumaWidth | LumaHeight | num_tile_columns | BaseTileRows | Min Num Tiles | Med Num Tiles | Max Num Tiles |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 3840 | 2160 | 10 | 4 | 30 | 40 | 50 |
| A | 2560 | 1600 | 6 | 4 | 24 | 30 | 36 |
| B | 1920 | 1080 | 5 | 4 | 15 | 20 | 25 |
| C | 832 | 480 | 2 | 2 | 2 | 4 | 6 |
| E | 1280 | 720 | 3 | 4 | 9 | 12 | 15 |

# Conclusion/Summary

The compromise reached during the 9th JCT-VC meeting to include both WPP and tiles in the Main Profile, makes practical parallel decoding difficult. If it is not possible to have a single parallel processing option always enabled, then restrictions are required on tiles and WPP to make “opportunistic” parallel decoding more practical. Restrictions are also necessary to make it easier and cheaper to implement reliable, conforming decoders.

For WPP, most of the required restrictions are already present.

For tiles, choosing a set of constraints is challenging, as there are a number of conflicting constraints. After considering those constraints, some level independent specifications for tile sizes are included, with some flexibility to allow better trade-offs for MTU size matching and RD optimisation. Readers should also refer to the accompanying spreadsheet, which demonstrates the effects of the tile constraints for a user defined picture resolution. Note that the proposed restrictions have the additional benefit that the worst case number of tiles is restricted compared to the existing working draft (e.g. 340 for Class B, 600 for Class A).

Note that the compression efficiency of HM using similar numbers of tiles has been evaluated in [5], [6], and [7]. Although these results were obtained with previous versions of HM, the results should still be representative, as the fundamentals behind tiles have not changed.

# References

- M. Zhou, “AHG4: Enable parallel decoding with tiles”, JCTVC-I0118, JCT-VC 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- S. Worrall, “AHG4: Profile Requirements For Facilitation Of Parallel Tile Decoding”, JCTVC-I0462, JCT-VC 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- A. Norkin, R. Sjöberg, “Level restrictions on tiles for line buffer reduction”, JCTVC-I0255, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- S. Kumar, G. Van der Auwera, M. Coban, Y.-K. Wang, M. Karczewicz, “AHG4: Virtual line buffer model and restriction on asymmetric tile configuration”, JCTVC-I0362, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- S. Worrall, “AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile”, JCTVC-I0153, JCT-VC 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- V. Jérôme, T. Jean-Marc, “On Tiles and Wavefront tools for parallelism”, JCT-VC-I0198, JCT-VC 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- Fuldseth et al., “Tiles”, JCTVC-F335, JCT-VC 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

Aspex Semiconductor does not have any current or pending patent rights relating to the technology described in this contribution.