| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0285 |
| --- | --- |

| Title: | Early by-pass mode switch of greater1_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Wei-Jung Chien, Joel Sole, Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal targets improving CABAC throughput by reducing context-coded bins for coefficients level coding, specifically greater1_flag. In HM7.0, maximum eight greater1_flags are context-coded in one coefficient group before switching to by-pass mode. This contribution proposes stopping context-coded greater1_flag after the occurrence of one or two greater1_flags equal to 1 in a coefficient group. The maximum context-coded greater1_flag is kept as eight in the proposal.

Simulation results of method 1 (stopping context-coded greater1_flag after the occurrence of two greater1_flags equal to 1) shows average 0.0%, 0.0%, 0.0%, 0.0%, -0.1%, 0.0% BD rate impact, respectively for AI-Main, AI-HE10, RA-Main, RA-HE10, and LD-Main, LD-HE10 of common test condition. The average BD rate impact is 0.0%, -0.1%, -0.1%, 0.0%, 0.0%, 0.0% in low QP test condition (QP=1, 5, 9, 13).

Simulation results of method 2 (stopping context-codec greater1_flag after the occurrence of one greater1_flag equals to 1) shows average 0.0%, 0.0%, 0.1%, 0.1%, 0.1%, 0.1% BD rate impact, respectively for AI-Main, AI-HE10, RA-Main, RA-HE10, and LD-Main, LD-HE10 of common test condition. The average BD rate impact is 0.0%, 0.0%, 0.2%, 0.3%, 0.2%, 0.4 % in low QP test condition.

# Introduction

In HEVC test model, quantized coefficients are coded based on the unit of coefficient group (CG) which consists of 16 coefficients in the inverse scan order. Five syntax elements significant_coeff_flag, coeff_abs_level_greater1_flag (greater1_flag), coeff_abs_level_greater2_flag (greater2_flag), coeff_sign_flag and coeff_abs_level_remaining are sequentially coded in each CG. The method proposed in [1] was adopted to reduce the context-coded bins of coefficients level information by restricting maximum greater1_flag number to 8 and maximum greater2_flag number to 1 in one CG. This contribution proposes a method to further reduce the context-coded bins of greater1_flag.

# Proposed method

In this proposal, beside the maximum number restriction of greater1_flags in one CG, we make an early switch to by-pass mode when one or two greater1_flag equal to 1 is encountered. The rationale for the early switch method is that after coding each coefficient with greater1_flag = 1 the probability of the remaining coefficients levels to be greater than 1 substantially increases.  Hence the explicitly context-coded great1-flag would not improve the compression efficiency for those coefficients.

Three methods are tested in this proposal.

- Method 1: switch off context-coded greater1_flag coding after the occurrence of two greater1_flags equal to 1.

- Method 2: switch off context-coded greater1_flag coding after the occurrence of one greater1_flag equals to 1.

# Simulation results

The proposed method is implemented on HM7.0 and simulated under the common test conditions [2]. The performance is compared to HM7.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate. Additionally, simulations for low QP conditions (QP=1, 5, 9, 13) are performed.

Tab.1 summarizes the results of the proposed method1 in common test condition and low QP test condition.

Tab.2 summarizes the results of the proposed method2 in common test condition and low QP test condition.

Tab.1 BD-Rate of proposed method 1

| Test configuration | Method 1 (Normal QP) |  |  | Method 1 (Low QP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| Main_Intra | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Main_RA | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% |
| Main_LD | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| HE10_Intra | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| HE10_RA | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| HE10_LD | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Average | -0.03% | 0.01% | -0.01% | -0.02% | 0.04% | 0.04% |

Tab.1 BD-Rate of proposed method 2

| Test configuration | Method 2 (Normal QP) |  |  | Method 2 (Low QP) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |
|  | Y | Cb | Cr | Y | Cb | Cr |
| Main_Intra | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Main_RA | 0.1% | 0.1% | 0.0% | 0.2% | 0.4% | 0.4% |
| Main_LD | 0.1% | 0.2% | -0.1% | 0.2% | 0.4% | 0.3% |
| HE10_Intra | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| HE10_RA | 0.1% | 0.1% | 0.0% | 0.3% | 0.5% | 0.4% |
| HE10_LD | 0.1% | 0.2% | 0.0% | 0.4% | 0.5% | 0.4% |
| Average | 0.05% | 0.11% | 0.00% | 0.20% | 0.33% | 0.28% |

Tab.3 summarize the average saving of total bins and context-coded bins number of the proposed method 1 by comparing them to HM7.0 in common test condition and low QP test condition.

Tab.4 summarize the average saving of total bins and context-coded bins number of the proposed method 2 by comparing them to HM7.0 in common test condition and low QP test condition.

Tab.3 Savings of average context-coded bin number of proposed method 1

| Test configuration | Method 1 (Normal QP) |  | Method 1 (Low QP) |  |
| --- | --- | --- | --- | --- |
|  | CABAC Bins |  |  |  |
|  | Total | Context-coded | Total | Context-coded |
| Main_Intra | -0.1% | -3.4% | 0.0% | -9.2% |
| Main_RA | 0.0% | -2.3% | -0.3% | -7.5% |
| Main_LD | -0.1% | -0.9% | 0.0% | -6.7% |
| HE10_Intra | -0.1% | -3.4% | 0.0% | -9.4% |
| HE10_RA | 0.0% | -2.4% | 0.0% | -7.8% |
| HE10_LD | 0.0% | -1.0% | 0.0% | -7.0% |
| Average | -0.1% | -2.2% | -0.1% | -7.9% |

Tab.4 Savings of average context-coded bin number of proposed method 2

| Test configuration | Method 2 (Normal QP) |  | Method 2 (Low QP) |  |
| --- | --- | --- | --- | --- |
|  | CABAC Bins |  |  |  |
|  | Total | Context-coded | Total | Context-coded |
| Main_Intra | -0.2% | -6.6% | -0.8% | -14.3% |
| Main_RA | -0.1% | -5.0% | -0.3% | -12.7% |
| Main_LD | 0.0% | -2.3% | -0.2% | -11.6% |
| HE10_Intra | -0.1% | -6.7% | -0.8% | -14.4% |
| HE10_RA | -0.1% | -5.3% | -0.4% | -13.2% |
| HE10_LD | 0.0% | -2.6% | -0.2% | -12.1% |
| Average | -0.1% | -4.7% | -0.5% | -13.1% |

# Conclusion

In this contribution, we proposed to switch off context-coded greater1_flag after one or two greater1_flag equal to 1 is encountered in each coefficients group. Experiment results show that the proposed methods reduce context-based coded bins averagely more than 10% for low QP test condition with negligible compression efficiency impact.

# Reference

[1] J. Chen, W.-J. Chien, R. Joshi, J. Sole, M. Karczewicz ,” Non-CE1: throughput improvement on CABAC coefficients level coding”, JCTVC-H0554, San Jose, Feb. 2012.

[2] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting, Geneva, April 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text modification

## Change for method 1

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| secondGreater1CoeffIdx = 16 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 && secondGreater1CoeffIdx == 16 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| else if (coeff_abs_level_greater1_flag[ n ]) |  |
| secondGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= 4 && !cu_ transquant_bypass _flag) |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 1+(n<=secondGreater1CoeffIdx) ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Change for method 2

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| …… |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 && firstGreater1CoeffIdx == -1 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ]) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= 4 && !cu_ transquant_bypass _flag) |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) && (n <= firstGreater1CoeffIdx) ? 3 : 1 ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |