| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0286_r3 WG11 Number: m25614 |
| --- | --- |

| Title: | AHG6: Reduction of block artifacts in HEVC for large blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document studies a problem with blocking artifacts in Riverbed, WestWindEasy and China Speed sequences and proposes modifications to HM7.0 that fix these problems. The proposed modifications reportedly reduce the blocking artifacts on Riverbed, China Speed and West Wind Easy sequences and also improve the visual quality on the sequences in the common test conditions. The proposed modifications result in the following changes in BD-rate: (0.1%, 0.0%, 0.1% and -0.1%) on Main profile and (0.3%, 0.2%, 0.6%, 0.2%) on HE10 configuration, the decoding time is similar to that of the anchor. The modifications do NOT include sending additional parameters to the deblocking filter.

# Introduction

During the 10th JCT-VC meeting in Geneva it has been reported in [1] that there are residual blocking artifacts in certain sequences that cannot be completely removed by the current deblocking filtering. These artifacts can be observed under QP 32 and 37 and sometimes even under QP 27. The sequences where the artifacts are observed are mostly outside the current test set, for example "West Winds Easy", "Controlled Burn", and "Riverbed" [1], and one sequence, "China Speed" is from the test set. It has been claimed that this problem with deblocking cannot be solved even by increasing the deblocking strength by setting the offsets to the maximum. Therefore, during the meeting it has been decided to include in the mandates of the AHG6 studying of the solution for extended deblocking adaptability range.

This document presents a study of the problem with the deblocking filtering and analyses the reasons which these artifacts are not removed by deblocking filtering. Then, it suggests a solution to this problem that significantly reduces the observed deblocking artifacts in the reported sequences and also improves the subjective quality in some other sequences (that are used in the current test set [3]). The reported changes in BD-Rate are:  (0.1%, 0.0%, 0.1% and -0.1%) on Main profile and (0.3%, 0.1%, 0.6%, 0.2%) on HE10 configuration and the decoding time is similar to that of the anchor.

# Source of the blocking artifacts in the reported sequences

## Location of blocking artifacts

The sequences reported in [1] have been studied on the topic of reducing the blocking artifacts. The sequences have been encoded and the blocking artifacts studied. It has been found that the reported artifacts are mostly caused by intra-predicted blocks of a large size (e.g. 32x32), where the transform coefficients are coarsely quantized. The blocking artifacts are quite visible even in motion since the block boundaries are usually aligned with the 32x32 sample grid. The examples of blocking such blocks are shown in Figure 1 and Figure 2.

(a)

(b)

Figure 1 West Winds Easy, Low delay configuration, QP37.  (a) Blocking artifacts, (b) Picture divided into CUs, intra-coded CU are marked by red color.

(a)

(b)

Figure 2 Riverbed, Random Access y configuration, QP37.  (a) Blocking artifacts, (b) Picture divided into CUs, intra-coded CU are marked by red color.

One can see from the figures above that the blocks are mostly created by intra-predicted blocks. The deblocking is applied for intra blocks with larger strength but it cannot, however, completely remove these blocking artifacts. The intra-predicted blocks in the pictures above are quite smooth, which means that the deblocking filtering should be applied to these block boundaries. Yet, it does not happen. After studying the problem, the reasons the deblocking does not remove those artifacts have been found. The next section studies the reasons why the current HM7.0 deblocking does not remove these artifacts.

## Why the artifacts are not removed by deblocking filtering

### Brief overview on HEVC deblocking decisions

First, the expressions below are calculated for the block boundary, where pixels pi,j belong to the block on one side of the block boundary and pixels qi,j belong to the other side of the block boundary (see Figure 3). These values are further used in the deblocking filtering decisions.

dp0 = | p2,0 − 2*p1,0 + p0,0 |;   dq0 = | q2,0 − 2*q1,0 + q0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | ;  dq3 = | q2,3 − 2*q1,3 + q0,3 |

dpq0 = dp0 + dq0;   dpq3 = dp3 + dq3;

dp = dp0 + dp3; 	 dq = dq0 + dq3

d = dpq0 + dpq3

p30 p20 p10 p00

p31 p21 p11 p01

p32 p22 p12 p02

A p33 p23 p13 p03

q00 q10 q20 q30

q01 q11 q21 q31

q02 q12 q22 q32

q03 q13 q23 q33  B

Figure 3 Vertical block boundary

The deblocking filtering is performed on the block boundary for lines i = 0..3  if the following condition holds:

d < β

Then there is a decision between applying a strong filter or a normal filter (which in its turn can modify from one to two pixels from the block boundary). The decisions for filtering one or two pixels from the block boundary use comparison of the values dp and dq with the threshold, approximately equal to β/6.

There is also another type of filtering, called a strong filter, which modifies three pixels from the block boundary. The following conditions shall hold in order the strong filter to be applied to the block boundary.

2*dpqi < (β>>2)					(1)

(|p3i - p0i| + |q0i – q3i|) < (β>>3)  	   	(2)

|p0i – q0i| < ((5*tC + 1)>>1)			(3)

Condition (1) checks that the signal on the sides of the block boundary is smooth, condition (2) checks that the signal on both side of the block boundary is flat for 4 pixels on from the block boundary, and the condition (3) makes sure that there is no big change in intensity between the pixels closest to the block boundary.

### Identified problems with intra-block boundaries

Intra-prediction filtering

Figure 4 shows one of the problems with deblocking of intra-blocks. Figure 4 (a) illustrates a DC intra prediction of a 32x32 block in the Riverbed sequence before adding a residual (the deblocking and SAO are turned off). Since for the large intra blocks (e.g. 32x32) the DC prediction value may be significantly different from the surrounding pixel, DC intra-filtering and intra-filtering for the vertical and horizontal modes may introduce singularities along the block boundary. These singularities may still remain after adding the reconstructed residual (see Figure 4 (b)). These singularities prevent the stronger modes of deblocking filtering from being turned on (since they are treated by deblocking as a structure at the block boundary). These artifacts are therefore still visible even after the deblocking filtering (see Figure 4(c)).

| - DC prediction mode, block 32x32, intra-filtering along a block boundary creates a singularity in prediction. | - Same block after overlaying the residual error, intra-prediction singularity creates an artifact at block boundary |
| --- | --- |
| - Remaining artifact after deblocking |  |

Figure 4 Riverbed. Example of blocking artifacts caused by intra prediction filtering.

Strong deblocking filtering decisions

The blocking artifacts visible in the reported sequences are mostly created by the 32x32 blocks. The deblocking filtering can only operate at the block boundary. It has been observed that only applying a weak deblocking filter is not enough to remove the artifacts between two 32x32 blocks with the significant difference between the reconstruction levels of the blocks, since the normal deblocking filter can only modify two samples from the block boundary. It is therefore necessary to apply a "strong" deblocking filter for such block boundaries, which modifies three pixels from the block boundary. Equations (1) – (3) show conditions necessary for applying the strong deblocking filter in HEVC.

These conditions for applying the strong filter are schematically explained in Figure 5 (a). An example of a block artifacs from Riverbed is shown in Figure 5 (b). One can see that conditions (2) and (3) are not fulfilled, therefore, strong deblocking filtering will not be applied to such a block boundary, although two sides of the block boundary are quite smooth. The reason is that the strong filtering mode is only applied if the signal on the sides of the block boundary is flat and the difference between values of the pixels closest to the block boundary (p0 and q0) is small. However, these conditions are not held for the intra blocks in sequences like Riverbed, since the signal on the sides of the block boundary has a form of a ramp and the difference between p0 and p1 may be large (see Figure 5 (b)).

| (a) | (b) |
| --- | --- |

Figure 5 (a) Conditions for applying strong filter in HEVC (b) Example of blocking artifact from Riverbed. Strong filtering is not applied since two conditions do not hold (that the signal on the sides is flat and the difference in intensity between two blocks is small).

# Proposed solution to reduce blocking artifacts

Since the problem with blocking artifacts in the observed sequences is a complex problem, it is proposed to modify the HM and specification text as described in the following.

## Restrict intra-prediction filtering in DC, vertical, and horizontal intra prediction modes to block sizes 4 and 8

It has been observed that intra-prediction boundary filtering sometimes results in visually disturbing artifacts at the block boundaries. This is mostly observed for intra-predicted blocks of size 32x32 and sometimes for intra blocks of size 16x16. It is therefore proposed to reduce application of the intra-prediction filtering in DC, vertical and horizontal modes to the blocks of size 4x4 and 8x8. The simulation results show that restricting the intra prediction to smaller block sized results in BD-Rate change of 0.1% in all-intra configurations (see Table 1). Most of the gain for intra-prediction filtering comes from smaller block sizes and applying it for larger blocks sizes results in visual artifacts on certain type of content without much gain. Therefore, it is suggested to reduce the intra-prediction filtering in DC, vertical (26) and horizontal (10) modes to blocks of size 16x16, 8x8 and 4x4.

Table 1 Intra filtering in DC, vertical and horizontal modes is restricted to blocks of size 4x4 and 8x8

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.5% | 0.5% | 0.2% | 0.4% | 0.3% |
| Class B | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% |
| Overall | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | -0.3% | 0.0% |
| Enc Time[%] | 108% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 93% |  |  |

Table 2 Intra filtering in DC, vertical and horizontal modes is restricted to blocks of size 4x4, 8x8, and 16x16

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 87% |  |  | 87% |  |  |
| Dec Time[%] | 82% |  |  | 79% |  |  |

## Modify decision for applying strong filter

It has been demonstrated in Section 2.2.2 that current strong filtering decision does not allow application of the strong deblocking filter to the boundaries between the large intra-predicted blocks, where it should be applied in order to improve the subjective video quality. It is therefore proposed to modify the strong filtering decisions in order for strong deblocking filter to be applied to the block boundary between the large intra-blocks.

It is proposed to modify the decisions for application of the strong filtering as shown below. The following conditions are checked for strong filter based on lines i = 0, 3.

2*dpqi < (β>>2)							(4)

(|p3i - p0i| + |q0i – q3i|) < (β>>2)  	   		(5) // remove a condition that 4 pixels from the block boundary are flat

dp0 = ((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  	(5) // instead, add a condition that 4 pixels //from the block boundary form a straight line (flat or inclined),

|p0i – q0i| < ((5*tC + 1)>>1)  (tC <<4)		      (6) //use a larger threshold in a check for the //ldifference between intensities of pixels p0 and p1

The proposed decisions are explained in Figure 6. The proposed decisions enable applying strong filtering when both sides of the block boundary have a form of ramp (even if they are not flat). The threshold for evaluating pixel value difference | p0 – q0 | is also increased.

| (a) | (b) |
| --- | --- |

Figure 6 (a) Proposed decisions for strong filtering. Two checks are included: that pixel values q0, q1, q2 and q3 form a line (the same for p0, p1, p2 and p3) and that | p0 – q0 | is below threshold. (b) Block artifact from Riverbed. Filter decision matches the artifact.

## Modify strong filter

Since the proposed filtering decisions turn on the strong filtering in the areas where it is not turned on in the current HEVC deblocking (e.g. on the signal that has a form of ramp), it is also necessary to modify the strong filter. Since the current HEVC filter is designed to be applied to a signal that is flat on both sides of the block boundary, the HEVC strong filter would modify a ramp when applied to it. Therefore, it is proposed to modify the filter coefficients that correspond to pixels p1, q1, p2, q2 so that the filter would not modify a ramp.

The proposed modifications to the strong filter calculations are shown below.

p0’ = Clip3( p0−tc0, p0+tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   // same as in HM7.0
q0’ = Clip3( q0−tc0, q0+tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )   // same as in HM7.0

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1'   = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 +   q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc3, p2+tc3, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc3, q2+tc3, ((p0 +  q1 + 3*q2 + 3*q3 +4 )>>3))

The effect of applying HEVC strong filter and the proposed filter to a ramp is shown in Figure 7. One can see from the figure that HEVC filter significantly disports the ramp when applied to it, while the proposed filter modified the ramp much less.

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 7 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red.  The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

The clipping has also been modified in order to avoid blurring of the reconstructed image by applying too strong filtering (since the strong filter is applied more often). Therefore, the clipping thresholds used in the equations above are reduced relative to the current strong filter in order to reduce amount of blurriness.

The following clipping values are used: tc0 = (3*tc+1)>>1; tc1 = tc, tc2 = (tc+1)>>1.

## Bs derivation

The Bs derivation is modified. Block boundaries that have Bs equal to 2 (one of the blocks is intra) and lie on the 32x32 grid are assigned the boundary strength 3 in case the size of at least one of the CUs forming the boundary is equal or greater than 32x32. Since the boundary strength is used in derivation of clipping offset tc, as (Bs-1)*2, the intra-blocks edges with BS = 3, use the tc offset of 4.

# Objective performance

The proposed algorithm has been implemented based on HM7.0.

## Performance on common test conditions

The objective performance of the proposed modifications to deblocking filtering has been evaluated. The results are shown in Table 3. One can see that the BD-Rate performance of the proposal is roughly the same for the main profile (except some small deviations of -0.1% to 0.1n). One can see from Table 3 that the losses on AI Main and LD Main are in fact below 0.1%. The performance on HE10 configuration is slightly worse with the worst case on LD B HE10 of 0.6% which mostly happens because to a loss on class E sequences, in particular sequence Jonny.

The decoding time on a single machine has been measured and is provided in Table 3. It has been found that the decoding time is similar to that of HM7.0 (the same on most configurations and slightly shorter on two configurations).

Table 3 Objective performance of the proposed method versus HM7.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.3% | 0.5% | 0.4% | 0.4% |
| Class B | 0.1% | 0.2% | 0.1% | 0.5% | 0.2% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.06% | 0.09% | 0.11% | 0.26% | 0.12% | 0.12% |
|  | 0.06% | 0.09% | 0.11% | 0.26% | 0.12% | 0.13% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | 0.0% | 0.3% | 0.4% | 0.1% |
| Class B | 0.0% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.3% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% |
| Overall | -0.03% | 0.07% | 0.00% | 0.15% | 0.18% | 0.01% |
|  | -0.03% | 0.04% | -0.01% | 0.15% | 0.17% | 0.01% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% | 0.5% | 0.4% | 0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.4% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% | -0.3% |
| Class E | 0.3% | -0.2% | 0.0% | 1.9% | 0.4% | 1.0% |
| Class F | 0.1% | 0.2% | 0.7% | 0.3% | 1.0% | 0.8% |
| Overall | 0.08% | 0.09% | 0.21% | 0.56% | 0.44% | 0.33% |
|  | 0.08% | 0.11% | 0.27% | 0.56% | 0.49% | 0.52% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | -0.2% | 0.1% | 0.3% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.2% | 0.0% | 0.2% |
| Class D | 0.0% | 0.0% | 0.6% | 0.1% | 0.4% | -0.8% |
| Class E | -0.2% | -1.0% | 0.5% | 0.7% | 0.6% | 1.0% |
| Class F | 0.0% | 0.2% | 0.0% | 0.1% | 0.7% | -0.3% |
| Overall | -0.12% | -0.09% | 0.11% | 0.22% | 0.39% | -0.07% |
|  | -0.13% | -0.06% | 0.13% | 0.22% | 0.45% | 0.07% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Performance on additional AHG6 sequences

The following additional sequences were suggested by the AHG6 for participation in the subjective quality test: Riverbed, DucksTakeOff, WestWindsEasy (and the sequence China speed from the test set). The objective results for additional test sequences are provided below. The QPs used for these test sequences are: QP 32, QP37 and QP 41.

Table 4 Additional sequences on Random Access Main configuration

| Sequence/QP | Coding scheme | bitrate | Y-PSNR |
| --- | --- | --- | --- |
| Riverbed QP32 | HM7.0 | 4753.014 | 33.5343 |
|  | Proposal | 4758.265 | 33.5618 |
| Riverbed QP37 | HM7.0 | 2340.792 | 30.8034 |
|  | Proposal | 2341.552 | 30.8366 |
| Riverbed QP41 | HM7.0 | 1269.62 | 28.7479 |
|  | Proposal | 1269.18 | 28.7755 |
| DucksTakeOff QP32 | HM7.0 | 13980.11 | 30.2157 |
|  | Proposal | 13982.78 | 30.2204 |
| DucksTakeOff QP37 | HM7.0 | 5820.6792 | 27.9738 |
|  | Proposal | 5819.352 | 27.9815 |
| DucksTakeOff QP41 | HM7.0 | 2878.7312 | 26.3534 |
|  | Proposal | 2879.313 | 26.3693 |
| WestWindEasy QP32 | HM7.0 | 3161.509 | 35.3162 |
|  | Proposal | 3163.272 | 35.3281 |
| WestWindEasy QP37 | HM7.0 | 1486.856 | 32.4935 |
|  | Proposal | 1485.808 | 32.5149 |
| WestWindEasy QP41 | HM7.0 | 793.2672 | 30.4628 |
|  | Proposal | 792.1168 | 30.488 |
| ChinaSpeed QP32 | HM7.0 | 1375.703 | 34.6839 |
|  | Proposal | 1375.398 | 34.693 |
| ChinaSpeed QP37 | HM7.0 | 691.7501 | 31.588 |
|  | Proposal | 692.1101 | 31.5908 |
| ChinaSpeed QP41 | HM7.0 | 396.7243 | 29.3256 |
|  | Proposal | 396.804 | 29.3348 |

Table 5 Additional sequences on Low Delay B Main configuration

| Sequence/QP | Proposal | bitrate | Y-PSNR |
| --- | --- | --- | --- |
| Riverbed QP32 | HM7.0 | 5859.474 | 34.6417 |
|  | Proposal | 5861.332 | 34.6548 |
| Riverbed QP37 | HM7.0 | 2855.325 | 31.6933 |
|  | Proposal | 2853.658 | 31.726 |
| Riverbed QP41 | HM7.0 | 1554.726 | 29.5166 |
|  | Proposal | 1552.378 | 29.5492 |
| DucksTakeOff QP32 | HM7.0 | 18542.1 | 30.37 |
|  | Proposal | 18541.3 | 30.3745 |
| DucksTakeOff QP37 | HM7.0 | 7588.072 | 27.7451 |
|  | Proposal | 7582.997 | 27.7539 |
| DucksTakeOff QP41 | HM7.0 | 3715.104 | 25.9805 |
|  | Proposal | 3713.483 | 25.9924 |
| WestWindEasy QP32 | HM7.0 | 3753.219 | 36.2091 |
|  | Proposal | 3751.095 | 36.2149 |
| WestWindEasy QP37 | HM7.0 | 1672.056 | 32.9926 |
|  | Proposal | 1667.71 | 33.0123 |
| WestWindEasy QP41 | HM7.0 | 853.7 | 30.7687 |
|  | Proposal | 849.7728 | 30.7765 |
| ChinaSpeed QP32 | HM7.0 | 1334.094 | 34.9814 |
|  | Proposal | 1333.655 | 34.9813 |
| ChinaSpeed QP37 | HM7.0 | 604.5355 | 31.7007 |
|  | Proposal | 603.9451 | 31.7125 |
| ChinaSpeed QP41 | HM7.0 | 308.2138 | 29.2775 |
|  | Proposal | 308.2051 | 29.2736 |

# Subjective results

Subjective quality of the proposal has been studied. It has been found that the proposal substantially decreases the amount of blocking artifacts on the sequences that were reported to contain a lot of blocking artifacts (West Wind Easy, Riverbed, China Speed).

The proposal has also been evaluated on the test sequences that belong to common test conditions. A "blind test" was used where the test person does not know the identities of the sequences. It has been found that that the proposal has same or better visual quality on most of studied sequences compared with HM7.0 on the common test conditions [3]. In general, it has been concluded that the proposal has better visual quality than HM7.0.

Examples of improvement achieved by the proposed deblocking filter are shown in Figure 8 - Figure 10.

| - HM7.0 | - Proposal |
| --- | --- |

Figure 8 Comparison of HM7.0 and proposal on West Wind Easy QP37

| - HM7.0 | - Proposal |
| --- | --- |

Figure 9 Comparison of HM7.0 and proposal on West Wind Easy QP32

| (a) HM7.0 | - Proposal |
| --- | --- |

Figure 10 Comparison of HM7.0 and proposal on China Speed QP32

# Conclusions

A study of blocking artifacts on certain sequences has been done. It is concluded that the artifacts are mostly caused my intra-prediction filtering on large blocks and by the fact that the decision for strong deblocking filter application does not match a pattern of intra blocking artifacts on those sequences. Modifications to the intra-prediction filtering and to deblocking filtering are proposed. The modification significantly reduce the blocking artifacts on the reported sequences (Riverbed, WestWindEasy, China Speed) without sending additional sequence-dependent parameters to the deblocking filter. Improvements of subjective quality on other sequences were also observed. The objective performance of the proposed algorithm is similar to HM7.0 on main profile. Some losses are reported on HE10 profile, in particular on Low-delay B configuration. The decoding time of the proposal is similar with that of the anchor.

It is therefore proposed to adopt the algorithm to the specification text and to the next version of HM software.

# References

- D.-K. Kwon, M. Budagavi, "Transform size dependent deblocking filter", JCTVC-I0244, Geneva, CH, May 2012.

- "High Efficiency Video Coding (HEVC) text specification draft 7", JCTVC-I1003, Geneva, CH, May 2012.

- F. Bossen, "Common HM test conditions and software reference configurations", JCTVC-I1100, Geneva, CH, May 2012.

# Appendix: Additional results for algorithm without Bs modification (tc_offset is increased from 2 to 4 instead)

An additional test has been done for another coding configuration. In this test, another algorithm, called Algorithm 2 has been tested. The Algorithm 2 is identical to algorithm described in the main part with one difference. In Algorithm 2, there no changes in calculation of Bs value, which stays exactly the same as in HM7.0. Instead, the tc_offset for intra blocks is increased from 2 to 4. The value of tc_offset equal to 4 was used in HM before the JCT-VC meeting in Torino. Changing the value of tc_offset from 2 to 4 has similar effect to assigning the value Bs = 3 as can be seen from the equation below. However, unlike in main algorithm, the change in tc_offset is applied to all intra-block boundaries (to both chroma and luma components).

The value of the variable tC′ is determined as specified as Error: Reference source not found based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 53, qPL + 4*( bS − 1) + ( tc_offset_div2 << 1) )	(8-1)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

Table 6 Objective results for Algorithm 2 (without Bs modification and with tc_offset change from 2 to 4)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.6% | 0.6% | 0.8% | 0.9% | 0.8% |
| Class B | 0.3% | 0.7% | 0.7% | 0.8% | 0.7% | 0.7% |
| Class C | 0.3% | 0.8% | 0.8% | 0.5% | 1.0% | 1.0% |
| Class D | 0.3% | 0.8% | 0.8% | 0.4% | 0.9% | 1.0% |
| Class E | 0.1% | 0.3% | 0.0% | 0.4% | 0.5% | 0.2% |
| Class F | 0.1% | 0.8% | 0.9% | 0.2% | 1.0% | 1.2% |
| Overall | 0.25% | 0.68% | 0.66% | 0.53% | 0.84% | 0.82% |
|  | 0.25% | 0.68% | 0.67% | 0.54% | 0.86% | 0.84% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.4% | 0.4% | 1.0% | 0.6% |
| Class B | 0.1% | 0.9% | 0.7% | 0.4% | 0.8% | 0.7% |
| Class C | 0.1% | 0.9% | 0.8% | 0.2% | 0.9% | 0.7% |
| Class D | 0.1% | 1.1% | 0.8% | 0.1% | 0.6% | 0.5% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 1.2% | 1.1% | 0.2% | 1.5% | 1.3% |
| Overall | 0.08% | 0.81% | 0.77% | 0.29% | 0.97% | 0.76% |
|  | 0.09% | 0.78% | 0.78% | 0.29% | 0.98% | 0.77% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.2% | 0.5% | 0.5% | 0.4% |
| Class C | 0.1% | 0.4% | 0.3% | 0.5% | 0.2% | 0.3% |
| Class D | 0.0% | 0.6% | 0.4% | 0.1% | 0.6% | 0.2% |
| Class E | 0.3% | -0.1% | 0.3% | 2.0% | 0.6% | 0.8% |
| Class F | 0.0% | 0.0% | 1.4% | 0.4% | 1.1% | 1.3% |
| Overall | 0.1% | 0.3% | 0.5% | 0.6% | 0.6% | 0.6% |
|  | 0.1% | 0.3% | 0.5% | 0.6% | 0.7% | 0.7% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.4% | -0.2% | 0.1% | 0.5% | 0.1% |
| Class C | 0.0% | -0.1% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class D | 0.0% | 0.6% | 0.7% | 0.1% | 0.1% | -0.4% |
| Class E | -0.1% | -0.9% | 0.7% | 0.7% | 0.8% | 0.9% |
| Class F | 0.0% | 0.0% | 0.5% | 0.2% | 1.1% | 0.3% |
| Overall | -0.09% | 0.05% | 0.34% | 0.25% | 0.52% | 0.21% |
|  | -0.10% | 0.04% | 0.32% | 0.25% | 0.52% | 0.37% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

One can see that the results for Algorithm 2 illustrate somewhat larger increase in BD-Rate on All-Intra configurations because of increased number of intra-coded block boundaries that are filtered with tc_offset = 4.

The objective results for additional sequences from AHG6 are provided below Table 7 and Table 8.

Table 7 Additional sequences on Random Access Main configuration for Algorithm 2 (without Bs modification and with tc_offset change from 2 to 4)

| Sequence/QP | Coding scheme | bitrate | Y-PSNR |
| --- | --- | --- | --- |
| Riverbed QP32 | HM7.0 | 4753.014 | 33.5343 |
|  | Proposal | 4759.892 | 33.5645 |
| Riverbed QP37 | HM7.0 | 2340.792 | 30.8034 |
|  | Proposal | 2341.432 | 30.8353 |
| Riverbed QP41 | HM7.0 | 1269.62 | 28.7479 |
|  | Proposal | 1269.296 | 28.7767 |
| DucksTakeOff QP32 | HM7.0 | 13980.11 | 30.2157 |
|  | Proposal | 13982.58 | 30.22 |
| DucksTakeOff QP37 | HM7.0 | 5820.679 | 27.9738 |
|  | Proposal | 5819.99 | 27.9812 |
| DucksTakeOff QP41 | HM7.0 | 2878.731 | 26.3534 |
|  | Proposal | 2880.146 | 26.3705 |
| WestWindEasy QP32 | HM7.0 | 3161.509 | 35.3162 |
|  | Proposal | 3163.59 | 35.3266 |
| WestWindEasy QP37 | HM7.0 | 1486.856 | 32.4935 |
|  | Proposal | 1486.143 | 32.5145 |
| WestWindEasy QP41 | HM7.0 | 793.2672 | 30.4628 |
|  | Proposal | 792.648 | 30.491 |
| ChinaSpeed QP32 | HM7.0 | 1375.703 | 34.6839 |
|  | Proposal | 1375.119 | 34.6888 |
| ChinaSpeed QP37 | HM7.0 | 691.7501 | 31.588 |
|  | Proposal | 691.9382 | 31.5897 |
| ChinaSpeed QP41 | HM7.0 | 396.7243 | 29.3256 |
|  | Proposal | 396.385 | 29.3257 |

Table 8 Additional sequences for Algorithm 2 on Low Delay B Main configuration

| Sequence/QP | Proposal | bitrate | Y-PSNR |
| --- | --- | --- | --- |
| Riverbed QP32 | HM7.0 | 5859.474 | 34.6417 |
|  | Proposal | 5861.072 | 34.655 |
| Riverbed QP37 | HM7.0 | 2855.325 | 31.6933 |
|  | Proposal | 2851.404 | 31.7215 |
| Riverbed QP41 | HM7.0 | 1554.726 | 29.5166 |
|  | Proposal | 1552.112 | 29.5495 |
| DucksTakeOff QP32 | HM7.0 | 18542.1 | 30.37 |
|  | Proposal | 18552.62 | 30.3761 |
| DucksTakeOff QP37 | HM7.0 | 7588.072 | 27.7451 |
|  | Proposal | 7581.936 | 27.7533 |
| DucksTakeOff QP41 | HM7.0 | 3715.104 | 25.9805 |
|  | Proposal | 3711.078 | 25.9933 |
| WestWindEasy QP32 | HM7.0 | 3753.219 | 36.2091 |
|  | Proposal | 3747.99 | 36.21 |
| WestWindEasy QP37 | HM7.0 | 1672.056 | 32.9926 |
|  | Proposal | 1666.871 | 33.0093 |
| WestWindEasy QP41 | HM7.0 | 853.7 | 30.7687 |
|  | Proposal | 848.9672 | 30.7862 |
| ChinaSpeed QP32 | HM7.0 | 1334.094 | 34.9814 |
|  | Proposal | 1333.117 | 34.984 |
| ChinaSpeed QP37 | HM7.0 | 604.5355 | 31.7007 |
|  | Proposal | 604.0157 | 31.7065 |
| ChinaSpeed QP41 | HM7.0 | 308.2138 | 29.2775 |
|  | Proposal | 308.8378 | 29.2752 |

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).