| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I1003_d7 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 7 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 7 of High efficiency video coding.

## Decoding process for coding units coded in intra prediction mode

##### Specification of Intra_Angular (26) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nS−1,

–	a variable nS specifying the prediction size

–	a variable cIdx specifying the chroma component of the current block

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 26.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as follows:

–	If cIdx is equal to 0 and Log2(nS) is less than 5,

predSamples[ x, y ] = p[ x, −1 ], with x = 1..nS−1, y = 0..nS−1	(8-1)

predSamples[ x, y ] = Clip1Y( p[ x, −1 ] + ( ( p[ −1, y ] – p[ −1, −1 ] ) >> 1 ) ), with x = 0, y = 0..nS−1

–	Otherwise,

predSamples[ x, y ] = p[ x, −1 ], with x, y = 0..nS−1		(8-2)

##### Specification of Intra_Angular (10) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nS−1,

–	a variable nS specifying the prediction block size.

–	a variable cIdx specifying the chroma component of the current block

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 10.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as follows:

–	If cIdx is equal to 0 and Log2(nS) is less than 5,

predSamples[ x, y ] = p[ −1, y ], with x = 0..nS−1, y = 1..nS−1	(8-3)

predSamples[ x, y ] = Clip1Y( p[ −1, y ] + ( ( p[ x, −1 ] – p[ −1, −1 ] ) >> 1 ), with x = 0..nS−1, y = 0

–	Otherwise,

predSamples[ x, y ] = p[ −1, y ], with x, y = 0..nS−1		(8-4)

##### Specification of Intra_DC prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = −1..2*nS−1,

–	a variable nS specifying the prediction block size.

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- A variable DCVal is derived as:

DCVal = x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS−1	(8-5)
where k=Log2(nS)

- Depending on the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and Log2(nS) is less than 5, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ −1, 0 ] + 2*DCVal + 1*p[ 0, −1 ] + 2 ) >> 2	(8-6)
predSamples[ x, 0 ] = ( 1*p[ x, −1 ] + 3*DCVal + 2 ) >> 2, with x = 1..nS−1	(8-7)
predSamples[ 0, y ] = ( 1*p[ −1, y ] + 3*DCVal + 2 ) >> 2, with y = 1..nS−1	(8-8)
predSamples[ x, y ] = DCVal, with x, y = 1..nS−1	(8-9)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS−1	(8-10)

## In-loop filter process

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a two-dimensional array of size (nS)x(nS), edgeFlags.

Output of this process is:

–	a two-dimensional array of size (nS)x(nS), bS specifying the boundary filtering strength.

The boundary filtering strength array bS for the current coding unit is derived as follows.

The variables xDi, yDj, xN and yN are derived as follows.

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i << 3 ), yDj is set equal to ( j << 2 ), xN is set equal to (1 << ( log2CbSize − 3 )) − 1 and yN is set equal to (1 << ( log2CbSize − 2 )) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to (1 << ( log2CbSize − 2 )) − 1 and yN is set equal to (1 << ( log2CbSize − 3 )) − 1.

For xDi with i=0..xN, the following applies.

For yDj with j=0..yN, the following applies.

- If edgeFlags[ xDi ][ yDj ] is equal to 1, the sample values are derived as follows.

- If edgeType is equal to EDGE_VER, sample p0 = recPictureL[ xC + xDi − 1 ][ yC + yDj ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

- Otherwise (edgeType is equal to EDGE_HOR), sample p0 = recPictureL[ xC + xDi ][ yC + yDj − 1 ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

Depending on p0 and q0, the variable bS[ xDi ][ yDj ] is derived as follows.

- If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, the following applies

    - If the sample p0 or q0 is in a CU which size is 32x32 or larger and the samples p0 or q0 are not in the same 32x32 partition

      - variable bS[ xDi ][ yDj ] is set equal to 3

    - Otherwise

      - variable bS[ xDi ][ yDj ] is set equal to 2.

- Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains non-zero transform coefficient level.

- Otherwise, the following applies.

    - When edgeType is equal to EDGE_HOR and yC + yDj − 1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), sample p0 = recPictureL[ xL ][ yC + yDj – 1 ] where xL is equal to ((( xC + xDi ) >> 3) << 3) + ((( xC + xDi ) >> 3) & 1) * 7.

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma frame samples.

    - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma frame samples,

    - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and all of the following conditions are true:

    - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction bocks is greater than or equal to 4 in quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma frame samples,

    - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise (edgeFlags[ xDi ][ yDj ] is equal to 0), the variable bS[ xDi ][ yDj ] is set equal to 0.

##### Decision process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dpq, β and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If dpq is less than ( β >> 2 ),  (abs(p3 – 2*p2 + p1) + abs(q3 – 2*q2 + q1))<<1  is less than ( β >> 2 ), and abs( p0 – q0 ) is less than (tC << 4), dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dE,

–	variables dEp1 and dEq1 containing decisions to filter samples p1 and q1 respectively,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	If the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

tc0 = (3*tc+1) >> 1

tc2 = (tc +1) >> 1

p0’ = Clip3( p0−tc0, p0+tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-11)

-

-

p1’ = Clip3( p1−tc, p1+tc, (p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3)	(8-14)

p2’ = Clip3( p2−tc2, p2+tc2, (3*p3 + 3*p2 + p1 + q0 + 4 )>>3 )	(8-15)

q0’ = Clip3( q0−tc0, q0+tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-16)

-

-

q1’ = Clip3( q1−tc, q1+tc, (q3 + 2*q2+ 2*q1 + q0+ 2*p0 + 4)>>3)	(8-19)

q2’= Clip3( q2−tc2, q2+tc2, ( p0 + q1 + 3*q2 + 3*q3 + 4 ) >> 3 )	(8-20)

–	Otherwise, nDp and nDq are set equal to 0 and the following weak filtering applies:

 = ( 9 * ( q0 –  p0 ) − 3 * ( q1 – p1 ) + 8 ) >> 4		(8-21)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( −tc, tc,  )		(8-22)

p0’ = Clip1Y( p0 +  )		(8-23)

q0’ = Clip1Y( q0 −  )		(8-24)

    - When dEp1 is equal to 1, the filtered sample value p1’ is specified as follows:

p = Clip3( −(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-25)

p1’ = Clip1Y( p1 + p )		(8-26)

    - When dEq1 is equal to 1, the filtered sample value q1’ is specified as follows:

q = Clip3( −(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-27)

q1’ = Clip1Y( q1 + q )		(8-28)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

When nDp is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.