| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0287 |
| --- | --- |

| Title: | Additional sample_ratio_idc values for square samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document proposes new aspect_ratio_idc values for signaling square samples with a sample scale factor other than one.  A number of video applications require support for bitstreams in which the picture resolution may change from one coded video sequence (CVS) to the next, while maintaining constant the 2D size and aspect ratio of the spatial span implied by the output pictures of the successive coded video sequences (CVSes).  In order to maintain the same implied spatial span from the output pictures of two consecutive CVSes with square samples but different picture widths and heights, the aspect_ratio_idc values of the two consecutive CVSes would need to be different.  It would also facilitate entering the bitstream at the RAP (random access point) picture corresponding to the start of a CVS in which the picture resolution changes but the sample aspect ratio does not.

# Introduction

A number of video applications require keeping constant the 2D size and aspect ratio of the spatial span implied by the output pictures of successive CVSes in the bitstream.  This is necessary for advertisement insertion in broadcast applications, where the picture resolution changes. The physical clock driving the output stage of a receiver must not change from one to the next CVS to avoid disruptions in the physical video output signal and to minimize the number of blank output pictures (if any).

When both, the picture resolution and the sample aspect ratio change in the second of two consecutive CVSes in the bitstream, its corresponding aspect_ratio_idc value may be chosen such that the 2D size and aspect ratio of the spatial span implied by the output pictures of the second CVS equals the implied spatial span corresponding to the first of the two consecutive CVSes.  The signalled respective aspect_ratio_idc values corresponding to the two consecutive CVSes are different because their sample aspect ratios are different.

In some cases the picture resolution changes in the second of the two consecutive CVSes but the sample aspect ratio does not.  In such case, both the horizontal and vertical resolution of the picture change.  For instance, a bitstream may have consecutive CVSes that change picture resolutions between 1280x720 and 1920x1080, or vice versa, but the sample aspect ratio remains square.  Such cases require signalling the output pictures of some CVSes with an aspect_ratio_idc that corresponds to square pixels that have a sample scale factor not equal to one.  For a video service where the 1920x1080 picture resolution is expected to be the main (or dominant) picture resolution, all the CVSes with 1920x1080 resolution will have an aspect_ratio_idc = 1 (i.e., corresponding to square pixels and a sample scale factor equal to 1).  That is, the aspect_ratio_idc =1 signals a 1:1 square pixel.  However, to fulfill the needs of the aforementioned application, the CVSes with 1280x720 resolution need to be signalled with  an aspect_ratio_idc value corresponding to square pixels and a sample scale factor equal to three over two (i.e., 1.5) so that the implied spatial span by the output pictures remains constant at 1920x1080 throughout the successive CVSes.  For these CVSes, the signalled aspect_ratio_idc value needs to imply a “1.5:1.5”  square pixel.

For a video service where the 1280x720 picture resolution is the dominant or main picture resolution, the CVSes with 1920x1080 resolution may be signalled with an aspect_ratio_idc value corresponding to square pixels and a sample scale factor equal to two over three (i.e., 0.6667).  In this case, CVSes with 1280x720 resolution would have aspect_ratio_idc = 1.   The implied spatial span by the output pictures of all the CVSes would then always be 1280x720.

In some applications a bitstream may contain CVSes with quarter-sized pictures with the same sample aspect ratio as the full-size pictures in other CVSes.  For example, a bitstream may contain a 1920x1080 CVS followed by a 960x540 CVS, both with square samples.  In such case, the signalled aspect_ratio_idc of the second CVS need to correspond to square pixels with a sample scale factor equal to 2 (i.e., 2:2 square samples) to imply a 1920x1080 spatial span.

It is proposed to use aspect_ratio_idc 17, 18, and 19 to signal the following square samples with a sample scale factor not equal to one:

| aspect_ratio_idc | Interpretation of aspect_ratio_idc (Informative - Examples of Use) |
| --- | --- |
| 17 | 1.5 : 1.5 sample aspect ratio; square sample with a scale factor equal to 3/2 (1280x720 16:9 frame output as 1920x1080 without horizontal overscan) |
| 18 | 0.667 : 0.667 sample aspect ratio; square sample with a scale factor equal to 2/3 (1920x1080 16:9 frame output as 1280x720 without horizontal overscan) |
| 19 | 2:2 sample aspect ratio; square sample with a scale factor equal to 2 (960x540 16:9 frame output as 1920x1080 without horizontal overscan) |

Alternatively, aspect_ratio_idc = 1 may be used by signalling the sample scale factor separately.  For instance, sample_scale_factor_flag  = 1 may specify the presence of a sample scale factor not equal to one.  When the sample_scale_factor_flag = 1, it specifies that the sample scale_factor_index field is present .  The sample scale_factor_index provides an index to the proposed table of sample scale factors that are deemed relevant to commercial insertion applications, such as 0.6667, 1.5, and 2.0.

Note that a bitstream may be entered at a CVS that does not contain the dominant picture resolution (i.e., the most common picture resolution expected in the bitstream) but that has the same sample aspect ratio as that of the CVS that contains the dominant picture resolution.  In such scenario, it is necessary to signal  a sample scale factor that is not equal to one to set the implied spatial span corresponding to the dominant picture resolution.

The sample_scale_factor_flag in VUI parameters signals the presence of a sample scale factor that is not equal to one.  If sample_scale_factor_flag equals 1, sample_scale_factor_index provides the sample scale factor in Table E.X

Relevant VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| sample_scale_factor_flag | u(1) |
| if(sample_scale_factor_flag) |  |
| sample_scale_factor_index | u(3) |
| } |  |

Table E.X -   Sample Scale Factor Table

| sample_scale_factor_index | sample_scale_factor |
| --- | --- |
| 0 | 1.5 (3/2) |
| 1 | 0.667 (2/3) |
| 2 | 2.0 |
| 3-7 | Reserved |

sample_scale_factor_flag equal to 1 specifies that sample_scale_factor_index is present. sample_scale_factor_flag equal to 0 specifies that sample_scale_factor_index is not present

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).