| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| sample_scale_factor_flag | u(1) |
| if(sample_scale_factor_flag) |  |
| sample_scale_factor_index | u(3) |
| } |  |
| } |  |

Table E.X -   Sample Scale Factor Table

| sample_scale_factor_index | sample_scale_factor |
| --- | --- |
| 0 | 1.0 |
| 1 | 1.5 (3/2) |
| 2 | 0.667 (2/3) |
| 3 | 2.0 |
| 4-7 | Reserved |

sample_scale_factor_flag equal to 1 specifies that sample_scale_factor_index is present. sample_scale_factor_flag equal to 0 specifies that sample_scale_factor_index is not present. When sample_scale_factor_flag is not present, its value is inferred to be equal to 0.

sample_scale_factor_index specifies the sample scale factor in Table E.X.  When sample_scale_factor_index is not present, its value is inferred to be equal to 0.