| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0288 |
| --- | --- |

| Title: | On Loop Filter Disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Rajan Joshi, Ye-Kui Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution consists of three parts. The first part recommends moving the SPS-level seq_loop_filter_across_slices_enabled_flag from SPS to PPS to place it at the same level as the loop_filter_across_tiles_enabled_flag.

The second part recommends that the HEVC deblocking filter process supports disabling the filtering of chroma block edges independent from luma block edges by defining a disable_deblocking_filter_idc in the PPS and slice header. It would be beneficial to include this functionality into the HEVC standard, which will form the base layer of the potential future HEVC SVC extension. In addition, chroma deblocking filtering may be disabled for grayscale content coding.

The third part proposes to remove the pcm_loop_filter_disable flag from the SPS and instead the in-loop filtering for the IPCM blocks is enabled or disabled using the cu_transquant_bypass_flag. This has the advantage of controlling loop filtering per IPCM block to support both lossless and lossy content, and it simplifies the HEVC draft text significantly.

# Part 1: Move seq_loop_filter_across_slices_enabled_flag From SPS To PPS

It is proposed to move the seq_loop_filter_across_slices_enabled_flag from SPS to PPS to place it at the same level as loop_filter_across_tiles_enabled_flag. The following tables include the syntax changes to SPS and PPS based on [1].

Table 1 SPS syntax changes

| …. | Descriptor |
| --- | --- |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| …. | u(1) |

Table 2 PPS changes

| …. | Descriptor |
| --- | --- |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| …. |  |

# Part 2: Chroma Deblocking Filter Process Disabling

## Introduction

The following describes the semantics of the disable_deblocking_filter_flag(idc) as it is specified in the HEVC committee draft [1] and the H.264/AVC specification [2].

### HEVC [1]

There is one disable_deblocking_filter_flag in the PPS and one in the slice header with the following semantics:

“pps_disable_deblocking_filter_flag equal to 1 specifies that the operation of deblocking filter shall not be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. pps_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. When not present, the value of pps_disable_deblocking_filter_flag is inferred to be equal to 0.

slice_header_disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall not be applied for the current slice. slice_header_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for the current slice.”

### H.264/AVC [2]

“disable_deblocking_filter_idc specifies whether the operation of the deblocking filter shall be disabled across some block edges of the slice and specifies for which edges the filtering is disabled. When disable_deblocking_filter_idc is not present in the slice header, the value of disable_deblocking_filter_idc shall be inferred to be equal to 0. The value of disable_deblocking_filter_idc shall be in the range of 0 to 2, inclusive.”

To summarize:

- 0: enables deblocking of the internal edges of current slice, as well as slice boundary edges

- 1: disables deblocking of the internal edges of current slice, as well as slice boundary edges

- 2: disables deblocking of slice boundary edges of current slice, while deblocking of internal slice edges is enabled

### H.264/AVC SVC Extension Deblocking Filter Process [2]

“disable_deblocking_filter_idc specifies whether the operation of the deblocking filter shall be disabled across some block edges of the slice, specifies for which edges the filtering is disabled, and specifies the order of deblocking filter operations. When disable_deblocking_filter_idc is not present in the slice header, the value of disable_deblocking_filter_idc shall be inferred to be equal to 0. The value of disable_deblocking_filter_idc shall be in the range of 0 to 6, inclusive.

- disable_deblocking_filter_idc equal to 0 specifies that all luma and chroma block edges of the slice are filtered.

- disable_deblocking_filter_idc equal to 1 specifies that deblocking is disabled for all block edges of the slice.

- disable_deblocking_filter_idc equal to 2 specifies that all luma and chroma block edges of the slice are filtered with exception of the block edges that coincide with slice boundaries.

- disable_deblocking_filter_idc equal to 3 specifies a two stage deblocking filter process for the slice: After filtering all block luma and chroma block edges that do not coincide with slice boundaries (as if disable_deblocking_filter_idc were equal to 2), the luma and chroma block edges that coincide with slice boundaries are filtered.

- disable_deblocking_filter_idc equal to 4 specifies that all luma block edges of the slice are filtered, but the deblocking of the chroma block edges is disabled.

- disable_deblocking_filter_idc equal to 5 specifies that all luma block edges of the slice are filtered with exception of the block edges that coincide with slice boundaries (as if disable_deblocking_filter_idc were equal to 2), and that deblocking for chroma block edges of the slice is disabled.

- disable_deblocking_filter_idc equal to 6 specifies that the deblocking for chroma block edges is disabled and that the two stage deblocking filter process is used for luma block edges of the slice: After filtering all block luma block edges that do not coincide with slice boundaries (as if disable_deblocking_filter_idc were equal to 2), the luma block edges that coincide with slice boundaries are filtered.

When no_inter_layer_pred_flag is equal to 1 or tcoeff_level_prediction_flag is equal to 1, the value of disable_deblocking_filter_idc shall be in the range of 0 to 2, inclusive.”

## Problem Description

Comparing the semantics of the disable_deblocking_filter_idc in the H.264/AVC base layer and SVC enhancement layers with the HEVC semantics of the disable_deblocking_filter_flag (PPS and slice header):

- In H.264/AVC and SVC, disable_deblocking_filter_idc equal to 2 disables deblocking filtering of the slice boundary. In HEVC, this functionality is supported by the seq_loop_filter_across_slices_enabled_flag (SPS or potentially PPS – see part 1) and the slice_loop_filter_across_slices_enabled_flag (slice header). The latter flags control all loop filtering across slice boundaries (deblocking, SAO, ALF).

- In the H.264/AVC SVC extension (enhancement layers), the disable_deblocking_filter_idc values 3 and 6 enable a two-stage deblocking filter process. In the first stage, non-slice boundary luma/chroma edges are filtered, followed by the slice boundary edges in the second stage. In HEVC, such a two-stage deblocking filter process is less relevant, because there are fewer dependencies between vertical and horizontal edge filtering compared with H.264/AVC.

- In the H.264/AVC SVC extension (enhancement layers), the disable_deblocking_filter_idc values 4, 5, and 6 disable the deblocking filter process for all chroma block edges (not for base layer and excluding other conditions).

The current HEVC deblocking filter process does not support disabling filtering of chroma block edges independent from luma block edges. It would be beneficial to include this functionality into the HEVC standard, which will form the base layer of the potential future HEVC SVC extension. In addition, chroma deblocking filtering may be disabled for grayscale content coding.

## Proposal Description

It is proposed to expand the HEVC deblocking filter adjustment parameters with the option to disable the deblocking filter process for chroma block edges. The proposed disable_deblocking_filter_idc is signaled in the PPS (pps_disable_deblocking_filter_idc) and slice header (slice_header_disable_deblocking_filter_idc). Table 3 and Table 4 illustrate the PPS and slice header syntax changes compared with [2]:

Table 3 Proposed PPS syntax

| … | Descriptor |
| --- | --- |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_idc | ue(v) |
| if( pps_disable_deblocking_filter_idc != 1) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |

Table 4 Proposed slice header syntax

| … | Descriptor |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_overriding_flag ) { |  |
| slice_header_disable_deblocking_filter_idc | ue(v) |
| if( slice_header_disable_deblocking_filter_idc != 1) { |  |
| slice_header_beta_offset_div2 | se(v) |
| slice_header_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ slice_header_disable_deblocking_filter_idc != 1 ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| … |  |

The semantics of the proposed HEVC disable_deblocking_filter_idc is as follows:

pps_disable_deblocking_filter_idc equal to 1 specifies that the operation of deblocking filter shall not be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. pps_disable_deblocking_filter_idc equal to 0 specifies that the operation of the deblocking filter shall be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. pps_disable_deblocking_filter_idc equal to 2 specifies that luma block edges of the slice are filtered, but the deblocking of the chroma block edges is disabled. When not present, the value of pps_disable_deblocking_filter_idc is inferred to be equal to 0.

slice_header_disable_deblocking_filter_idc equal to 1 specifies that the operation of the deblocking filter shall not be applied for the current slice. slice_header_disable_deblocking_filter_idc equal to 0 specifies that the operation of the deblocking filter shall be applied for the current slice. slice_header_disable_deblocking_filter_idc equal to 2 specifies that luma block edges of the slice are filtered, but the deblocking of the chroma block edges is disabled.

# Part 3: On IPCM Loop Filter Disabling and cu_transquant_bypass_flag

## Proposal Description

It is proposed to remove the pcm_loop_filter_disable flag from the SPS. Instead, the in-loop filtering for the IPCM blocks is enabled or disabled using the cu_transquant_bypass_flag as follows. In the HEVC specification [1], the pcm_flag and cu_transquant_bypass_flag can be 1 at the same time. If this happens, the cu_transquant_bypass_flag is used to enable or disable in-loop filtering for IPCM blocks at a block level instead of over the entire sequence.

Table 5

| pcm_flag | cu_transquant_bypass_flag | In-loop filtering |
| --- | --- | --- |
| 0 | 0 | enabled |
| 0 | 1 | disabled |
| 1 | 0 | enabled |
| 1 | 1 | disabled |

In Table 5, if the pcm_flag equals 0 and cu_transquant_bypass_flag is present (if transquant_bypass_enable_flag equals 1), then cu_transquant_bypass_flag equals to 1 means that the transform, quantization and all loop filters are bypassed for the “lossless” coding unit, otherwise, none of the former are bypassed for the coding unit. If the pcm_flag equals 1 and cu_transquant_bypass_flag is present (if transquant_bypass_enable_flag equals 1), then in the table above if cu_transquant_bypass_flag equals 1 then all loop filters are bypassed for the PCM codeblock (actually transform and quantization are also not applied to PCM), otherwise, the loop filters are applied to the PCM codeblock.

Note that the cu_transquant_bypass_flag may be renamed cu_transquant_loopfilter_bypass_flag to more accurately reflect its functionality.

In order to have the cu_transquant_loopfilter_bypass_flag signaled for IPCM, the condition based on the transquant_bypass_enable_flag is modified to “if (transquant_bypass_enable_flag or pcm_flag)”. In addition, the signaling of the cu_transquant_loopfilter_bypass_flag is moved towards the end of the coding unit syntax, but before the “no_residual_data_flag”. This is required for the pcm_flag to be available. Note that in “IPCM burst mode”, multiple IPCM codeblocks may be signaled together. In this case, only one cu_transquant_loopfilter_bypass_flag is signaled for all consecutive IPCM blocks and the flag value applies to all consecutive IPCM codeblocks.

The following tables illustrate the syntax changes based on [1].

Table 6 SPS syntax change (yellow highlight)

| … |  |
| --- | --- |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| … |  |

Table 7  Coding unit syntax change (yellow highlight)

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| …. |  |
| if( transquant_bypass_enable_flag ¦¦ pcm_flag ) { |  |
| cu_transquant_loopfilter_bypass_flag | ae(v) |
| } Note: cu_transquant_loopfilter_bypass_flag is moved towards the end of coding unit syntax, before the “no_residual_data_flag” |  |
| if( !pcm_flag ) { |  |
| if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| …. |  |

# Conclusions

The first part recommends adopting moving the SPS-level seq_loop_filter_across_slices_enabled_flag from SPS to PPS to place it at the same level as the loop_filter_across_tiles_enabled_flag.

The second part recommends adopting that the HEVC deblocking filter process supports disabling the filtering of chroma block edges independent from luma block edges by defining the disable_deblocking_filter_idc in the PPS and slice header. It is beneficial to include this functionality into the HEVC standard, which will form the base layer of the potential future HEVC SVC extension. In addition, chroma deblocking filtering may be disabled for grayscale content coding.

The third part proposes adopting the removal of the pcm_loop_filter_disable flag from the SPS and instead the in-loop filtering for the IPCM mode codeblocks is enabled or disabled using the cu_transquant_bypass_flag. This has the advantage of controlling loop filtering per IPCM block to support both lossless and lossy content, and it simplifies the HEVC draft text significantly.

# Committee Draft Text Changes

## Part 1: Move seq_loop_filter_across_slices_enabled_flag From SPS To PPS

See Table 1 for SPS syntax change and Table 2 for PPS syntax change.

## Part 2: Chroma Deblocking Filter Process Disabling

See Table 3 for PPS syntax change and Table 4 for slice header syntax change followed by the semantics.

## Part 3: On IPCM Loop Filter Disabling and cu_transquant_bypass_flag

See Table 6 for SPS syntax change and Table 7 for coding unit syntax change.

Other changes such as removal of all occurrences of pcm_loop_filter_disable_flag [1]:

8.7.2.4.7 Filtering process for a luma sample

“…

When nDp is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

…”

8.7.2.4.8 Filtering process for a chroma sample

“…

When one or more of the following conditions are true, the filtered sample value, p0’ is substituted by the corresponding input sample value p0.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

When one or more of the following conditions are true, the filtered sample value, q0’ is substituted by the corresponding input sample value q0.

–	pcm_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

–	cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

…”

8.7.3.1 Coding tree block modification process

“…

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- saoTypeIdx is equal to 0.

…”

8.7.4.3 Coding tree block filtering process for luma samples

“…

- If one or more of the following conditions are true, the adaptive loop filter process is not applied for the current coding block.

  - pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1.

  - cu_transquant_bypass_flag is equal to 1.

…”

8.7.4.4 Coding tree block filtering process for chroma samples

“…

- If one or more of the following conditions are true, the adaptive loop filter process is not applied for the current coding block.

  - pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1.

  - cu_transquant_bypass_flag is equal to 1.

…”

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” 9th JCT-VC Meeting, Geneva, Switzerland, Apr.-May 2012, Doc. JCTVC-I1003_d5

[2] “Advanced video coding for generic audiovisual services”, Recommendation ITU-T H.264, Mar. 2010

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).