| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0289 |
| --- | --- |

| Title: | On parallel partitions in profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Aaron Wells, Chad Fogg 2975 San Ysidro Way Santa Clara, CA 95051 Address line 3 | Tel: Email: | +1 408 734 8888 awells@ambarella.com chadfogg@gmail.com |
| Source: | Ambarella and Harmonic Inc. |  |  |

_____________________________

# Abstract

This proposal requests that among the final generic HEVC bitstream restrictions, the smallest tile dimensions permitted are: 512 pixels wide (rather than the current Annex A limit of 384 pixels) by 256 pixels tall (height is not currently restricted).  It is desirable that tiles be permitted to encompass the full size of the picture in the vertical and/or horizontal dimensions.  In particular, the authors advocate that the final restrictions should permit an encoder to optionally partition a 4K @ 60 fps (Level 5.2) sequence into an arrangement of 4 approximately uniformly spaced tile columns with no tile rows (i.e. 4x1 geometry). Should mandatory parallelism be imposed on bitstreams (max tile size), then it is requested that the total pixels per tile not exceed 2,228,224 (the current MaxLumaFS values for Level 4.2) for any level.  Finally, if Main Profile is to have any parallel partition tools at all, then tiles should be included (as per current HEVC draft), or both tiles and wavefront tools be placed into a parallel partition profile though used mutually exclusively within a sequence (as per current HEVC draft). The authors recognize that the market place will implement both these parallel partition schemes anyway, thus creating not just another “But VLC will play it!” scenario, rather, high volume semiconductor decoders will be obligated as well.

# Proposal

The following generic restrictions on bitstreams belonging to profiles that permit tiles (tiles_or_entropy_coding_sync_idc = 1) are suggested:

Minimum width:  (column_width*CtbSize >=512)

Minimum height:  (row_height*CtbSize) >=256

Maximum height: PicHeightInCtbs (i.e. no additional restrictions)

Maximum width: PicWidthInCtbs (i.e. no additional restrictions)

Should Main Profile (or a Main Profile + Tiles) mandate upper limits on tile dimensions, the size should be limited to the effective processing capabilities of a single-core 1080p60 decoder:

column_width * row_height * CtbSize * CtbSize <= 2,228,224 pixels

# Patent rights declaration(s)

Ambarella and Harmonic do not have any current or pending patent rights relating to the technology described in this contribution.