| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0291 |
| --- | --- |

| Title: | Instructive (and sometimes evil) conformance bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Aaron Wells 2975 San Ysidro Way Santa Clara, CA 95051 | Tel: Email: | +1 408 734 8888 chadfogg@gmail.com awells@ambarella.com |
| Source: | Harmonic and Ambarella |  |  |

_____________________________

# Abstract

The authors believe that conformance bitstreams that do not push the legal limit permitted by the Profile & Level can lull the implementer into under-designing the performance capabilities of their decoders.  Once decoders are discovered to crash or drop frames under some legal stream conditions, a new, canonical interoperability point is established within industry that must be tracked by encoder vendors for the lifetime of the specification. For HEVC, it is highly recommended that JCT create test definitions and example streams that maximize the performance with the aim of creating one true interoperability point for each profile. If the performance is judged to be too high for implementers to meet, then JCT should lower the profile and level limits to match baseline implementation expectations. Otherwise, there are as many potential profiles & levels as there are decoder designs that must be supported by encoders.

# Introduction

There is no true AVC conformance point in practice. Most decoders deployed today will drop frames when passed a legitimate, though improbable, performance intense bitstream.  Some decoders are known to crash when presented low-complexity streams that include obscure corner cases that the designer did not consider. Other still, crash due to bugs that may have been discovered had more thorough test streams been exercised, that for example, tried more syntax combinations.

The basic design philosophy of the proposed test streams is to pack the maximum number of elements of a given type (motion vectors, coefficients, etc.) sustained over the longest buffer-permitted time window. A hypothetical decoder that approximates what most vendors will likely follow a common structure (hardwired, DSP, CPU) that can be used as the basis of designing the particular values of elements. For example, motion vectors that miss the most DRAM page boundaries while referencing random locations in the frame buffer to minimize cache hits.

Part 4 of the MPEG-2 | H.262 served as the conformance test stream set designed towards these patterns designed by adhoc group [1][2]. A number of commercial test bitstream products followed from companies such as Sarnoff and Allegro. AVC test streams http://wftp3.itu.int/av-arch/jvt-site/draft_conformance/ provide a test set for both consumer and professional (4:2:2) profiles [3]. The 8-bit 4:2:0 High Profile test streams, the set which is most analogous to HEVC version 1, are summarized in the two tables below.

| Bitstream | entropy_coding_mode_flag | Cabac_init_idc adaptively changed in slice header? | Decoding of level prefix > 16 | Direction prediction (T=temporal, S=spatial) | Direct_8x8_inference_flag | 4x4 transform (Y=Yes, N=No) | Seq_scaling_matrix_flag | Pic_scaling_matrix_flag | PAFF | mb_adaptive_field_frame_flag | Disable_deblocking_filter_idc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FREH-1, FREH-28 | 0 |  | Y | S | 0 | N | 1 | 1 |  |  |  |
| FREH-2, FREH-29 | 1 | N |  | T | 0 | Y | 1 | 1 |  |  |  |
| FREH-3, FREH-30 | 1 | Y |  | S | 1 | Y | 1 | 1 |  |  |  |
| FREH-4, FREH-31 | 1 | Y |  | S | 1 | Y | D |  | Y |  |  |
| FREH-5, FREH-32 | 1 | Y |  | S | 1 | Y | D |  |  | 1 |  |
| FREH-6, FREH-33 | 0 |  | Y | T | 1 | N | 1 | 1 | Y | 1 |  |
| FREH-7, FREH-34 | 1 |  |  | T | 1 | Y | 1 | 1 | Y | 1 |  |
| FREH-8 | 1 |  |  | T | 1 | Y | 0 | 0 | Fr |  | 1 |
| FREH-9 | 1 |  |  | T | 1 | Y | 0 | 0 |  |  |  |
| FREH-10 | 1 |  |  | T | 1 | Y | 0 | 0 | Fe |  | 1 |
| FREH-11 | 1 |  |  | T | 1 | Y | 0 | 0 | Fe |  |  |
| FREH-12, FREH-39 | 1 |  |  | T |  | Y | D |  | Fr |  |  |
| FREH-13, FREH-14, FREH-15 | 1 |  |  | T | 1 | Y | D |  | Fr |  |  |
| FREH-16 | 1 |  |  | T | 1 | N | 1 |  | Fr |  |  |
| FREH-17 | 1 |  |  | T | 1 | N | 1 |  | Y | 1 |  |
| FREH-18 | 0 |  |  | T | 1 | Y | 0 | 0 | Fr |  |  |
| FREH-19 | 0 |  |  | T | 1 | Y | 0 | 0 | Fr |  | 1 |
| FREH-20 | 0 |  |  | T | 1 | Y | 0 | 0 | Fe |  | 1 |
| FREH-21 | 0 |  |  | T | 1 | Y | 0 | 0 | Fe |  |  |
| FREH-22 | 0 |  |  | S | 1 | Y | D |  | Fe |  |  |

All streams (FREH-1 and FREH-45) have 8x8 transforms. Some also have 4x4  as indicated in the table above.

All streams (FREH-1 through FREH-45) have single slice, I, P, and B pictures with NAL units encapsulated into the byte stream format specified in Annex B. pic_order_cnt_type = 0.

FREH-23 through FREH-45 have both 8x8 and 4x4 transform.

| Bitstream | entropy_coding_mode_flag (0=CAVLC, 1=CABAC) | Direction prediction (T=temporal, S=spatial) | Direct_8x8_inference_flag | seq_scaling_matrix_present_flag (D=default matrix used) | pic_scaling_matrix_flag | PAFF (P), all fields (Fe), All Frames (Fr) | mb_adaptive_field_frame_flag | Disable_deblocking_filter_idc | Chroma_format_idc | Reference picture list reordering and MMCO | Second_chroma_qp_index_offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FREH-23 | 0 | S | 1 | D |  | Fe |  |  |  |  |  |
| FREH-24 | 0 | S | 1 | D |  | Fr | 1 |  |  |  |  |
| FREH-25 | 0 | T | 1 | 1 |  | Fr |  |  | 0 |  |  |
| FREH-26 | 1 | T | 1 |  |  | Fr |  |  | 0 |  |  |
| FREH-27 | 1 | T |  | 1 |  | Fr |  |  |  |  | 2 |
| FREH-35 | 1 | T | 1 | 1 | 1 | Fr |  | 1 |  |  |  |
| FREH-36 | 1 | T | 1 | 1 | 1 | Fr |  |  |  |  |  |
| FREH-37 | 1 | T | 1 | 1 | 1 | Fe |  | 1 |  |  |  |
| FREH-38 | 1 | T | 1 | 1 | 1 | Fe |  |  |  |  |  |
| FREH-40, FREH-41 | 1 | S | 1 | 0 | 0 | Fr |  |  |  | Y |  |
| FREH-42 | 1 | S | 1 | 0 | 0 | Fr | 1 |  |  | Y |  |
| FREH-43 | 1 | S | 1 | 0 | 0 | Fr | 1 |  |  |  |  |
| FREH-44 | 0 | S | 1 | 0 | 0 | Fr | 1 |  |  |  |  |
| FREH-45 | 1 | S | 1 | 1 | 0 | Fr |  |  |  | Y |  |

# Proposed Test bitstreams

In addition to the patterns described in section 6.6 (Specification of the test bitstreams) in [3], the following further test bitstreams patterns are proposed which are notably lacking in [3]. Several may be combined into one bitstream.

- Maximum number of motion vectors permitted per LCU / treeblock.

- Motion vectors are random, maximize page DRAM misses, that also minimize cache hits.

- reference picture lists change as frequently as legally permitted in both PPS and Slice headers.

- SliceRate is the maximum legally permitted, with the longest run possible of one slice per LCU/CTU.

- A bitstream that tests all deblocking edge modes.

- Maximum number of tiles, changing geometry on every picture

- Pictures with minimum TB/LCU size with max. SAO signaling rate.

# References

[1] C. Fogg, “Report on the ad-hoc group for video conformance testing”, MPEG 94/008

[2] C. Fogg , “Report on the ad-hoc group for video conformance testing”, MPEG 94/202

[3] H.261.1 Conformance Testing

# Patent rights declaration(s)

Harmonic and Ambarella do not have any current or pending patent rights relating to the technology described in this contribution.