| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0296 M25624 |
| --- | --- |

| Title: | Cross-check of SAO on/off flags coding in JCTVC-J0269 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Do-Kyoung Kwon | Tel: Email: Tel: Email: | +1-214-480-1562 wskim@ti.com +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check results for SAO of/off flags coding proposed by Sony in JCTVC-J0296. Source codes and test results provided by the proponents are verified in this document. It has been confirmed that source codes are modified on top of HM-7 as described in JCTVC-J0296 and test results exactly match to those provided by the proponents.

# Introduction

In JCTVC-J0269 [1], it is proposed to decouple SAO on/off from SAO type and jointly code them for all color components. Further simplification is made compared to the previous work, JCTVC-I0193, by removing a mapping table and reducing number of context coded bins.

In addition, it is tested in conjunction with SAO type simplification proposed in JCTVC-J0268.

# Experimental Results

Two sets of test results are provided by the proponents under the common test condition [2], where the first set includes the results of SAO on/off flag coding, and the second set includes the results when SAO on/off flag coding is combined with SAO type coding proposed in JCTVC-J0268. It is confirmed that the test results exactly match to those provided by the proponent. Note that exact encoding/decoding time are not measured.

Table 1. BD-rate results of the SAO on/off flags coding compared to HM-7.0 anchor.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.2% |
| Class B | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | -0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | -0.1% |
| Overall | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.7% | -0.5% | 0.0% | -0.2% | -0.4% |
| Class B | -0.1% | -0.4% | -0.5% | -0.1% | -0.3% | -0.3% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | -0.5% | -0.4% | -0.2% | -0.4% | -0.3% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.3% |
|  | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.3% |
| Class F | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.0% | -1.1% | -0.1% | -0.6% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.1% | -0.3% | -0.1% |
| Class D | -0.3% | 0.0% | 0.3% | -0.2% | 0.1% | -0.1% |
| Class E | -0.6% | -2.2% | -2.5% | -0.7% | -0.8% | -0.7% |
| Overall | -0.3% | -0.8% | -0.9% | -0.3% | -0.4% | -0.4% |
|  | -0.3% | -0.7% | -0.8% | -0.3% | -0.4% | -0.3% |
| Class F | -0.3% | -0.9% | -0.6% | -0.1% | -0.7% | -0.3% |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -1.0% | -1.3% | -0.2% | -0.4% | -0.7% |
| Class C | -0.2% | -0.7% | -0.3% | -0.2% | -0.4% | -0.1% |
| Class D | -0.3% | 0.5% | 0.4% | -0.3% | -0.6% | -0.7% |
| Class E | -0.9% | -3.0% | -2.6% | -0.8% | -0.5% | -1.1% |
| Overall | -0.4% | -0.9% | -0.9% | -0.3% | -0.5% | -0.6% |
|  | -0.4% | -0.9% | -0.9% | -0.3% | -0.5% | -0.5% |
| Class F | -0.3% | -0.6% | -0.8% | -0.3% | -0.3% | -0.8% |
| Enc Time[%] | 106% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |

Table 2. BD-rate results of the SAO on/off flags coding with SAO type coding simplification compared to HM-7.0 anchor.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.4% | -0.5% | 0.1% | -0.3% | -0.4% |
| Class B | 0.1% | -0.3% | -0.5% | 0.1% | -0.3% | -0.5% |
| Class C | 0.1% | -0.1% | -0.2% | 0.1% | -0.2% | -0.3% |
| Class D | 0.1% | -0.3% | -0.4% | 0.0% | -0.1% | -0.2% |
| Class E | 0.1% | -0.7% | -0.9% | 0.1% | -0.4% | -0.6% |
| Overall | 0.1% | -0.4% | -0.5% | 0.1% | -0.3% | -0.4% |
|  | 0.1% | -0.3% | -0.5% | 0.1% | -0.3% | -0.4% |
| Class F | 0.1% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -1.2% | -1.1% | -0.1% | -0.6% | -0.8% |
| Class B | -0.1% | -0.7% | -0.8% | -0.1% | -0.5% | -0.7% |
| Class C | -0.3% | -0.4% | -0.5% | -0.2% | -0.3% | -0.5% |
| Class D | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.7% | -0.7% | -0.2% | -0.5% | -0.7% |
|  | -0.3% | -0.7% | -0.7% | -0.2% | -0.5% | -0.7% |
| Class F | -0.2% | -0.3% | -0.5% | -0.3% | -0.5% | -0.8% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.5% | -1.6% | -0.2% | -0.7% | -1.1% |
| Class C | -0.2% | -0.5% | -0.7% | -0.2% | -0.7% | -0.4% |
| Class D | -0.4% | -0.3% | 0.1% | -0.3% | -0.2% | -0.1% |
| Class E | -0.6% | -3.0% | -3.2% | -0.7% | -1.1% | -0.6% |
| Overall | -0.3% | -1.3% | -1.3% | -0.3% | -0.7% | -0.6% |
|  | -0.3% | -1.2% | -1.2% | -0.3% | -0.6% | -0.5% |
| Class F | -0.3% | -1.5% | -0.6% | -0.2% | -0.6% | 0.2% |
| Enc Time[%] | 101% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -1.4% | -1.8% | -0.2% | -0.7% | -1.0% |
| Class C | -0.2% | -0.8% | -0.8% | -0.1% | -0.5% | -0.4% |
| Class D | -0.3% | -0.1% | -0.3% | -0.2% | 0.0% | -0.8% |
| Class E | -0.9% | -3.2% | -3.2% | -0.9% | -1.6% | -1.2% |
| Overall | -0.4% | -1.3% | -1.4% | -0.3% | -0.7% | -0.8% |
|  | -0.4% | -1.2% | -1.4% | -0.3% | -0.6% | -0.8% |
| Class F | -0.6% | -0.6% | -1.2% | -0.3% | -0.5% | -0.7% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |

# Conclusion

In this contribution SAO on/off flags coding proposed in JCTVC-J0269 is cross-checked. It is confirmed that the test results exactly match to those provided by the proponent. This proposal simplifies SAO parameter coding in HM-7.0, and at the same time it improves coding efficiency. Therefore, it is recommended to review this proposal with other similar proposals and consider change of the current SAO parameter coding scheme in HM-7.0.

# Reference

- J. Xu and A. Tabatabai “AHG6: SAO on/off flags coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0269, Jul. 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.