| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0297 WG11 Number: m25625 |
| --- | --- |

| Title: | AHG5: High throughput binarization method for reference index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Louis Kerofsky, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In HM-7.0, ref_idx requires 15 context coded bins in worst-case scenario.  In order to improve the throughput efficiency, this contribution proposes a new binarization method combining truncated unary and fixed length coding (TUFLC). In the proposal, the first four bins are context coded and the remaining bins are bypass coded.  It is asserted that the proposed method 1 and method 2 reduce the worst case number of context coded bins from 15 bins to 4 bins and 2 bins, respectively. It is also asserted that the proposed method reduces the worst case number of bins (both context coded and bypassed coded) from 15 bins to 8 bins.

# Proposed reference index coding with TU-FLC binarization

The number of context-coded bins for reference index takes about 21% of the context-coded bins of a 16x16 inter-coded LCU in the worst-case scenario in worst-case scenario. In order to reduce this worst case number, this contribution introduces a simplified ref_idx coding based on concatenated binarization between Truncated Unary code and Fixed Length Code (TUFLC). Table 1 shows comparison between two binarization methods (HM7.0 vs. TUFLC (Method1/Method2). The red colored bins represents context coded bin.

- Method 1: TUFLC with 4 context bins

- Method 2: TUFLC with 2 context bins, where the same context model introduced in [1] is used

Table 1. Comparison between different binariztion methods.

| Ref index | HM7.0 | Method 1 | Method 2 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 10 | 10 | 10 |
| 2 | 110 | 110 | 110 |
| 3 | 1110 | 1110 | 1110 |
| 4 | 11110 | 1111 xxxx | 1111 xxxx |
| 5 | 111110 | 1111 xxxx | 1111 xxxx |
| 6 | 1111110 | 1111 xxxx | 1111 xxxx |
| 7 | 11111110 | 1111 xxxx | 1111 xxxx |
| 8 | 111111110 | 1111 xxxx | 1111 xxxx |
| 9 | 1111111110 | 1111 xxxx | 1111 xxxx |
| 10 | 11111111110 | 1111 xxxx | 1111 xxxx |
| 11 | 111111111110 | 1111 xxxx | 1111 xxxx |
| 12 | 1111111111110 | 1111 xxxx | 1111 xxxx |
| 13 | 11111111111110 | 1111 xxxx | 1111 xxxx |
| 14 | 111111111111110 | 1111 xxxx | 1111 xxxx |
| 15 | 111111111111111 | 1111 xxxx | 1111 xxxx |

The throughput performance between HM 7.0 and proposed method (Method 1/Method 2) in worst-case scenario has also been compared in Table 2.

|  | HM7.0 | Method1 | Method2 |
| --- | --- | --- | --- |
| Max_total_bins | 15 | 8 | 8 |
| Max_ctx_bins | 15 | 4 | 2 |
| Max_bypass_bins | 0 | 4 | 6 |

# Simulation results

Simulations were performed under the common conditions [2].

Table 1. Simulation results in HM-7.0 (Method1-4context bins).

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.000% | 0.000% | 0.000% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.000% | 0.000% | 0.000% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

Table 5. Simulation results in HM-7.0 (Method2-2context bins).

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.17% | -0.15% | 0.01% | -0.13% | 0.03% |
| Class B | 0.01% | -0.02% | 0.00% | 0.00% | 0.01% | 0.02% |
| Class C | -0.01% | 0.05% | 0.06% | 0.01% | -0.03% | -0.06% |
| Class D | 0.02% | -0.04% | -0.02% | -0.02% | 0.04% | -0.01% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.02% | 0.00% | -0.03% | 0.00% |
|  | 0.01% | -0.04% | -0.03% | 0.00% | -0.02% | -0.01% |
| Class F | 0.01% | 0.06% | 0.02% | -0.01% | 0.00% | 0.04% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.23% | 0.09% | -0.01% | 0.13% | 0.07% |
| Class C | 0.01% | -0.01% | 0.00% | 0.00% | 0.06% | 0.25% |
| Class D | 0.01% | 0.21% | 0.06% | -0.03% | 0.03% | 0.04% |
| Class E | -0.04% | 0.05% | -0.27% | 0.09% | -0.34% | 0.15% |
| Overall | 0.00% | 0.13% | -0.01% | 0.01% | 0.00% | 0.13% |
|  | 0.00% | 0.07% | -0.02% | 0.01% | 0.06% | 0.19% |
| Class F | -0.07% | 0.17% | 0.09% | 0.09% | 0.44% | 0.77% |

# Conclusions

This contribution has presented a simplified ref_idx coding based on TUFLC. The proposed high throughput binarization coding method provides significant improvement of throughput performance including worst-case scenario without any coding loss. Given this level of performance and the importance of anticipating a wide variety of high throughput applications in the HEVC standard, it is recommended to adopt this method into the draft text.

# References

- V. Seregin et al, “Bypass bins for reference index coding,” JCTVC-I0352, Geneva, Apr. 2012.

- V. Sze et al, “Reduction in context coded bins for ref_idx and cu_qp_delta,” JCTVC-I0204, Geneva, Apr. 2012.

- V. Sze et al, “Bypass bins for reference index and delta QP coding,” JCTVC-I0594, Geneva, Apr. 2012

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Changes to the CD text

<Method 1>

Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ref_idx_l0 | 1 | Prefix: TU, cMax = min(num_ref_idx_l0_active_minus1, 4) Suffix: FL, whose length in bits is 4 | 2 | Table 9-4 | Prefix: 0 Suffix: na (use Decode Bypass) |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 | Table 9-4 | Prefix: 3 Suffix: na (use Decode Bypass) |
| ref_idx_l1 | 1 | TU, cMax = min(num_ref_idx_l1_active_minus1,4) Suffix: FL, whose length in bits is 4 | 2 | Table 9-4 | Prefix: 0 Suffix: na (use Decode Bypass) |
|  | 2 |  | 2 | Table 9-4 | Prefix: 3 Suffix: na (use Decode Bypass) |

<Method 2>

Table 9-3 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-4 |  | 0..2 0..1 | 3..5 2..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-4 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 2 | 4 3 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-54 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ref_idx_l0 | 1 | Prefix: TU, cMax = min(num_ref_idx_l0_active_minus1, 4) Suffix: FL, whose length in bits is 4 | 2 1 | Table 9-4 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 1 | Table 9-4 | 3 2 |
| ref_idx_l1 | 1 | TU, cMax = min(num_ref_idx_l1_active_minus1,4) Suffix: FL, whose length in bits is 4 | 2 1 | Table 9-4 | 0 |
|  | 2 |  | 2 1 | Table 9-4 | 3 2 |

Table 9-69 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| ref_idx_l0 | Table 9-4 | 0 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |
| ref_idx_l0, ref_idx_l1 | Table 9-4 | 3 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |