| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0298 WG11 Number: m25626 |
| --- | --- |

| Title: | AHG5: High throughput binarization for cu_qp_delta coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Louis Kerofsky, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In HM-7.0, cu_qp_delta requires up to 26 and 32 context coded bins in main profile and HE_10 configuration, respectively.  In order to improve the throughput efficiency, this contribution proposes a new efficient binarization method by combining truncated unary and fixed length coding (TUFLC).  Only a predefined number (cMax) of bins are context coded and the rest of the bins are bypass coded.  It is reported that the proposed method reduces the worst case number of context coded bins from 26 (or 32 in HE_10)bins to 4 bins. Moreover, it is reported that the total number of bins are reduced from 17 (or 33 in HE10) to 16 bins for all condition. The complexity reduction results in a BD-rate change from -0.02% to 0.02% on average in AHG5 common test conditions that is asserted to be negligible.

# Introduction

Since hardware is designed considering the worst case-scenario, it is important to reduce the worst case number of context coded bins.  In worst case scenario, the number of context-coded bins of cu_qp_delta syntax elements takes 26 and 32 context-coded for main profile and he_10 configuration, respectively. To improve the throughput of the CABAC, the number of context coded bins should be reduced. This is due to the fact that they are binarized with truncated unary and all the bins are context coded.  This contribution proposes a new efficient binarization method by combining truncated unary and fixed length coding (TUFLC).  Only predefined number of bins (cMax) is context coded and the rest are bypass coded.

# cu_qp_delta coding based on TUFLC binarization

This contribution describes a simplified cu_qp_delta coding method based on concatenated binarization between Truncated Unary code and Fixed Length Code (TUFLC). Table 1 shows the relationship between cu_qp_delta and codeword from two binarization method (HM7.0 vs. TUFLC with c=9, cMax=4). The red colored bins represents context coded bin. The throughput performance between HM 7.0 and proposed TUFLC in worst-case scenario has been compared in Table 2.

Table 1 Proposed binarization method (TUFLC, c=9, cMax=4)

Table 2 comparison of maximum number of total/context/bypass bins in worst-case scenario

|  | HM7.0 Main | Proposed Main | HM7.0 Main | Proposed Main |
| --- | --- | --- | --- | --- |
| Max_total_bins | 27 | 16 | 33 | 16 |
| Max_ctx_bins | 26 | 4 | 32 | 4 |
| Max_bypass_bins | 1 (sign) | 12 (11+sign) | 1 (sign) | 12 (11+sign) |

In the HEVC draft 7, significant_flag, sign_flag and abs_minus1 are coded for cu_qp_delta. Here, significant_flag is context coded and sign_flag is bypass coded. In the proposed method, some (cMax-1) bins in abs_minus1 are context coded and the rest are bypass coded. However, the alternative coding between context coding and bypass coding causes reduction in throughput performance. Therefore, sign_flag is considered to be coded right after abs_minus1 coding.

By comparing Error: Reference source not found and Error: Reference source not found, the TUFLC based cu_qp_delta coding offers the following advantages over the current HM in terms of complexity:

- Reduce the maximum numbers of bins from 33 to 16;

- Reduce the maximum numbers of context bins from 32 to 4;

- No interleaving of context-coded bins and bypass-coded bins.

# Simulation results

Simulation was conducted according to the AHG5 common test conditions based on JCTVC-I1100 using additional command options as follows;

MaxDeltaQP 		: 0  	# CU-based multi-QP optimization 
MaxCuDQPDepth  	: 3 	# Max depth of a minimum CuDQP for sub-LCU-level delta QP 
DeltaQpRD         	: 0  	# Slice-based multi-QP optimization 
RDOQ              	: 1 	# RDOQ 
ChromaQpOffset    	: 0  	# ChromaQPOffset 
ChromaQpOffset2nd 	: 0  	# ChromaQPOffset2nd 
AdaptiveQP  		: 1  	# QP adaptation based on a psycho-visual model (0: None, 1: TM5Step3) 
MaxQPAdaptationRange	: 12	# dQP range of QP adaptation

Table3 shows the summary and the followings are observed:

- Average luma BD-rate changes of 0.01% for All Intra Main, 0.00% for Random Access Main, -0.01% for Low Delay B Main.

- Average BD-rate changes of 0.02% for All Intra HE10, -0.02% for Random Access HE10, -0.02% for Low Delay B HE10.

Table 3 Summary of simulation results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.01% | 0.00% | 0.02% | 0.00% | 0.06% |
| Class B | 0.00% | 0.00% | -0.01% | 0.01% | 0.00% | 0.01% |
| Class C | 0.00% | 0.02% | 0.02% | 0.00% | 0.01% | 0.02% |
| Class D | -0.01% | 0.00% | 0.04% | 0.01% | -0.03% | -0.02% |
| Class E | 0.05% | 0.03% | 0.04% | 0.05% | 0.09% | 0.07% |
| Overall | 0.01% | 0.01% | 0.02% | 0.02% | 0.01% | 0.02% |
|  | 0.01% | 0.01% | 0.02% | 0.02% | 0.01% | 0.02% |
| Class F | 0.11% | 0.11% | 0.08% | 0.13% | 0.13% | 0.01% |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.10% | 0.10% | -0.07% | 0.15% | 0.26% |
| Class B | -0.01% | -0.06% | 0.08% | 0.00% | 0.03% | -0.05% |
| Class C | 0.02% | -0.03% | 0.17% | 0.00% | 0.00% | 0.13% |
| Class D | 0.02% | 0.28% | 0.12% | 0.01% | 0.30% | -0.12% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.11% | -0.02% | 0.11% | 0.05% |
|  | 0.00% | 0.03% | 0.08% | -0.02% | 0.11% | 0.09% |
| Class F | 0.03% | -0.12% | -0.01% | 0.13% | -0.09% | 0.18% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.02% | -0.28% | -0.09% | 0.00% | -0.04% | 0.51% |
| Class C | -0.02% | 0.24% | 0.23% | 0.01% | 0.20% | -0.05% |
| Class D | -0.01% | 0.06% | 0.64% | -0.08% | 0.09% | -0.08% |
| Class E | -0.01% | -0.31% | -0.25% | -0.04% | -0.30% | 0.61% |
| Overall | -0.01% | -0.07% | 0.14% | -0.02% | 0.00% | 0.24% |
|  | -0.01% | -0.05% | 0.13% | -0.02% | 0.03% | 0.15% |
| Class F | 0.20% | 0.79% | 0.44% | 0.25% | -0.05% | 0.74% |

# Conclusions

This contribution has presented a simplified cu_qp_delta coding based on TUFLC. The proposed high throughput coding method provides significant improvement of throughput performance including worst-case scenario without coding loss. Given this level of performance and the importance of anticipating a wide variety of high throughput applications in the HEVC standard, it is recommended to adopt this method into the draft text.

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Changes to the CD text

Table 9-1 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  |  |  |  |  |
| initValue | 119 | 119 | 119 | 119 |  |  |  |  |  |

#### Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The prefix bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	
	
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	cNum = absV − 1
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}

If absVal-1 >=cMax then the suffix absV-1-cMax is binarized using 5 bit fixed length coding process.

If absV >0 then sign data is coded as follows:

signV = ( synVal > 0 ) ? 0 : 1

put( signV )