| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0299 WG11 Number: m25627 |
| --- | --- |

| Title: | AHG4: On entry point coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

Two modifications to the coding of entry point offsets are reported.  These modifications are asserted to provide some gains in coding efficiency for scenarios where the number of bytes between entry points are small.  The first modification changes the binarization process for coding entry point offsets; the second allows signaling a “minimum entry point value” and then signaling the entry point information relative to this minimum value.  Results show the proposed scheme changes BD-rate about -0.50% on average in Class E sequence with 24 entry points per frame.

# Introduction

We consider two methods to further improve the coding efficiency of entry point offset information.  The first method modifies the binarization process for the entry point information.  Specifically, we use a combination of quotient and remainder coding, where the quotient is coded ue(v), the remainder is coded u(v), and a divisor is sent in the bit-stream.  This is in comparison to the current method of signaling the entry points using u(v).

The second method modifies the meaning of the entry point offset information in the bit-stream.  In the current draft text, the entry point information signals the difference (in bytes) between two entry points.  Here, we signal this difference relative to a minimum difference value.  This minimum difference value is also transmitted in the bit-stream.

The syntax and semantics are as follows:

Table 1 proposed entry point location information

| if( tiles_or_entropy_coding_sync_idc > 0 ) | Descriptor |
| --- | --- |
| { |  |
| num_entry_point_offsets | ue(v) |
| if (num_entry_point_offset>0) { |  |
| min_offset_minus1 | ue(v) |
| divisor_power_of_two_minus1 | ue(v) |
| for (i=0; i<num_of_tiles_minus1_slice; i++) { |  |
| entry_point_quotient[i] | ue(v) |
| entry_point_remainder[i] | u(v) |
| } |  |
| } |  |
| } |  |

num_entry_point_offsets specifies the number of entry_point_quotient[i] and entry_ponit_remainder[i] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to (num_tile_columns_minus1 +1) * (num_tile_rows_minus1+1) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

min_offset_minus1 plus 1 specifies the minimum value of the entry point offset [i] in the slice.

divisor_power_of_two_minus1 plus 1 specifies the power of two of the divisor which is equal to 2 divisor_power_of_two_minus1+1.

entry_point_quotient[i] specifies the quotient which is obtained from dividing entry_point_offset[i] by the given divisor where the divisor is equal to 2 divisor_power_of_two_minus1+1

entry_point_remainder[i] specifies the remainder which is obtained when divide entry_point_offset[i] by the given divisor which is equal to 2 divisor_power_of_two_minus1+1. The value of syntax element remainder [i] shall be in the range of 0 to 2 divisor_power_of_two_minus1+1 -1.

The entry_point_offset is then calculated as follows:

entry_point_offset [i] = entry_point_quotient [i] *(1 << (divisor_power_of_two_minus1 +1)) + entry_point_remainder [i] + min_offset_minus1 +1;

# Simulation results

We implemented the proposed method into the HM7.0 software and tested with the recommended configuration under the common QP test condition (QP=22, 27, 32, and 37).  Two experiment conditions are considered as follows;

- EXP1: 32 tiles for class A, class B, class E

- EXP2: The second experiment condition in JCTVC-F335.

Table 2. Tile partitioning for experiment 2 (in F335).

|  | Reference |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Number of slices | Slice sizes (in units of LCUs) | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
|  |  |  | horizontally | vertically | horizontally | vertically |
| A | 25 | 40 | 5 | 5 | 8 | 5 |
| B | 17 | 30 | 6 | 3 | 5 | 6 |
| C | 4 | 26 | 2 | 2 | 7 | 4 |
| D | 1 | 28 | 1 | 1 | 7 | 4 |
| E | 12 | 20 | 4 | 3 | 5 | 4 |

Table 3	 Summary of simulation results (EXP1-32 Tiles)

Table 4 Summary of simulation results (EXP2 in JCTVC-F335)

# Conclusions

This contribution reports potential coding efficiency improvements for entry point offset coding. It is recommended to adopt this method into the draft text.

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Changes to the CD text

### 7.3.3 Slice header syntax

| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| --- | --- |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| min_offset_minus1 | ue(v) |
| divisor_power_of_two_minus1 | ue(v) |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_quotient[i] | u(v) |
| entry_point_remainder[i] | u(v) |
| } |  |
| } |  |

7.4.3 Slice header semantics

min_offset_minus1 plus 1 specifies the minimum value of the entry point offset [i] in the slice.

divisor_power_of_two_minus1 plus 1 specifies the power of two of the divisor which is equal to 2 divisor_power_of_two_minus1+1.

entry_point_quotient[i] specifies the quotient which is obtained from dividing entry_point_offset[i] by the given divisor where the divisor is equal to 2 divisor_power_of_two_minus1+1

entry_point_remainder[i] specifies the remainder which is obtained when divide entry_point_offset[i] by the given divisor which is equal to 2 divisor_power_of_two_minus1+1. The value of syntax element remainder [i] shall be in the range of 0 to 2 divisor_power_of_two_minus1+1 -1.