| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0301_r1 WG11 Number: m25629 |
| --- | --- |

| Title: | Cross-verification of Luma intra planar prediction filter (JCTVC-J0271) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of the JCTVC-J0271, “Luma intra planar prediction filter.” The software was examined, and simulations were performed for the cases when the filter is removed and for when the filter is applied to both DC and planar prediction. The simulation results performed for this cross-verification match those provided by the proponents. The software changes are consistent with the proposal.

# Introduction and examination of software

In JCTVC-J0271 “Luma intra planar prediction filter” [1], when both neighboring blocks are available, HM applies xDCPredFiltering() if the Intra prediction mode for the current prediction block is DC. JCTVC-J0271 [1] proposes applying the filter also when the Intra prediction mode is planar. Performance with the filter turned off is also tested. The changes to the software are simply modify the check for Intra prediction mode so that the filter is applied when the Intra prediction mode is DC or planar.

# Simulation results

The proposal was compared to a reference of HM 7.0, using common test conditions [2].

## Filter off for all modes

When the filter was turned off for all modes, the BD-Rate results were as follows. Because JCTVC-J0271 reported results with 3 fractional digits, we do the same here.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.207% | 0.344% | 0.341% | 0.237% | 0.288% | 0.302% |
| Class B | 0.113% | 0.210% | 0.230% | 0.149% | 0.217% | 0.200% |
| Class C | 0.085% | 0.117% | 0.175% | 0.076% | 0.143% | 0.175% |
| Class D | 0.069% | 0.132% | 0.134% | 0.057% | 0.172% | 0.131% |
| Class E | 0.086% | 0.114% | 0.101% | 0.053% | 0.118% | 0.170% |
| Overall | 0.113% | 0.188% | 0.203% | 0.119% | 0.193% | 0.197% |
|  | 0.117% | 0.187% | 0.204% | 0.123% | 0.192% | 0.199% |
| Class F | -0.182% | -0.001% | 0.021% | -0.200% | -0.326% | -0.114% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

## Filter applied with both DC and planar modes

Here, the filter was applied for the planar Intra prediction mode as well as for DC mode.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.109% | -0.113% | -0.142% | -0.132% | -0.156% | -0.198% |
| Class B | -0.057% | -0.108% | -0.145% | -0.089% | -0.133% | -0.198% |
| Class C | -0.080% | -0.133% | -0.112% | -0.068% | -0.122% | -0.120% |
| Class D | -0.085% | -0.102% | -0.070% | -0.064% | -0.124% | -0.181% |
| Class E | -0.227% | -0.197% | -0.190% | -0.182% | -0.204% | -0.109% |
| Overall | -0.103% | -0.126% | -0.129% | -0.102% | -0.144% | -0.166% |
|  | -0.104% | -0.129% | -0.122% | -0.103% | -0.142% | -0.156% |
| Class F | -0.096% | -0.030% | -0.113% | -0.066% | -0.191% | -0.067% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well. The modifications to the software are consistent with the simplifications described in the proposal.

# References

- C. Auyeung, “Luma intra planar prediction filter,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0271, Stockholm, SE, July 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, Geneva, CH, May 2012.