| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0302 |
| --- | --- |

| Title: | Restricted usage of motion vectors for long-term reference picture in motion vector prediction process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Youngo Park JeongHoon Park | Email: | ilkoo.kim@samsung.com youngo.park@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, two solutions are proposed to improve the current design by marking motion vectors from LTRPs as unavailable instead of using it without scaling. The first solution is not to insert motion vectors into the candidate list by marking it unavailable if the motion vectors are from LTRPs. The motion vector predictor is marked as unavailable before the scaling process is performed. The benefit of this approach is that by removing the inefficient motion vectors which are scaled or non-scaled motion vectors from LTRPs, more efficient motion vectors can be included in the list. The second solution is that motion vectors are not inserted into the candidate list when the scaled motion vectors are likely to be inefficient. In this solution, the differentiation between LTRP and short-term reference picture (STRP) is not required. When POC difference (Tr) between reference picture of current PU and reference picture of candidate PU (co-located PU or neighbor PU) are larger than pre-determined threshold (THpoc_diff), motion vector scaling is not used. The motion vector predictor is marked as unavailable before the scaling process is performed. Both solutions are applied to both spatial and temporal motion vector prediction. Although average coding efficiency gains from the second solution for each configuration are negligible, coding efficiency gains for Class F under Low delay B main and Low delay B HE10 are 0.1% and 0.1%, respectively.

# Introduction and problem statement

At the previous Geneva meeting, a MVP scaling issue for long-term reference pictures (LTRPs) was raised and a solution from JCTVC-I0422 [3] was adopted. In the solution, if a motion vector is from LTRPs, the motion vector is utilized without scaling in Merge and AMVP process. This approach is directly inherited from H.264/AVC, which deals with the case with the same way for temporal direct mode.

In this contribution, two solutions are proposed to improve the current design by marking motion vectors from LTRPs as unavailable instead of using it without scaling.

# Removing motion vectors from LTRPs in the candidate list

## Solution 1

The first solution is not to insert motion vectors into the candidate list by marking it unavailable if the motion vectors are from LTRPs. The motion vector predictor is marked as unavailable before the scaling process is performed. The benefit of this approach is that by removing the inefficient motion vectors which are scaled or non-scaled motion vectors from LTRPs, more efficient motion vectors can be included in the list. This method can be utilized for temporal motion vector predictor for Merge and AMVP process, and spatial motion vector prediction for AMVP process.

## Solution 2

The second solution is that motion vectors are not inserted into the candidate list when the scaled motion vectors are likely to be inefficient. In this solution, the differentiation between LTRP and short-term reference picture (STRP) is not required. The principle of the decision is as follows. When POC difference (Tr) between reference picture of current PU and reference picture of candidate PU (co-located PU or neighbor PU) are larger than pre-determined threshold (THpoc_diff), motion vector scaling is not used. The motion vector predictor is marked as unavailable before the scaling process is performed. This process is applied to both spatial and temporal scaling. This method was proposed in JCTVC-G551 [2] and implemented again based on the HM-7.0. Please see the JCTVC-G551 for details.

# Experimental results

An experiment for solution 2 was conducted using HM-7.1 under common test condition [1]. The pre-defined thresholds (MV_SCALING_RESTRICTION_TH and MV_SCALING_RESTRICTION_TH2) are set as 8 and 16, respectively, for common test conditions. Average coding efficiency gains from this restriction for each configuration are negligible. For class F sequence, the coding efficiency gains are 0.0% (RA-Main), 0.0% (RA-HE10), 0.1% (LD-Main), and 0.1% (LD-HE10).

Table 1. Experimental results when motion vector scaling is restricted. (8, 16)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class D | -0.1% | -0.2% | 0.3% | 0.0% | -0.1% | -0.3% |
| Class E | -0.1% | -0.6% | -0.3% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% | 1.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

# Conclusions

In this contribution, restricted usages of motion vectors from LTRPs were proposed. The first solution proposed not to insert motion vectors into the candidate list by marking it unavailable if the motion vectors are from LTRPs. The second solution was that when POC difference (Tr) between reference picture of current PU and reference picture of candidate PU (collocated PU or neighbor PU) are larger than pre-determined threshold (THpoc_diff), motion vector scaling is restricted. The motion vector predictor is marked as unavailable before the scaling process is performed. This restriction is applied to both spatial and temporal scaling. Although average coding efficiency gains from the second solution for each configuration are negligible, coding efficiency gains for Class F under Low delay B main and Low delay B HE10 are 0.1% and 0.1%, respectively.

# Suggested text specification

## Solution 1

8.5.2.1.6	Derivation process for motion vector predictor candidates

Inputs to this process are

–	a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

–	the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

–	the motion vectors mvLXN of the neighbouring prediction units,

–	the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8 3 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

1.	Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

2.	Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

3.	When one or more of the following conditions are true, the variable isScaledFlagLX is set equal to 1.

–	the prediction unit covering luma location ( xA0, yA0 ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] and PredMode is not MODE_INTRA.

–	the prediction unit covering luma location ( xA1, yA1 ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] and PredMode is not MODE_INTRA.

4.	For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

–	If the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to ListX.

–	Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xAk ][ yAk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to ListY and mvLXA is set equal to mvLXA.

5.	When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

–	If the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, and RefPicListA[ refIdxLX[ xAk ][ yAk ] ] is short-term reference pictures, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to ListX.

–	Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, and RefPicListA[ refIdxLY[ xAk ][ yAk ] ] is short-term reference pictures,availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to ListY.

–	When availableFlagLXA is equal to 1, and both RefPicListA[ refIdxA ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8 126)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 127)

mvLXA = Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvLXA ) *

( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8 128)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListA[ refIdxA ] ) )	(8 129)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8 130)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

1.	Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line. [Ed. (BB): Define MinPuSize in the SPS but the derivation should depend on the use of an AMP flag ]

2.	When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8 131)

xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8 132)

xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8 133)

3.	Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

4.	For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

–	If the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to ListX.

–	Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xBk ][ yBk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to ListY.

5.	When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB and availableFlagLXA is set equal to 1.

6.	When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

–	If the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and RefPicListB[ refIdxLX[ xBk ][ yBk ] ] is short-term reference pictures, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to ListX.

–	Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, and RefPicListB[ refIdxLY[ xBk ][ yBk ] ] is short-term reference pictures, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to ListY.

–	When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListB[ refIdxB ] ) is not equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and both RefPicListB[ refIdxB ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8 134)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 135)

mvLXB =Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvLXA ) *

( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8 136)

[Ed. (GJS): I believe the thing that is being clipped is an integer, so does that make sense?]

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListB[ refIdxB ] ) )	(8 137)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8 138)

8.5.2.1.7	Derivation process for temporal luma motion vector prediction

Inputs to this process are

–	a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

–	the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

–	the motion vector prediction mvLXCol,

–	the availability flag availableFlagLXCol.

The function RefPicOrderCnt( picX, refIdx, LX ) returns the picture order count PicOrderCntVal of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

RefPicOrderCnt( picX, refIdx, LX ) = PicOrderCnt(RefPicListX[ refIdx ] of the picture picX)	(8 141)

Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

–	If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

–	Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

1.	The variable colPu is derived as follows

yPRb = yP + nPSH	(8 139)

–	If ( yP >> Log2CtbSize ) is equal to ( yPRb >> Log2CtbSize ), the horizontal component of the right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8 140)

and the variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

–	Otherwise ( ( yP >> Log2CtbSize ) is not equal to ( yPRb >> Log2CtbSize ) ), colPu is marked as "unavailable".

2.	When colPu is coded in an intra prediction mode or colPu is marked as "unavailable", the following applies.

–	Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 )	(8 141)

yPCtr = ( yP + ( nPSH >> 1 )	(8 142)

–	The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

3.	( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The function LongTermRefPic( picX, refIdx, LX ) is defined as follows. If the reference picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) returns 1; otherwise LongTermRefPic( picX, refIdx, LX ) returns 0.

The variables mvLXCol and availableFlagLXCol are derived as follows.

–	If one or more of the following conditions are true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

–	colPu is coded in an intra prediction mode.

–	colPu is marked as "unavailable".

–	pic_temporal_mvp_enable_flag is equal to 0.

–	Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

–	If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxL1[ xPCol ][ yPCol ], and L1, respectively.

–	Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

–	If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxL0[ xPCol ][ yPCol ], and L0, respectively.

–	Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

–	If PicOrderCnt( pic ) of every picture pic in every reference picture lists is less than or equal to PicOrderCntVal, mvCol, refIdxCol, and listCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxLX[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

–	Otherwise (PicOrderCnt( pic ) of at least one picture pic in at least one reference picture list is greater than PicOrderCntVal, mvCol, refIdxCol and listCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxLN[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

and if LongTermRefPic( colPic, refIdxCol, listCol ) is equal to 1, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. Otherwise the variable availableFlagLXCol is set equal to 1 and the following applies.

–	If RefPicListX[ refIdxLX ] is a long-term reference picture, or LongTermRefPic( colPic, refIdxCol, listCol ) is equal to 1, or PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ),

mvLXCol = mvCol	(8 143)

–	Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(8 144)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 145)

mvLXCol =  Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvCol ) *

( (Abs( DistScaleFactor * mvCol ) + 127 ) >> 8 ) )	(8 146)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) )	(8 147)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX [ refIdxLX ] ) )	(8 148)

## Solution 2

Derivation process for motion vector predictor candidates

Inputs to this process are

a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

the motion vectors mvLXN of the neighbouring prediction units,

the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8 3 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

When one or more of the following conditions are true, the variable isScaledFlagLX is set equal to 1.

the prediction unit covering luma location ( xA0, yA0 ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] and PredMode is not MODE_INTRA.

the prediction unit covering luma location ( xA1, yA1 ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ] and PredMode is not MODE_INTRA.

For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

If the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to ListX.

Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xAk ][ yAk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to ListY and mvLXA is set equal to mvLXA.

When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

If the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, and abs(RefPicOrderCnt( currPic, refIdxLX, LX)– RefPicOrderCnt( currPic, refIdxLX[ xAk ][ yAk ], LX) is equal or smaller than MvScalingThreshold, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to ListX.

- MvScalingThreshold is derived as follows.

		For variables DiffPocB and DiffPocD, the following assignments are made

			DiffPocB = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX, LX )

			DiffPocD = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX[ xAk ][ yAk ], LX)

		If ( (DiffPocB < 0 &&  DiffPocD < 0) || (DiffPocB > 0 && DiffPocD > 0)), MvScalingThreshold is set equal to 8.

		Otherwise, MvScalingThreshold is set equal to 16.

Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, and abs(RefPicOrderCnt( currPic, refIdxLX, LX) – RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal or smaller than MvScalingThreshold, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to ListY.

- MvScalingThreshold is derived as follows.

		For variables DiffPocB and DiffPocD, the following assignments are made

			DiffPocB = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX, LX )

			DiffPocD = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY)

		If ( (DiffPocB < 0 &&  DiffPocD < 0) || (DiffPocB > 0 && DiffPocD > 0)), MvScalingThreshold is set equal to 8.

		Otherwise, MvScalingThreshold is set equal to 16.

When availableFlagLXA is equal to 1, and both RefPicListA[ refIdxA ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8 126)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 127)

mvLXA = Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvLXA ) *

( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8 128)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListA[ refIdxA ] ) )	(8 129)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8 130)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line. [Ed. (BB): Define MinPuSize in the SPS but the derivation should depend on the use of an AMP flag ]

When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8 131)

xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8 132)

xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8 133)

Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

If the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to ListX.

Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xBk ][ yBk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to ListY.

When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB and availableFlagLXA is set equal to 1.

When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

If the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and abs(RefPicOrderCnt( currPic, refIdxLX, LX)– RefPicOrderCnt( currPic, refIdxLX[ xBk ][ yBk ], LX) is equal or smaller than MvScalingThreshold, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to ListX.

- MvScalingThreshold is derived as follows.

		For variables DiffPocB and DiffPocD, the following assignments are made

			DiffPocB = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX, LX )

			DiffPocD = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX[ xAk ][ yAk ], LX)

		If ( (DiffPocB < 0 &&  DiffPocD < 0) || (DiffPocB > 0 && DiffPocD > 0)), MvScalingThreshold is set equal to 8.

		Otherwise, MvScalingThreshold is set equal to 16.

Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, and abs(RefPicOrderCnt( currPic, refIdxLX, LX) – RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal or smaller than MvScalingThreshold, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to ListY.

- MvScalingThreshold is derived as follows.

		For variables DiffPocB and DiffPocD, the following assignments are made

			DiffPocB = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLX, LX )

			DiffPocD = PicOrderCnt(currPic) – RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY)

		If ( (DiffPocB < 0 &&  DiffPocD < 0) || (DiffPocB > 0 && DiffPocD > 0)), MvScalingThreshold is set equal to 8.

		Otherwise, MvScalingThreshold is set equal to 16.

When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListB[ refIdxB ] ) is not equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and both RefPicListB[ refIdxB ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8 134)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 135)

mvLXB =Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvLXA ) *

( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8 136)

[Ed. (GJS): I believe the thing that is being clipped is an integer, so does that make sense?]

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListB[ refIdxB ] ) )	(8 137)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8 138)

8.5.2.1.7	Derivation process for temporal luma motion vector prediction

Inputs to this process are

–	a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

–	the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

–	the motion vector prediction mvLXCol,

–	the availability flag availableFlagLXCol.

The function RefPicOrderCnt( picX, refIdx, LX ) returns the picture order count PicOrderCntVal of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

RefPicOrderCnt( picX, refIdx, LX ) = PicOrderCnt(RefPicListX[ refIdx ] of the picture picX)	(8 141)

Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

–	If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

–	Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

1.	The variable colPu is derived as follows

yPRb = yP + nPSH	(8 139)

–	If ( yP >> Log2CtbSize ) is equal to ( yPRb >> Log2CtbSize ), the horizontal component of the right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8 140)

and the variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

–	Otherwise ( ( yP >> Log2CtbSize ) is not equal to ( yPRb >> Log2CtbSize ) ), colPu is marked as "unavailable".

2.	When colPu is coded in an intra prediction mode or colPu is marked as "unavailable", the following applies.

–	Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 )	(8 141)

yPCtr = ( yP + ( nPSH >> 1 )	(8 142)

–	The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

3.	( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The function LongTermRefPic( picX, refIdx, LX ) is defined as follows. If the reference picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) returns 1; otherwise LongTermRefPic( picX, refIdx, LX ) returns 0.

The variables mvLXCol and availableFlagLXCol are derived as follows.

–	If one or more of the following conditions are true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

–	colPu is coded in an intra prediction mode.

–	colPu is marked as "unavailable".

–	pic_temporal_mvp_enable_flag is equal to 0.

–	Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

–	If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxL1[ xPCol ][ yPCol ], and L1, respectively.

–	Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

–	If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxL0[ xPCol ][ yPCol ], and L0, respectively.

–	Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

–	If PicOrderCnt( pic ) of every picture pic in every reference picture lists is less than or equal to PicOrderCntVal, mvCol, refIdxCol, and listCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxLX[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

–	Otherwise (PicOrderCnt( pic ) of at least one picture pic in at least one reference picture list is greater than PicOrderCntVal, mvCol, refIdxCol and listCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxLN[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

If abs(RefPicOrderCnt( currPic, refIdxLX, LX ) – RefPicOrderCnt( colPic, refIdxCol, ListCol )) is larger than MvScalingThreshold, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. MvScalingThreshold is derived as follows.

1.	For variables DiffPocB and DiffPocD, the following assignments are made

		DiffPocB = PicOrderCnt(currPic) - RefPicOrderCnt( currPic, refIdxLX, LX )

	DiffPocD = PicOrderCnt(colPic)  - RefPicOrderCnt( colPic, refIdxCol, ListCol ).

2.	If ( (DiffPocB < 0 &&  DiffPocD < 0) || (DiffPocB > 0 && DiffPocD > 0)), MvScalingThreshold is set equal to 8.

3.	Otherwise, MvScalingThreshold is set equal to 16.

Otherwise, the variable availableFlagLXCol is set equal to 1 and the following applies.

–	If RefPicListX[ refIdxLX ] is a long-term reference picture, or LongTermRefPic( colPic, refIdxCol, listCol ) is equal to 1, or PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ),

mvLXCol = mvCol	(8 143)

–	Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(8 144)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8 145)

mvLXCol =  Clip3( −8192, 8191.75, Sign2( DistScaleFactor * mvCol ) *

( (Abs( DistScaleFactor * mvCol ) + 127 ) >> 8 ) )	(8 146)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) )	(8 147)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX [ refIdxLX ] ) )	(8 148)

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May. 2012.

- I. –K. Kim, Y. Park, N. Shlyakhov, and J. H. Park, “Restriction on motion vector scaling for Merge and AMVP,” Document of Joint Collaborative Team on Video Coding, JCTVC-G551, Nov. 2011.

- C. S. Lim and S. M. T. Naing, “MVP scaling issue for LTRPs,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0422, May. 2012.

- JCTVC-I1003_d5.doc

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).