| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0303 |
| --- | --- |

| Title: | Simplification on context derivation of cbf_luma syntax element |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim JeongHoon Park | Email: | ilkoo.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

For simplification of context selection of cbf_luma syntax element, maximum transform size checking for cbf_luma is removed in this proposal. Average loss from this simplification is negligible (0.0%). It is also observed that 0.1% and 0.2% losses for class F in random access HE10 and low delay B HE10 configurations, respectively.

# Introduction

In current HEVC draft specification, context of cbf_luma syntax element is determined based on following condition.

( trafoDepth = = 0 ) | | ( log2TrafoSize = =  Log2MaxTrafoSize )

If above condition is true, ctxIdxInc is set equal to 1. Otherwise, ctxIdxInc is set equal to 0. Since luma_cbf is encoded and parsed at the leaf node of transform tree, this condition specifies the real root of transform tree in which actual transform is applied. Although characteristics of cbf are dependent on both residual property and transform size, it is rather complex to check equality between current transform size and maximum transform size every time. For simplification, maximum transform size checking is removed in this proposal. The condition is simplified as follows.

( trafoDepth = = 0 )

Figure 1 illustrates an example cbf_luma coding when current coding block (CB) size is equal to 64x64 and maximum transform block (TB) size is equal to 32x32. When trafoDepth is equal to 0, transform tree is implicitly split without transform split flag signalling. Numbers in leaf nodes are values of cbf_luma. Main difference between HM-7.0 (a) and proposed simplification (b) is the value of ctxIdxInc when transform depth (trafoDepth) is equal to 1. Since the condition on maximum transform size, the value of ctxIdxInc is 0 for proposed simplification.

Figure 1. Example of cbf_luma coding when coding block (CB) size is equal to 64x64 and maximum transform block (TB) size is equal to 32x32. Numbers in leaf nodes are values of cbf_luma. Main difference between HM-7.0 (a) and proposed simplification (b) is the value of ctxIdxInc when transform depth (trafoDepth) is equal to 1.

# Experimental results

A test is conducted based on HM-7.0 under common test condition. The provided runtime is not accurate since heterogeneous cluster systems are used. Average loss from this simplification is negligible (0.0%). It is also observed that 0.1% and 0.2% losses for class F in random access HE10 and low delay B HE10 configurations, respectively.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | 0.3% | 0.2% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.1% | 0.3% | 0.1% | -0.7% | 0.0% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.1% | 0.2% | 0.2% | -0.1% | 0.5% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |

# Suggested text specification

Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| cbf_luma | Error: Reference source not found | 0 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
|  |  | 4 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |

# Conclusions

For simplification of context selection of cbf_luma syntax element, maximum transform size checking for cbf_luma is removed in this proposal. Average loss from this simplification is negligible (0.0%). It is also observed that 0.1% and 0.2% losses for class F in random access HE10 and low delay B HE10 configurations, respectively. It is recommended to include this simplification to the next version of text draft specification of HEVC.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).