| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0304 |
| --- | --- |

| Title: | Context assignment for parallel coefficient level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Joel Sole, Liwei Guo, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In current HM, the context modeling of the syntax element coeff_abs_level_greater1_flag is dependent on the value of previous coded coeff_abs_level_greater1_flag. Up to 8 coeff_abs_level_greater1_flag might be coded consecutively. This dependency hinders the throughput of the entropy coding and allegedly, it is one bottleneck in current CABAC design. This contribution removes the dependency of previous coded coeff_abs_level_greater1_flag for context modeling.

# Introduction

In current HM7.0, there are 4 context sets for luma components and 2 context sets for chroma components used in the modeling of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag contexts.

The index of the context set, ctxSet, is selected based on the location of the current coefficient group and the number of coeff_abs_level_greater1_flag being one in previous coded coefficient groups (numGreater1). Each coefficient group consists of 16 consecutive coefficients in the inverse scan order, and there is no overlapped coefficients between the groups. For coeff_abs_level_greater1_flag, the index of the context within a context set, greater1Ctx, is determined based on the trailing ones (coeff_abs_level_greater1_flag being 0) in the same coefficient group up to a maximum of 3. The context index is derived as:

ctxIdx_level_greater1  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )           (1)

For the context modeling of the syntax element coeff_abs_level_greater1_flag, the dependency between the consecutive coeff_abs_level_greater1_flags hinders the maximum throughput of the entropy coder. By removing the dependency, multiple bins per cycle decoding can be achieved for this syntax element.

# Proposed Solution

In [1], the context modeling dependency between current coeff_abs_level_greater1_flag and the number of trailing ones in the same coefficient group is removed. The proposed modifications have been made with the consideration of the current HM. The modifications are as follow:

- greater1Ctx is equal to the half of the accumulated number of coded coeff_abs_level_greater1_flag in the same subset and is thresholded at 3 for both luma and chroma components. The derivation is shown in Eq (2).

greater1Ctx = (# of coeff_abs_level_greater1_flag +1)>>1                           (2)

With the modification, the total number of contexts remains unchanged. The coding efficiency impact due to these changes is shown in Section 3.

- A second method that is a variant of the derivation in equation (2) is tested. In this case, the variable numOne that is used in HM7.0 to store the number of coefficients larger than 1 in the previous coefficient groups is put in the derivation as follows:

greater1Ctx = (# of coeff_abs_level_greater1_flag +1 + numOne)>>1                           (3)

# Simulation results

The proposed method is implemented on HM7.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM7.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 1 and 2 summarize the experimental results for QP={22, 27, 32, 37} and QP={12, 17, 22, 27}, respectively. Results for method 2 are shown in table 3.

Table 1 Simulation results for QP={22, 27, 32, 37}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.23% | 0.23% | 0.21% | 0.23% | 0.26% | 0.20% |
| Class B | 0.11% | 0.17% | 0.15% | 0.10% | 0.17% | 0.14% |
| Class C | 0.14% | 0.21% | 0.18% | 0.12% | 0.16% | 0.16% |
| Class D | 0.13% | 0.20% | 0.20% | 0.12% | 0.18% | 0.16% |
| Class E | 0.17% | 0.28% | 0.22% | 0.17% | 0.27% | 0.22% |
| Overall | 0.15% | 0.21% | 0.19% | 0.14% | 0.21% | 0.17% |
|  | 0.15% | 0.21% | 0.19% | 0.14% | 0.21% | 0.18% |
| Class F | 0.36% | 0.41% | 0.34% | 0.36% | 0.25% | 0.21% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.15% | 0.27% | 0.07% | 0.42% | 0.54% |
| Class B | -0.01% | 0.21% | 0.20% | 0.00% | 0.16% | 0.16% |
| Class C | -0.04% | 0.19% | 0.25% | 0.00% | 0.33% | 0.15% |
| Class D | 0.06% | 0.43% | 0.16% | 0.03% | 0.32% | -0.05% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.18% | 0.22% | 0.02% | 0.30% | 0.20% |
|  | 0.06% | 0.18% | 0.23% | 0.04% | 0.26% | 0.18% |
| Class F | 0.17% | 0.41% | 0.31% | 0.17% | 0.45% | 0.05% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.36% | 0.00% | -0.02% | 0.67% | 0.36% |
| Class C | -0.03% | 0.38% | 0.34% | 0.00% | 0.33% | 0.23% |
| Class D | 0.15% | 1.25% | 0.85% | 0.07% | 1.20% | 0.42% |
| Class E | -0.17% | -0.75% | -0.07% | 0.19% | 0.26% | 0.65% |
| Overall | 0.01% | 0.38% | 0.28% | 0.05% | 0.64% | 0.39% |
|  | 0.01% | 0.37% | 0.28% | 0.05% | 0.70% | 0.42% |
| Class F | 0.04% | 0.15% | 0.82% | 0.30% | 0.33% | 0.49% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 2 Simulation results for QP={12, 17, 22, 27}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.27% | 0.30% | 0.30% | 0.29% | 0.32% | 0.32% |
| Class B | 0.12% | 0.22% | 0.20% | 0.14% | 0.22% | 0.18% |
| Class C | 0.20% | 0.25% | 0.23% | 0.17% | 0.21% | 0.19% |
| Class D | 0.23% | 0.23% | 0.21% | 0.21% | 0.19% | 0.18% |
| Class E | 0.26% | 0.37% | 0.30% | 0.27% | 0.40% | 0.29% |
| Overall | 0.21% | 0.27% | 0.24% | 0.21% | 0.26% | 0.22% |
|  | 0.21% | 0.28% | 0.24% | 0.21% | 0.26% | 0.24% |
| Class F | 0.47% | 0.40% | 0.42% | 0.39% | 0.21% | 0.22% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.21% | 0.07% | 0.11% | 0.18% | 0.25% | 0.18% |
| Class B | 0.10% | 0.16% | 0.23% | 0.08% | 0.17% | 0.18% |
| Class C | 0.16% | 0.24% | 0.17% | 0.16% | 0.25% | 0.23% |
| Class D | 0.21% | 0.13% | 0.12% | 0.19% | 0.10% | -0.02% |
| Class E |  |  |  |  |  |  |
| Overall | 0.17% | 0.15% | 0.16% | 0.15% | 0.19% | 0.15% |
|  | 0.17% | 0.17% | 0.20% | 0.15% | 0.18% | 0.16% |
| Class F | 0.40% | 0.34% | 0.26% | 0.29% | 0.39% | 0.25% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.08% | 0.11% | 0.17% | 0.06% | 0.14% | 0.20% |
| Class C | 0.16% | 0.24% | 0.25% | 0.13% | 0.24% | 0.13% |
| Class D | 0.21% | 0.20% | 0.05% | 0.19% | 0.34% | 0.16% |
| Class E | 0.04% | -0.14% | 0.14% | 0.08% | 0.22% | 0.22% |
| Overall | 0.12% | 0.12% | 0.15% | 0.11% | 0.23% | 0.17% |
|  | 0.13% | 0.16% | 0.17% | 0.11% | 0.24% | 0.19% |
| Class F | 0.41% | 0.37% | 0.37% | 0.26% | 0.15% | 0.14% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3 Simulation results for method 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.15% | 0.11% | 0.06% | 0.12% | 0.18% |
| Class B | 0.06% | 0.11% | 0.10% | 0.05% | 0.12% | 0.09% |
| Class C | 0.13% | 0.16% | 0.18% | 0.11% | 0.14% | 0.10% |
| Class D | 0.13% | 0.26% | 0.22% | 0.11% | 0.17% | 0.17% |
| Class E | 0.17% | 0.25% | 0.21% | 0.15% | 0.18% | 0.18% |
| Overall | 0.11% | 0.18% | 0.16% | 0.09% | 0.14% | 0.14% |
|  | 0.11% | 0.17% | 0.16% | 0.09% | 0.14% | 0.15% |
| Class F | 0.35% | 0.44% | 0.31% | 0.35% | 0.22% | 0.28% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | 0.23% | 0.16% | 0.00% | 0.49% | 0.09% |
| Class B | -0.05% | 0.25% | 0.03% | -0.02% | 0.13% | -0.03% |
| Class C | 0.01% | 0.34% | 0.10% | -0.01% | 0.32% | 0.06% |
| Class D | 0.08% | 0.37% | 0.21% | 0.07% | 0.28% | -0.03% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.30% | 0.12% | 0.01% | 0.29% | 0.02% |
|  | 0.06% | 0.30% | 0.11% | 0.02% | 0.30% | 0.02% |
| Class F | 0.19% | 0.46% | 0.35% | 0.20% | 0.30% | -0.15% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.47% | 0.17% | -0.01% | 0.59% | -0.01% |
| Class C | 0.01% | 0.43% | 0.16% | 0.02% | 0.27% | 0.48% |
| Class D | 0.10% | 0.69% | 0.36% | 0.07% | 0.83% | 0.23% |
| Class E | -0.19% | 0.43% | 0.38% | 0.12% | 0.65% | -0.21% |
| Overall | 0.00% | 0.51% | 0.26% | 0.04% | 0.58% | 0.13% |
|  | 0.00% | 0.57% | 0.25% | 0.04% | 0.64% | 0.26% |
| Class F | 0.09% | -0.35% | 1.53% | 0.29% | 1.04% | 1.17% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |

# Conclusions

In this contribution, we propose to remove the dependency on the value of coeff_abs_level_greater1_flag in the same coefficient group for the context selection of coeff_abs_level_greater1_flag. With the proposed modification, CABAC throughput is improved. The coding performance impact from the proposed scheme is small based on the simulation results.

# References

[1] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, May, 2012.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text changes

##### 9.2.3.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the colour component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0 or cIdx is greater than 0, the following applies.

ctxSet  =  0		(9-1)

- Otherwise (i is greater than 0 and cIdx is equal to 0), the following applies.

ctxSet  =  2		(9-2)

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause Error: Reference source not found for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-3)

- The variable greater1Ctx is set equal to 10.

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater1Ctx is set equal to the variable greater1Ctx that has been derived during the last invocation of this subclause incremented by 1.

- When greater1Ctx is greater than 0, the variable lastGreater1Flag is set equal to the syntax element coeff_abs_level_greater1_flag that has been used during the last invocation of this subclause and greater1Ctx is modifed as follows.

- If lastGreater1Flag is equal to 1, greater1Ctx is set equal to 0.

- Otherwise (lastGreater1Flag is equal to 0), greater1Ctx is incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, (greater1Ctx+1) >>1)	(9-4)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  16